[University name]

[Faculty / Department]

Digital Systems 2 (Microprocessors)

Project 1

VOID RUNNER

A vertical space shooter on an Arduino Mega

432 single LEDs · a 9-button keypad · 4 buzzers · no modules, no libraries

| Student | [Student name] |
| --- | --- |
| Student ID | [Student ID] |
| Teacher | [Teacher name] |
| Course | Digital Systems 2 (Microprocessors) |
| Semester | [Semester, e.g. Fall 2026] |
| Date | [Submission date] |
| Platform | Arduino Mega 2560, simulated in Wokwi |
| Source code | https://github.com/Arash1Kazemi/void-runner |

# The hardware

Figure 1. The whole circuit in Wokwi: 432 LEDs (left), Arduino Mega 2560 (top), 3 × 3 push-button keypad and 4 buzzers (right). The LEDs show a real moment of the game: the WARDEN boss fight.

# 3. Project performance

VOID RUNNER is a complete arcade game built only from the parts the course allows: one Arduino Mega 2560, single LEDs, single push buttons and buzzers. There are no ready-made modules (no LED matrix module, no keypad module) and no libraries; the screen, the keypad and the four-voice music are all driven by our own code. The player flies a small ship at the bottom of a 16 × 24 LED screen, shoots enemies and space rocks, catches ammo crates and extra lives, and fights a boss at the end of each of the three phases. The project runs in the Wokwi simulator (Wokwi for VS Code and wokwi.com).

## 3.1 Parts and connections

| Part | Count | Connection to the Mega |
| --- | --- | --- |
| Arduino Mega 2560 | 1 | – |
| Single LEDs (wokwi-led), one matrix | 432 | 16 × 24 game screen + 3 status bars of 16. 27 row wires (anodes) on pins 22–45 and 47–49, 16 column wires (cathodes) on A0–A15 |
| Push buttons (wokwi-pushbutton), 3 × 3 matrix | 9 | rows on pins 18–20, columns on 15–17 |
| Buzzers (wokwi-buzzer) | 4 | pins 5, 6, 11, 46 (outputs of Timer3, Timer4, Timer1, Timer5) |

Table 1. The parts of the device.

53 of the Mega's pins are used, and every header has one job: the double header drives the LED rows, the analog header the LED columns, the communication header the keypad and the PWM header the buzzers. No resistors are needed in the simulation. The circuit file (diagram.json) and the pin list of the code (pins.h) are both generated by one program (tools/gen_circuit.py), which also checks every wire, so the circuit and the code always agree.

Figure 2. Simplified wiring: how every LED, key and buzzer is connected.

## 3.2 The LED screen

All 432 LEDs form one matrix of 27 rows × 16 columns: the + leg of every LED is on its row wire, the – leg on its column wire. The Mega switches on one row at a time and pulls the columns of the LEDs that should light LOW. A Timer2 interrupt moves to the next row about 1900 times per second, so the whole screen is redrawn 71 times per second and the eye sees a steady picture. The columns are written with two port registers (PORTF, PORTK) at once: this costs about 3 % of the processor, where digitalWrite would cost about 35 %.

The game draws into a picture in memory (frame, one 16-bit word per row) and copies it to the screen when it is finished, so a half-drawn picture is never shown.

| Area | LED colour | Use in the game |
| --- | --- | --- |
| Status bar 1 | red | phase progress, then the boss's health |
| Status bar 2 | yellow | ammo (blinks when low) |
| Status bar 3 | green | lives (one block of 3 LEDs per life) |
| Game rows 0–5 | red | where enemies and bosses enter, "WARN", "ZONE" |
| Game rows 6–17 | white | open space, message pages |
| Game rows 18–23 | green | the player's zone |

Table 2. LED colours. Each LED has a fixed colour, like the colour strips of the 1978 Space Invaders cabinet.

Figure 3. The LED panel in Wokwi during a boss fight.

Brightness. Each LED is lit only 1/27 of the time. Wokwi shows brightness as (share of time on)^(1/gamma); with the default gamma a lit LED looked only 32 % bright, which was hard to see. Every LED therefore has gamma = 30 (90 %), which is what a real multiplexed display does by driving each LED harder in its short time slot. Every LED also has fps = 300, so moving objects stay sharp, and everything that blinks stays on and off for at least 160 ms so the blink is visible.

Text never scrolls (scrolling letters smeared into a white block in the simulator). Every message is a still page of short words, at most four small letters per line, shown for 0.9 s, with big digits and signs where a word is not needed.

## 3.3 The keypad and the controls

The 9 push buttons form a 3 × 3 matrix, one button for each key the game uses. The code drives one row LOW at a time and reads the four columns with the internal pull-ups, so a pressed key reads LOW. The rows that are not being read are switched to inputs, so two pressed keys can never short a HIGH and a LOW output together. Every key is debounced. The buttons are labelled like keyboard keys and also react to the PC keyboard in Wokwi.

Figure 4. The keypad in Wokwi.

| Key | Action |
| --- | --- |
| W A S D | move (tap = one pixel, hold = glide) |
| R | fire (tap = one bullet, hold = auto-fire) · OK in the menus |
| Q | back · pause in the game |
| P | pause / continue |
| M | sound on / off (any screen, saved) |
| B | brightness 100 / 50 / 20 % (any screen, saved) |

Table 3. The keys.

Firing: press R for one bullet; keep holding and another follows every 0.2 s (a chirp when auto-fire starts, and the ship's nose blinks). Moving: a tap moves one pixel, a hold makes the ship glide after 0.17 s. The newest key wins: a move key stops the fire, and while R is held the ship does not glide.

## 3.4 The four buzzers

Each buzzer is connected to the output pin of its own 16-bit hardware timer. In CTC mode with "toggle on compare match" the timer flips the pin by itself, so a note costs no processor time and four notes can sound at once (Arduino's tone() can play only one). The timers run on a slow clock (÷256), which keeps notes up to about C7 within 1 % of their pitch.

Figure 5. The four buzzers in Wokwi.

| Buzzer | Pin / timer | Job |
| --- | --- | --- |
| 1 | 5 / Timer3 | lead guitar (melody) |
| 2 | 6 / Timer4 | bass |
| 3 | 11 / Timer1 | every sound effect and menu sound (music only in short jingles) |
| 4 | 46 / Timer5 | rhythm guitar: a fifth above the bass (bass + rhythm = a power chord) |

Table 4. The voices.

## 3.5 Start-up logo and welcome music

When the device starts, a 5.6-second logo plays in sync with the welcome melody: the picture runs on the song's own clock, so every movement lands on a note. "VOID" is a vertical sign of bold 10 × 5 letters, and the status bars fill up like a start-up progress bar. Any key skips the logo.

| 0.6 s warp | 1.6 s the ship | 3.1 s letters | 4.2 s RUNNER | 4.9 s flash | 5.4 s the sign |
| --- | --- | --- | --- | --- | --- |

Figure 6. The start-up logo.

| Time | Picture | Sound |
| --- | --- | --- |
| 0.0 – 1.2 s | warp: star streaks speed up | rising whoosh + low A2 drone |
| 1.2 – 2.0 s | the ship rises and brakes | braking notes E4 C4 A3 |
| 2.0 – 2.4 s | the ship shoots up and out | quick rising glide |
| 2.4 – 3.6 s | V, O, I, D light up one by one, flickering on; drones fly past | A4 C5 E5 A5, a crackle on each flicker |
| 3.6 – 4.8 s | "RUNNER" crosses on a dark ribbon, towed by the ship | melody + harmony + bass line |
| 4.8 – 5.6 s | the whole screen flashes, then the sign glows | A-minor chord |

Table 5. Timeline of the logo.

## 3.6 Title screen, demo and menu

The title screen keeps the VOID sign lit while ships fly past it, and every 7 seconds a banner shows the pages "PUSH", a big R, "TO", "PLAY". If nobody touches a key for 20 seconds a demo starts: the autopilot plays the game by itself until any key is pressed. R opens the menu: PLAY, SHIP, OPTS and BEST (W/S choose, R open, Q back); the chosen item is underlined.

| title | banner | menu |
| --- | --- | --- |

Figure 7. Title screen and main menu.

## 3.7 Hangar and options

In the hangar the player chooses one of three ships with A/D. The ship is shown three times bigger, and the three status bars show its speed, ammo and fire power. The options are pages: LITE (brightness), SND (sound), DIFF (EASY / NORM / HARD) and TEST (the LED test). W/S choose a page, A/D or R change it, page dots at the bottom show where you are. Every setting is saved in the EEPROM.

| BOLT | DART | TWIN |
| --- | --- | --- |

Figure 8. The hangar.

| Ship | Speed | Fire | Ammo | Note |
| --- | --- | --- | --- | --- |
| BOLT | 70 ms / pixel | every 0.20 s | 60 | balanced |
| DART | 45 ms / pixel | every 0.22 s | 40 | fastest, less ammo |
| TWIN | 95 ms / pixel | every 0.26 s | 80 | two bullets per shot (2 ammo), slowest |

Table 6. The three ships.

| LITE | SND | DIFF | TEST |
| --- | --- | --- | --- |

Figure 9. The option pages.

## 3.8 Playing the game

A game has three phases. Each phase starts with the pages "ZONE" and a big phase number, the phase name and "GO!" with a fanfare. Then the enemies and rocks of the phase arrive from its spawn list. When the list is finished and the screen is empty, "WARN" and a blinking warning sign appear with a siren while the red bar charges up, and the boss comes with its own music. When the boss explodes, the pages "ZONE DONE", the bonus points and "LIFE UP" follow, and the next phase begins. After the last boss the victory screen comes straight away.

| ZONE 1 | phase name | GO! | phase 1 | phase 1 | phase 1 |
| --- | --- | --- | --- | --- | --- |

Figure 10. Start of phase 1, ROCK BELT, and play.

| Topic | Rule |
| --- | --- |
| Lives | EASY 4, NORMAL 3, HARD 3 (at most 4). A hit: explosion, screen shake and 2 s of blinking shield. +1 life after each phase; extra-life pickups. |
| Ammo | Per ship (60 / 40 / 80), full at the start of every phase. Ammo crates give +20 / 15 / 12 by difficulty. At 5 or less a warning beeps, the yellow bar blinks and a crate is always sent. Empty: a dry click. |
| Enemies | All slower than the player; at most 4 / 5 / 6 on the screen at once (phase 1 / 2 / 3). They shoot only from the upper rows and never right next to the ship, and they never move onto each other, so the small screen stays readable. |
| Space rocks | Hollow, lumpy shapes (small 3 × 3, big 4 × 3) that fall slowly and drift; a big rock takes 2 hits and breaks into two small ones. They hurt on contact. |
| Explosions | Only the bosses and the player's ship explode into flying pixels; enemies just vanish with a boom (flying debris looked like bullets). |
| Collisions | Pixel-exact: a hit counts only when lit pixels touch. |
| Score | Drone 10, weaver 15, miner 20, cruiser 40, rocks 5 / 10, mine 2, pickup 5, bosses 200 / 300 / 500, phase bonus 100 × phase; at most 9999. |

Table 7. The rules of the game.

| drone | weaver | mine-layer | cruiser | small rock | big rock | ammo crate | extra life |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 11. Enemies, rocks and pickups, pixel by pixel.

| Enemy | Behaviour |
| --- | --- |
| Drone | sinks slowly, shoots straight down |
| Weaver | sinks while zig-zagging, shoots now and then |
| Mine-layer | crosses the top of the screen and drops mines (mines can be shot) |
| Cruiser | 5 hits; stops near the top, slides sideways, fires shots aimed at the player, drops an extra life |
| Rocks | fall and drift sideways; a big rock breaks into two |

Table 8. Enemy types.

## 3.9 The status bars

The three bars above the game screen show the state of the game at a glance. Red: how far the phase has come; during the warning it charges up, and in the boss fight it shows the boss's health, dropping with every hit. Yellow: the ammo left (it blinks when 5 or fewer are left). Green: the lives, one block of three LEDs per life.

## 3.10 The three phases and their bosses

| Phase | Enemies | Boss |
| --- | --- | --- |
| 1 – ROCK BELT | drones and many rocks | WARDEN: wide (10 × 4), slides along the top, 3-way spread, drops rocks; below half health faster with a 5-way spread |
| 2 – DEEP MIST | weavers, mine-layers, drones | HUNTER: small and fast; dashes to a new spot, fires an aimed pair, and every 7 s blinks and dives at the player |
| 3 – THE VOID | cruisers, everything else, rock showers | VOID KING: huge (12 × 5), 5-way spread, calls in drones; at half health its eye opens (roar, shake), then aimed triple volleys and rock rain |

Table 9. Phases and bosses. Boss health: 24 / 22 / 40, scaled by the difficulty.

| WARN | warning sign | WARDEN | WARDEN | boss down | ZONE DONE |
| --- | --- | --- | --- | --- | --- |

Figure 12. Phase 1: the warning, the WARDEN fight and the end of the phase.

| bonus | LIFE UP | phase 2 | phase 2 | HUNTER | HUNTER |
| --- | --- | --- | --- | --- | --- |

Figure 13. Phase 2, DEEP MIST, and the HUNTER.

| phase 3 | phase 3 | VOID KING | VOID KING | eye open | form 2 |
| --- | --- | --- | --- | --- | --- |

Figure 14. Phase 3, THE VOID, and the VOID KING in both forms.

| WARDEN | HUNTER | VOID KING | VOID KING, form 2 |
| --- | --- | --- | --- |

Figure 15. The bosses, pixel by pixel.

| Difficulty | Lives | Enemy fire | Boss health | Ammo crate |
| --- | --- | --- | --- | --- |
| EASY | 4 | 1.5 × slower | 80 % | +20 |
| NORMAL | 3 | normal | 100 % | +15 |
| HARD | 3 | 1.33 × faster | 130 % | +12 |

Table 10. Difficulty levels (OPTS → DIFF).

## 3.11 Pause, game over, victory and best score

P (or Q) pauses the game: the game clock stops, the score and a big pause sign are shown, and the pages "P:GO" / "Q:EX" explain the keys. When the last life is lost, GAME OVER shows the score (blinking for a new record) and the best score. After the VOID KING the victory screen shows fireworks and the pages "YOU WIN!", "VOID SAFE" and the score. The BEST page shows the high score, how far that game got (ZONE 1–3, or WIN) and its difficulty; it is kept in the EEPROM, so it survives a power cut on a real Arduino.

| pause | pause | game over | game over | no record yet |
| --- | --- | --- | --- | --- |

Figure 16. Pause and game over.

| YOU WIN! | VOID SAFE | score | BEST | how far | difficulty |
| --- | --- | --- | --- | --- | --- |

Figure 17. Victory and the best score.

## 3.12 Sound and music

All music is written as note lists, one per buzzer, stored in flash. The in-game songs are original metal tracks written for this project in a short notation (tools/compose_music.py): buzzer 1 plays the lead guitar, buzzer 2 the bass and buzzer 4 a rhythm guitar a fifth above the bass, which together sound like a power chord. Buzzer 3 is kept free for sound effects, so an effect never cuts the music.

| Song | Style | Character |
| --- | --- | --- |
| ROCK BELT (phase 1) | fast thrash, E minor, about 190 BPM, loop 10.2 s | the gallop rhythm on low E; a question-and-answer melody |
| DEEP MIST (phase 2) | slow and heavy, drop-D, half-time, loop 12.8 s | short muted chugs with gaps; sparse, spooky lead |
| THE VOID (phase 3) | syncopated groove, E minor, loop 9.6 s | off-beat stabs; a hook you can hum |
| Boss (all three) | tremolo picking, chromatic, about 210 BPM, loop 4.5 s | nonstop 16th notes; a wailing lead figure |

Table 11. The in-game songs.

Other music: the start-up melody, the phase fanfare, phase clear, victory and game over. Sound effects use priorities, so an important sound is never cut by a small one: shot, auto-fire chirp, empty click, hit, explosion, big explosion, player hit, pickup, extra life, low ammo, warning siren, boss roar and the LED-test sweep. Explosions are noise (a random frequency every 6 ms); the other effects slide between two frequencies.

## 3.13 LED test

OPTS → TEST checks the hardware: every LED on, then a row sweep (status bars first), a column sweep and a checkerboard, with rising beeps. A broken LED or wire is easy to spot.

| all on | rows | columns | checkerboard |
| --- | --- | --- | --- |

Figure 18. The LED test.

## 3.14 Reliability and measurements

Nothing blocks: there is no delay() in the game. A new picture is made every 20 ms, and each part of the program does one small step per picture. The game has its own clock (gameMs) that stops while paused.

Screen: the Timer2 interrupt shows the picture whatever the game code is doing; 3 % of the processor.

Simulation speed: the timers run on slow clocks and unused timers are stopped, because Wokwi spends time on every timer count. Measured: about 93 % of real time during the logo and 91 % with every LED lit.

Memory: only fixed-size arrays (no malloc), so nothing can run out during a game. 33 KB of 248 KB flash and 2.0 KB of 8 KB RAM are used; fonts, pixel art, music and spawn lists are kept in flash.

Fairness: at most 4–6 enemies at once, no shots from close range, a crate always comes when the ammo runs low, and every phase was checked to be winnable.

## 3.15 Testing

Besides playing it in Wokwi, the real game code is compiled on the PC together with a small stand-in for the Arduino (registers as variables, a simulated clock, a simulated keypad). Test scripts press keys and check the Serial output and the screen, so a whole game runs in under a second. An autopilot that plays like a careful player was used to check that every phase can be won, and to tune the difficulty.

| Test | What it checks | Result |
| --- | --- | --- |
| controls | tap, glide, row limit, single shot, auto-fire rate, move cancels fire, no glide while firing | pass |
| screens | logo skip, title, menu, hangar, options, LED test, best, pause, game over and new record, global keys, power cycle (EEPROM), demo | pass |
| playthrough | all three phases and bosses, VOID KING form 2, victory | pass |
| balance | autopilot without shield, reacting only every 250 ms (roughly a human), 48 games per difficulty | EASY 47/48, NORMAL 38/48, HARD 27/48 won |
| QA run | four test agents played and stress-tested every part (1000+ games and random key runs) | no crash or hang; findings fixed or listed |

Table 12. Automatic tests.

Serial test commands (typed in the Wokwi terminal, starting with !) help with testing: !d shows the screen as text, !s the game state, !1–!3 start at a phase, !b skips to the boss, !i gives a shield and !x turns on the autopilot.

# 4. Project code

The program is written in C++ for the Arduino Mega (Arduino core functions plus direct register access where speed or timing needs it; no libraries). It is one sketch, void_runner.ino, with one header file per module; in Wokwi every file is a tab. All files together are compiled as one unit. The game logic advances in steps of 20 ms; the screen and the sound run on hardware timers.

| File | Contents | Lines |
| --- | --- | --- |
| void_runner.ino | The main file | 283 |
| config.h | Settings | 43 |
| pins.h | Pin numbers (generated) | 39 |
| display.h | The LED screen | 243 |
| input.h | The keypad | 74 |
| sound.h | The sound engine | 244 |
| tunes.h | Songs and sound effects | 201 |
| storage.h | EEPROM: settings and best score | 64 |
| font.h | Fonts (generated) | 99 |
| art.h | Pixel art | 147 |
| logo.h | Start-up logo and welcome melody | 161 |
| title.h | Title screen | 34 |
| menu.h | Main menu | 53 |
| world.h | Everything on the game screen | 268 |
| player.h | The player's ship | 143 |
| enemies.h | Enemies and rocks | 167 |
| levels.h | The three phases | 89 |
| boss.h | The bosses | 200 |
| game.h | One game | 536 |
| autopilot.h | Autopilot | 143 |
| screens.h | The other screens | 319 |
| total |  | 3550 |

Table 13. The files of the program.

Each file below starts with a short explanation and the list of its functions (taken from the description written above every function), followed by the full source code.

## 4.1 void_runner.ino – The main file

The sketch that Wokwi compiles; every other file is a tab of it (one translation unit). setup() stops the unused timers, starts the screen, the keypad and the sound, and loads the saved settings. loop() makes one picture every 20 ms: it reads the keys, handles the global keys M and B, runs the current screen and plays the sound. The screens form a state machine (logo → title → menu → game → pause / game over / victory …): each screen function returns the screen to show next. The Serial test commands (!d, !s, …) are handled here too.

Figure 19. What void_runner.ino draws or uses.

| Function | What it does |
| --- | --- |
| goTo(Screen next) | Switches to another screen: remembers when it opened, logs it, and does what that screen needs first (start the logo music, stop sounds, save the score, ...). |
| globalKeys() | Keys that work on every screen: B = brightness, M = sound on/off (both saved). |
| serialCommands() | Test commands over Serial start with '!', so letters typed by mistake into the Wokwi terminal do nothing (they usually mean the keyboard is talking to the terminal instead of the buttons): !d print the screen as text !s print the game state !1 !2 !3 start a game at phase 1, 2 or 3 !b skip to the boss !w the boss loses all its health !i shield on/off (no hits) !x autopilot on/off !f fast mode on/off (8 game steps per picture) !k lose the last life |
| drawScreen(uint32_t now) | One picture of the current screen: handles its keys, draws it and switches to another screen when needed (the screen state machine). |
| setup() | Runs once at power-on: Serial, LED screen, keypad, buzzers and the saved settings, then the start-up logo. |
| loop() | Runs over and over: reads the keypad, keeps the music going, reads test commands, and every 20 ms draws a new picture. |

### Source code

1  /*

2   * VOID RUNNER -- a vertical space shooter on an Arduino Mega.

3   *

4   * Hardware (see diagram.json; pins.h is generated with it by tools/gen_circuit.py):

5   *   - 16 x 24 game screen + 3 status bars, 432 single LEDs in one row/column matrix

6   *   - 3 x 3 keypad of single push buttons (W A S D move, R fire / OK, Q back,

7   *     P pause, M sound on/off, B brightness)

8   *   - 4 buzzers, one hardware timer each: lead, bass, effects, rhythm guitar

9   *

10   * Code tabs:

11   *   config.h   settings              display.h  LED screen, drawing, text

12   *   input.h    keypad                sound.h    buzzers, music engine, effects

13   *   tunes.h    songs and effects     storage.h  settings + high score in EEPROM

14   *   art.h      pixel art             font.h     two pixel fonts (generated)

15   *   logo.h     start-up logo         title.h    title screen

16   *   menu.h     main menu             screens.h  hangar, options, best, pause, end screens

17   *   world.h    everything on the game screen (player, shots, enemies, boss ...)

18   *   player.h   the player's ship     enemies.h  enemy and rock behaviour

19   *   boss.h     the three bosses      levels.h   the three phases (spawn lists)

20   *   game.h     one game: phases, collisions, status bars

21   *   autopilot.h  the ship flies itself (the title screen's demo, automatic tests)

22   *

23   * This file: the screen state machine, the Serial test commands and the main loop.

24   */

25  #include "pins.h"

26  #include "config.h"

27  #include "display.h"

28  #include "input.h"

29  #include "sound.h"

30  #include "storage.h"

31  #include "logo.h"

32  #include "title.h"

33  #include "menu.h"

34  #include "game.h"

35  #include "screens.h"

36

37  Screen screen = SCR_LOGO;

38  uint32_t screenStart = 0;                // when the current screen started (ms)

39  uint16_t frameNo = 0;                    // counts pictures, for blinking

40  uint32_t frameWorkUs = 0, frameWorkMaxUs = 0;   // CPU time of one picture (test info)

41  const uint32_t DEMO_AFTER_MS = 20000;    // title screen untouched this long: demo game

42

43  const char SCREEN_NAMES[][9] PROGMEM = {

44    "logo", "title", "menu", "hangar", "options", "best",

45    "game", "pause", "gameover", "victory", "ledtest"

46  };

47

48  // Switches to another screen: remembers when it opened, logs it, and does what that

49  // screen needs first (start the logo music, stop sounds, save the score, ...).

50  void goTo(Screen next) {

51    Screen from = screen;

52    screen = next;

53    screenStart = millis();

54    Serial.print(F("[VR] screen "));

55    Serial.println((const __FlashStringHelper*)SCREEN_NAMES[next]);

56    switch (next) {

57      case SCR_LOGO:     logoBegin(); break;

58      case SCR_TITLE:

59        soundStop();

60        if (demoMode) {                    // the demo ends

61          demoMode = false;

62          autopilot = false;

63        }

64        break;

65      case SCR_MENU:

66        if (from == SCR_PAUSE || from == SCR_GAMEOVER || from == SCR_VICTORY) soundStop();

67        break;

68      case SCR_HANGAR:   hangarBegin(); break;

69      case SCR_PAUSE:    soundStop(); break;

70      case SCR_GAME:

71        if (from == SCR_PAUSE) playStageMusic();   // a new game is started by gameStart()

72        break;

73      case SCR_GAMEOVER:

74        recordScore(game.phase + 1);

75        soundPlaySong(SONG_GAMEOVER, 150, false);

76        break;

77      case SCR_VICTORY:

78        recordScore(4);

79        nextFirework = 0;

80        soundPlaySong(SONG_VICTORY, 100, false);

81        Serial.print(F("[VR] victory score="));

82        Serial.println(player.score);

83        break;

84      case SCR_LEDTEST:  soundSfx(SFX(SFX_TEST), 5); break;

85      default: break;

86    }

87  }

88

89  // Keys that work on every screen: B = brightness, M = sound on/off (both saved).

90  void globalKeys() {

91    if (keyPressed(KEY_BRIGHTNESS)) {

92      settings.brightness = (settings.brightness + 1) % sizeof(BRIGHTNESS_LEVELS);

93      applyBrightness();

94      settingsSave();

95      soundBeep(1200, 40);

96      Serial.print(F("[VR] brightness "));

97      Serial.println(BRIGHTNESS_LEVELS[settings.brightness]);

98    }

99    if (keyPressed(KEY_MUTE)) {

100      settings.muted = !settings.muted;

101      soundSetMuted(settings.muted);

102      settingsSave();

103      soundBeep(1200, 40);

104      Serial.println(settings.muted ? F("[VR] sound off") : F("[VR] sound on"));

105    }

106  }

107

108  // Test commands over Serial start with '!', so letters typed by mistake into the Wokwi

109  // terminal do nothing (they usually mean the keyboard is talking to the terminal

110  // instead of the buttons):

111  //   !d  print the screen as text      !s  print the game state

112  //   !1 !2 !3  start a game at phase 1, 2 or 3

113  //   !b  skip to the boss              !w  the boss loses all its health

114  //   !i  shield on/off (no hits)       !x  autopilot on/off

115  //   !f  fast mode on/off (8 game steps per picture)

116  //   !k  lose the last life

117  void serialCommands() {

118    static bool command = false;

119    while (Serial.available() > 0) {

120      char c = Serial.read();

121      if (c == '!') {

122        command = true;

123        continue;

124      }

125      if (!command) {

126        if (isAlpha(c))

127          Serial.println(F("[VR] keys are going to the Serial terminal: click an empty spot of the circuit"));

128        continue;

129      }

130      command = false;

131      bool inGame = screen == SCR_GAME;

132      switch (c) {

133        case 'd': displayDump(); break;

134        case 's':

135          gamePrintState();

136          Serial.print(F("[VR] frame work us="));

137          Serial.print(frameWorkUs);

138          Serial.print(F(" max="));

139          Serial.println(frameWorkMaxUs);

140          frameWorkMaxUs = 0;

141          break;

142        case '1': case '2': case '3':

143          gameStart(c - '1');

144          goTo(SCR_GAME);

145          break;

146        case 'b': if (inGame) gameSkipToBoss(); break;

147        case 'w': if (inGame && game.stage == GS_BOSS) boss.hp = 0; break;

148        case 'i':

149          testShield = !testShield;

150          Serial.println(testShield ? F("[VR] shield on") : F("[VR] shield off"));

151          break;

152        case 'x':

153          autopilot = !autopilot;

154          Serial.println(autopilot ? F("[VR] autopilot on") : F("[VR] autopilot off"));

155          break;

156        case 'f':

157          gameSpeed = gameSpeed == 1 ? 8 : 1;

158          Serial.println(gameSpeed > 1 ? F("[VR] fast on") : F("[VR] fast off"));

159          break;

160        case 'k':

161          if (inGame && player.lives) {

162            testShield = false;

163            player.shieldUntil = 0;

164            player.lives = 1;

165            playerHit(F("test"));

166          }

167          break;

168      }

169    }

170  }

171

172  // One picture of the current screen: handles its keys, draws it and switches to another

173  // screen when needed (the screen state machine).

174  void drawScreen(uint32_t now) {

175    uint32_t t = now - screenStart;

176    Screen next = screen;

177    switch (screen) {

178      case SCR_LOGO: {

179        uint16_t skipKeys = keysPressed & ~(keyBit(KEY_BRIGHTNESS) | keyBit(KEY_MUTE));

180        if (logoUpdate(frameNo) || skipKeys) next = SCR_TITLE;   // any other key skips the logo

181        break;

182      }

183      case SCR_TITLE:

184        titleDraw(t, frameNo);

185        if (keyPressed(KEY_FIRE)) {

186          soundBeep(2000, 60);

187          next = SCR_MENU;

188        } else if (t >= DEMO_AFTER_MS) {   // nobody is playing: show a demo game

189          demoMode = true;

190          autopilot = true;

191          gameStart(0);

192          next = SCR_GAME;

193          Serial.println(F("[VR] demo"));

194        }

195        break;

196      case SCR_MENU: {

197        int8_t choice = menuUpdate();

198        if (choice == MENU_BACK) next = SCR_TITLE;

199        else if (choice == MENU_ITEM_PLAY) {

200          gameStart(0);

201          next = SCR_GAME;

202        }

203        else if (choice == MENU_ITEM_SHIP) next = SCR_HANGAR;

204        else if (choice == MENU_ITEM_OPTS) next = SCR_OPTIONS;

205        else if (choice == MENU_ITEM_BEST) next = SCR_BEST;

206        else menuDraw();

207        break;

208      }

209      case SCR_HANGAR:   next = hangarFrame(now, frameNo); break;

210      case SCR_OPTIONS:  next = optionsFrame(frameNo); break;

211      case SCR_BEST:     next = bestFrame(frameNo); break;

212      case SCR_GAME: {

213        if (demoMode) {                    // demo: any key (but B and M) goes back to the title

214          uint16_t anyKey = keysPressed & ~(keyBit(KEY_BRIGHTNESS) | keyBit(KEY_MUTE));

215          if (anyKey || gameFrame(frameNo) != GAME_ON) next = SCR_TITLE;

216          break;

217        }

218        if (keyPressed(KEY_PAUSE) || keyPressed(KEY_BACK)) {

219          next = SCR_PAUSE;

220          break;

221        }

222        GameResult result = gameFrame(frameNo);

223        if (result == GAME_OVER) next = SCR_GAMEOVER;

224        if (result == GAME_WON) next = SCR_VICTORY;

225        break;

226      }

227      case SCR_PAUSE:

228        pauseDraw(t, frameNo);

229        if (keyPressed(KEY_PAUSE) || keyPressed(KEY_FIRE)) next = SCR_GAME;

230        if (keyPressed(KEY_BACK)) {

231          recordScore(game.phase + 1);

232          next = SCR_MENU;

233        }

234        break;

235      case SCR_GAMEOVER: next = gameOverFrame(t, frameNo); break;

236      case SCR_VICTORY:  next = victoryFrame(t); break;

237      case SCR_LEDTEST:  next = ledTestFrame(t); break;

238    }

239    if (next != screen) goTo(next);

240  }

241

242  // Runs once at power-on: Serial, LED screen, keypad, buzzers and the saved settings,

243  // then the start-up logo.

244  void setup() {

245    Serial.begin(115200);

246    displayBegin();

247    inputBegin();

248    soundBegin();

249    settingsLoad();

250    applyBrightness();

251    soundSetMuted(settings.muted);

252    randomSeed(analogRead(A15) ^ micros());

253    Serial.println(F("[VR] VOID RUNNER"));

254    Serial.print(F("[VR] saved: best="));

255    Serial.print(settings.best);

256    Serial.print(F(" ship="));

257    Serial.print(settings.ship);

258    Serial.print(F(" difficulty="));

259    Serial.println(settings.difficulty);

260    Serial.println(F("[VR] ready"));

261    goTo(SCR_LOGO);

262  }

263

264  // Runs over and over: reads the keypad, keeps the music going, reads test commands, and

265  // every 20 ms draws a new picture.

266  void loop() {

267    uint32_t now = millis();

268    inputUpdate(now);

269    soundUpdate(now);

270    serialCommands();

271    static uint32_t lastFrame = 0;

272    if (now - lastFrame >= FRAME_MS) {     // one new picture every 20 ms

273      lastFrame = now;

274      frameNo++;

275      uint32_t workStart = micros();

276      globalKeys();

277      drawScreen(now);

278      present();

279      inputEndFrame();

280      frameWorkUs = micros() - workStart;

281      if (frameWorkUs > frameWorkMaxUs) frameWorkMaxUs = frameWorkUs;

282    }

283  }

## 4.2 config.h – Settings

All settings in one place: the frame time, the names of the keys, the list of screens, the difficulty tables (lives, enemy fire speed, boss health, ammo per crate) and game constants such as the number of lives and the phase bonus. Changing the balance of the game only needs this file.

### Source code

1  /*

2   * config.h -- settings of VOID RUNNER in one place (timing, keys, game balance).

3   */

4  #pragma once

5  #include <Arduino.h>

6

7  // ---- timing

8  const uint8_t FRAME_MS   = 20;           // a new picture is drawn every 20 ms (50 per second)

9  const uint8_t REFRESH_HZ = 71;           // the LED matrix is scanned 71 times per second

10

11  // ---- brightness levels (key B): % of its time slot that each LED row stays lit

12  const uint8_t BRIGHTNESS_LEVELS[] = {100, 50, 20};

13

14  // ---- keys (labels of the keypad buttons = PC keyboard keys)

15  const char KEY_UP = 'W', KEY_LEFT = 'A', KEY_DOWN = 'S', KEY_RIGHT = 'D';

16  const char KEY_FIRE = 'R';               // fire, and "OK" in menus

17  const char KEY_BACK = 'Q';

18  const char KEY_PAUSE = 'P';

19  const char KEY_MUTE = 'M';

20  const char KEY_BRIGHTNESS = 'B';

21

22  // ---- the screens of the game

23  enum Screen : uint8_t {

24    SCR_LOGO, SCR_TITLE, SCR_MENU, SCR_HANGAR, SCR_OPTIONS, SCR_BEST,

25    SCR_GAME, SCR_PAUSE, SCR_GAMEOVER, SCR_VICTORY, SCR_LEDTEST

26  };

27

28  // ---- difficulty (OPTS): lives at the start, enemy fire speed, ammo per pack

29  enum Difficulty : uint8_t { DIFF_EASY, DIFF_NORMAL, DIFF_HARD };

30  const uint8_t DIFF_LIVES[3]      = {4, 3, 3};

31  const uint8_t DIFF_FIRE_SCALE[3] = {150, 100, 75};   // % of the enemy fire interval

32  const uint8_t DIFF_BOSS_HP[3]    = {80, 100, 130};   // % of the boss health

33  const uint8_t DIFF_PACK_AMMO[3]  = {20, 15, 12};

34

35  // ---- player

36  const uint8_t MAX_LIVES = 4;

37  const uint8_t LOW_AMMO = 5;              // warning beep + blinking ammo bar from here

38  const uint16_t MOVE_REPEAT_DELAY = 170;  // hold a move key this long before it glides

39  const uint16_t HIT_SHIELD_MS = 2000;     // blinking, can't be hit, after a hit

40  const uint8_t PLAYER_SHOT_MS = 35;       // ms per pixel of a player bullet

41

42  // ---- scoring

43  const uint16_t PHASE_BONUS = 100;        // x phase number, at the end of each phase

## 4.3 pins.h – Pin numbers (generated)

Generated together with the circuit by tools/gen_circuit.py, so the pins in the code always match the wires of the circuit: the 27 LED row pins, the 16 LED column pins (the bits of PORTF and PORTK), the keypad row and column pins and the four buzzer pins.

Figure 20. Belongs to pins.h.

### Source code

1  /*

2   * pins.h -- GENERATED by tools/gen_circuit.py together with diagram.json.

3   * Do not edit by hand: change the generator and run it again, so the circuit

4   * and the code always match.

5   */

6  #pragma once

7  #include <Arduino.h>

8

9  // ---- LED screen: 16 columns x 24 game rows + 3 status bars, one big matrix

10  const uint8_t SCREEN_W    = 16;

11  const uint8_t SCREEN_H    = 24;

12  const uint8_t STATUS_ROWS = 3;

13  const uint8_t TOTAL_ROWS  = SCREEN_H + STATUS_ROWS;

14

15  // Anode pin of every LED row: rows 0-23 = game screen top to bottom,

16  // rows 24-26 = status bars top to bottom (all on the double header).

17  const uint8_t ROW_PINS[TOTAL_ROWS] = {

18    44, 45, 42, 43, 40, 41, 38, 39, 36, 37, 34, 35, 32, 33, 30, 31, 28, 29, 26, 27, 24, 25, 22, 23, 48, 49, 47

19  };

20

21  // Cathode pin of every LED column, left to right: column x is wired to Ax,

22  // so columns 0-7 are PORTF bits 0-7 and columns 8-15 are PORTK bits 0-7.

23  const uint8_t COL_PINS[SCREEN_W] = {

24    A0, A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15

25  };

26

27  // ---- 3 x 3 push-button keypad: contact 1 = row line, contact 2 = column line

28  const uint8_t KEY_ROWS = 3, KEY_COLS = 3;

29  const uint8_t KEY_ROW_PINS[KEY_ROWS] = { 18, 19, 20 };

30  const uint8_t KEY_COL_PINS[KEY_COLS] = { 17, 16, 15 };

31  const char KEY_NAMES[KEY_ROWS][KEY_COLS] = {

32    {'Q', 'W', 'R'},

33    {'A', 'S', 'D'},

34    {'P', 'M', 'B'}

35  };

36

37  // ---- buzzers, each on the "A" output of its own 16-bit timer

38  const uint8_t BUZZER_COUNT = 4;

39  const uint8_t BUZZER_PINS[BUZZER_COUNT] = { 5, 6, 11, 46 };   // Timer3 OC3A, Timer4 OC4A, Timer1 OC1A, Timer5 OC5A

## 4.4 display.h – The LED screen

The Timer2 interrupt that lights one LED row at a time (71 full pictures per second), the picture in memory (frame) and every drawing function: pixels, rectangles, bitmaps, letters of the two fonts, numbers, message pages and the status bars. Brightness (100 / 50 / 20 %) is made by switching the row off early in its time slot.

Figure 21. Belongs to display.h.

| Function | What it does |
| --- | --- |
| displayBegin() | Starts the LED screen: row and column pins as outputs with every LED off, then Timer2 in CTC mode, so the scan interrupt runs 27 x 71 times per second. |
| displaySetBrightness(uint8_t percent) | Sets how long each row stays lit inside its time slot (100 = all of it). Below 100 % the second Timer2 match (COMPB) switches the row off early. |
| present() | Copies the finished picture to the scan buffer in one go, so the interrupt never shows a half-drawn picture. |
| clearScreen() | Switches off every LED of the game screen in `frame` (the status bars stay). |
| setPixel(int16_t x, int16_t y, bool on) | Turns one LED of the game screen on or off in `frame`, shifted by the screen shake. Pixels outside the screen are ignored, so sprites may stick out at the edges. |
| fillRect(int16_t x, int16_t y, int16_t w, int16_t h, bool on) | Turns a w x h block of LEDs on or off (bars, ribbons, clearing an area). |
| drawBitmap(int16_t x, int16_t y, const uint16_t* rows, uint8_t w, uint8_t h, bool on) | Bitmaps are stored in flash, one 16-bit word per row, leftmost pixel = top bit. |
| drawChar(int16_t x, int16_t y, char ch, bool big, uint8_t scale = 1) | small = 3x5 font (4 pixels per letter), big = 5x7 font (6 pixels per letter). Strings are kept in flash: call with PSTR("TEXT"). Draws one letter or digit with its top-left at (x, y); unknown characters are skipped. scale = 2 draws every pixel as a 2 x 2 block (the big phase number). |
| textWidth(const char* text, bool big) | Width in pixels of a text in the small or big font (for centring). |
| drawText(int16_t x, int16_t y, const char* text, bool big) | Draws a text from flash, letter by letter; letters outside the screen are skipped. |
| drawTextCentered(int16_t y, const char* text, bool big) | Draws a text centred horizontally at row y. |
| pageAt(uint32_t t, uint8_t count) | Which page of a sequence of `count` pages to show t ms after it started (the last one stays). |
| clearRows(int16_t y, uint8_t h) | Switches off h rows of the game screen from row y (a clean background for a page). |
| drawPage(const char* line1, const char* line2) | Draws a page of one or two short lines (from flash) in the white zone, rows 6-17, which is cleared first. line2 may be NULL. |
| numberDigits(uint16_t n) | Counts the digits of a number (for its width). |
| numberWidth(uint16_t n, bool big) | Width in pixels of a number in the small or big font. |
| drawNumber(int16_t x, int16_t y, uint16_t n, bool big) | Draws a number digit by digit, starting from the right (no leading zeros). |
| drawNumberCentered(int16_t y, uint16_t n, bool big) | Draws a number centred horizontally at row y. |
| drawBitmapScaled(int16_t x, int16_t y, const uint16_t* rows, uint8_t w, uint8_t h, uint8_t scale) | Bitmap drawn `scale` times bigger (ship preview in the hangar). |
| setStatusBits(uint8_t bar, uint16_t bits) | Three bars of 16 LEDs above the game screen (matrix rows 24, 25, 26). Sets all 16 LEDs of one bar at once (bit x = LED x). |
| setStatusBar(uint8_t bar, uint8_t level) | Fills a status bar from the left with `level` LEDs (0-16), like a gauge. |
| displayDump() | Prints the picture as text over Serial ('#' = on), status bars first. Used by the automatic tests (command !d). |

### Source code

1  /*

2   * display.h -- the LED screen.

3   *

4   * 16 x 24 game screen + 3 status bars of single LEDs, wired as ONE matrix:

5   * 27 rows (LED anodes, pins 22-48) and 16 columns (LED cathodes, A0-A15).

6   *

7   * The picture lives in `frame`: one 16-bit word per LED row, bit x = column x.

8   * A Timer2 interrupt lights one row at a time; all 27 rows are shown 71 times per

9   * second, so the picture stays steady whatever the game code is doing.

10   *

11   * Blinking: Wokwi shows each LED's brightness averaged over 100 ms, so anything that

12   * should visibly blink stays on and off for longer than that (frameNo & 8 = 160 ms).

13   */

14  #pragma once

15  #include "pins.h"

16  #include "config.h"

17  #include "font.h"

18

19  uint16_t frame[TOTAL_ROWS];              // the game draws here

20  volatile uint16_t shown[TOTAL_ROWS];     // the scan interrupt shows this copy

21  volatile uint8_t scanRow = 0;            // row that is lit right now

22  const uint8_t SCAN_TICK_US = 16;         // Timer2 runs at 16 MHz / 256

23

24  // ================================================================= scanning

25  // Every Timer2 match: old row off, columns of the next row, next row on.

26  // Column x is wired to pin Ax, so A0-A7 are the bits of PORTF and A8-A15 the bits of

27  // PORTK: 8 columns are written in one instruction (12 us per row instead of 140 us with

28  // digitalWrite, measured in Stage 0).

29  ISR(TIMER2_COMPA_vect) {

30    uint8_t row = scanRow;

31    digitalWrite(ROW_PINS[row], LOW);

32    row = (row + 1 == TOTAL_ROWS) ? 0 : row + 1;

33    uint16_t bits = shown[row];

34    PORTF = (uint8_t)~lowByte(bits);       // a LOW column lights its LED

35    PORTK = (uint8_t)~highByte(bits);

36    digitalWrite(ROW_PINS[row], HIGH);

37    scanRow = row;

38  }

39

40  // Brightness: a second match part way through the time slot switches the row off early.

41  ISR(TIMER2_COMPB_vect) {

42    digitalWrite(ROW_PINS[scanRow], LOW);

43  }

44

45  // Starts the LED screen: row and column pins as outputs with every LED off, then Timer2

46  // in CTC mode, so the scan interrupt runs 27 x 71 times per second.

47  void displayBegin() {

48    for (uint8_t r = 0; r < TOTAL_ROWS; r++) {

49      pinMode(ROW_PINS[r], OUTPUT);

50      digitalWrite(ROW_PINS[r], LOW);

51    }

52    for (uint8_t x = 0; x < SCREEN_W; x++) {

53      pinMode(COL_PINS[x], OUTPUT);

54      digitalWrite(COL_PINS[x], HIGH);

55    }

56    TCCR2A = _BV(WGM21);                   // CTC mode: count 0..OCR2A, then start again

57    TCCR2B = _BV(CS22) | _BV(CS21);        // prescaler 256: one count = 16 us

58    OCR2A = 1000000UL / ((uint32_t)REFRESH_HZ * TOTAL_ROWS * SCAN_TICK_US) - 1;

59    TIMSK2 = _BV(OCIE2A);

60  }

61

62  // Sets how long each row stays lit inside its time slot (100 = all of it). Below 100 %

63  // the second Timer2 match (COMPB) switches the row off early.

64  void displaySetBrightness(uint8_t percent) {

65    if (percent >= 100) {

66      TIMSK2 &= ~_BV(OCIE2B);

67      return;

68    }

69    OCR2B = (uint16_t)(OCR2A + 1) * percent / 100;

70    TIMSK2 |= _BV(OCIE2B);

71  }

72

73  // Copies the finished picture to the scan buffer in one go, so the interrupt never

74  // shows a half-drawn picture.

75  void present() {

76    noInterrupts();

77    for (uint8_t r = 0; r < TOTAL_ROWS; r++) shown[r] = frame[r];

78    interrupts();

79  }

80

81  // ================================================================= drawing (game screen)

82  // Switches off every LED of the game screen in `frame` (the status bars stay).

83  void clearScreen() {

84    for (uint8_t y = 0; y < SCREEN_H; y++) frame[y] = 0;

85  }

86

87  int8_t shakeX = 0, shakeY = 0;           // screen shake: everything is drawn shifted

88

89  // Turns one LED of the game screen on or off in `frame`, shifted by the screen shake.

90  // Pixels outside the screen are ignored, so sprites may stick out at the edges.

91  void setPixel(int16_t x, int16_t y, bool on) {

92    x += shakeX;

93    y += shakeY;

94    if (x < 0 || x >= SCREEN_W || y < 0 || y >= SCREEN_H) return;

95    if (on) frame[y] |= 1u << x;

96    else frame[y] &= ~(1u << x);

97  }

98

99  // Turns a w x h block of LEDs on or off (bars, ribbons, clearing an area).

100  void fillRect(int16_t x, int16_t y, int16_t w, int16_t h, bool on) {

101    for (int16_t j = y; j < y + h; j++)

102      for (int16_t i = x; i < x + w; i++) setPixel(i, j, on);

103  }

104

105  // Bitmaps are stored in flash, one 16-bit word per row, leftmost pixel = top bit.

106  void drawBitmap(int16_t x, int16_t y, const uint16_t* rows, uint8_t w, uint8_t h, bool on) {

107    for (uint8_t r = 0; r < h; r++) {

108      uint16_t bits = pgm_read_word(rows + r);

109      for (uint8_t c = 0; c < w; c++)

110        if (bits & (0x8000u >> c)) setPixel(x + c, y + r, on);

111    }

112  }

113

114  // ================================================================= text

115  // small = 3x5 font (4 pixels per letter), big = 5x7 font (6 pixels per letter).

116  // Strings are kept in flash: call with PSTR("TEXT").

117  // Draws one letter or digit with its top-left at (x, y); unknown characters are skipped.

118  // scale = 2 draws every pixel as a 2 x 2 block (the big phase number).

119  void drawChar(int16_t x, int16_t y, char ch, bool big, uint8_t scale = 1) {

120    const char* chars = big ? FONT5x7_CHARS : FONT3x5_CHARS;

121    const char* hit = ch ? strchr_P(chars, ch) : NULL;

122    if (!hit) return;

123    uint8_t w = big ? 5 : 3, h = big ? 7 : 5;

124    const uint8_t* glyph = (big ? FONT5x7 : FONT3x5) + (hit - chars) * w;

125    for (uint8_t c = 0; c < w; c++) {

126      uint8_t bits = pgm_read_byte(glyph + c);

127      for (uint8_t r = 0; r < h; r++)

128        if (bits & (1 << r)) fillRect(x + c * scale, y + r * scale, scale, scale, true);

129    }

130  }

131

132  // Width in pixels of a text in the small or big font (for centring).

133  int16_t textWidth(const char* text, bool big) {

134    uint8_t n = strlen_P(text);

135    return n ? n * (big ? 6 : 4) - 1 : 0;

136  }

137

138  // Draws a text from flash, letter by letter; letters outside the screen are skipped.

139  void drawText(int16_t x, int16_t y, const char* text, bool big) {

140    uint8_t step = big ? 6 : 4;

141    for (const char* p = text; char ch = pgm_read_byte(p); p++, x += step)

142      if (x > -step && x < SCREEN_W) drawChar(x, y, ch, big);

143  }

144

145  // Draws a text centred horizontally at row y.

146  void drawTextCentered(int16_t y, const char* text, bool big) {

147    drawText((SCREEN_W - textWidth(text, big)) / 2, y, text, big);

148  }

149

150  // ================================================================= pages

151  // Messages are shown as still "pages" of short words (up to 4 small letters per line),

152  // one after another, never as scrolling text: Wokwi averages each LED's brightness, so

153  // moving letters smear into a white block that nobody can read.

154  const uint16_t PAGE_MS = 900;            // how long one page stays

155

156  // Which page of a sequence of `count` pages to show t ms after it started (the last one

157  // stays).

158  uint8_t pageAt(uint32_t t, uint8_t count) {

159    uint32_t page = t / PAGE_MS;

160    return page < count ? page : count - 1;

161  }

162

163  // Switches off h rows of the game screen from row y (a clean background for a page).

164  void clearRows(int16_t y, uint8_t h) {

165    fillRect(0, y, SCREEN_W, h, false);

166  }

167

168  // Draws a page of one or two short lines (from flash) in the white zone, rows 6-17,

169  // which is cleared first. line2 may be NULL.

170  void drawPage(const char* line1, const char* line2) {

171    clearRows(6, 12);

172    if (line2) {

173      drawTextCentered(7, line1, false);

174      drawTextCentered(13, line2, false);

175    } else {

176      drawTextCentered(10, line1, false);

177    }

178  }

179

180  // ================================================================= numbers

181  // Counts the digits of a number (for its width).

182  uint8_t numberDigits(uint16_t n) {

183    uint8_t d = 1;

184    while (n >= 10) {

185      n /= 10;

186      d++;

187    }

188    return d;

189  }

190

191  // Width in pixels of a number in the small or big font.

192  int16_t numberWidth(uint16_t n, bool big) {

193    return numberDigits(n) * (big ? 6 : 4) - 1;

194  }

195

196  // Draws a number digit by digit, starting from the right (no leading zeros).

197  void drawNumber(int16_t x, int16_t y, uint16_t n, bool big) {

198    uint8_t step = big ? 6 : 4;

199    for (int8_t i = numberDigits(n) - 1; i >= 0; i--) {

200      drawChar(x + i * step, y, '0' + n % 10, big);

201      n /= 10;

202    }

203  }

204

205  // Draws a number centred horizontally at row y.

206  void drawNumberCentered(int16_t y, uint16_t n, bool big) {

207    drawNumber((SCREEN_W - numberWidth(n, big)) / 2, y, n, big);

208  }

209

210  // Bitmap drawn `scale` times bigger (ship preview in the hangar).

211  void drawBitmapScaled(int16_t x, int16_t y, const uint16_t* rows, uint8_t w, uint8_t h, uint8_t scale) {

212    for (uint8_t r = 0; r < h; r++) {

213      uint16_t bits = pgm_read_word(rows + r);

214      for (uint8_t c = 0; c < w; c++)

215        if (bits & (0x8000u >> c)) fillRect(x + c * scale, y + r * scale, scale, scale, true);

216    }

217  }

218

219  // ================================================================= status bars

220  // Three bars of 16 LEDs above the game screen (matrix rows 24, 25, 26).

221  // Sets all 16 LEDs of one bar at once (bit x = LED x).

222  void setStatusBits(uint8_t bar, uint16_t bits) {

223    frame[SCREEN_H + bar] = bits;

224  }

225

226  // Fills a status bar from the left with `level` LEDs (0-16), like a gauge.

227  void setStatusBar(uint8_t bar, uint8_t level) {

228    setStatusBits(bar, level >= 16 ? 0xFFFF : (uint16_t)((1u << level) - 1));

229  }

230

231  // ================================================================= test helper

232  // Prints the picture as text over Serial ('#' = on), status bars first. Used by the

233  // automatic tests (command !d).

234  void displayDump() {

235    Serial.println(F("[DUMP] begin"));

236    for (uint8_t i = 0; i < TOTAL_ROWS; i++) {

237      uint8_t r = (i < STATUS_ROWS) ? SCREEN_H + i : i - STATUS_ROWS;   // top to bottom

238      if (i == STATUS_ROWS) Serial.println(F("----------------"));

239      for (uint8_t x = 0; x < SCREEN_W; x++) Serial.print(frame[r] & (1u << x) ? '#' : '.');

240      Serial.println();

241    }

242    Serial.println(F("[DUMP] end"));

243  }

## 4.5 input.h – The keypad

Scans the 3 × 3 push-button matrix row by row, as taught in the course, and debounces every key. It remembers which keys are held and which were just pressed or released, so the other parts can ask "was R just pressed?" or "is D held?".

Figure 22. Belongs to input.h.

| Function | What it does |
| --- | --- |
| inputBegin() | Sets up the keypad pins: rows switched off (INPUT), columns with pull-up resistors. |
| keyBit(char key) | The bit of a key (by its label, e.g. 'R') in keysHeld and keysPressed. |
| keyHeld(char key) | True while the key is held down. |
| keyPressed(char key) | True only in the picture in which the key went down (one press = one action). |
| inputEndFrame() | Forgets this picture's key presses (called at the end of every picture). |
| readKeys() | Reads the whole keypad once: drives one row LOW at a time and reads the three columns. Returns one bit per key that is down (no debouncing here). |
| inputUpdate(uint32_t now) | Reads the keypad every 5 ms. A change only counts once the keys read the same four times in a row: that hides the contact bounce that Wokwi simulates. |

### Source code

1  /*

2   * input.h -- the 3 x 3 push-button keypad (the 9 keys of the game).

3   *

4   * Same method as the course: drive one row LOW and read the three columns

5   * (INPUT_PULLUP, so a pressed key reads LOW). Rows that are not being read are set to

6   * INPUT instead of HIGH: with two keys of one column pressed, a HIGH and a LOW output

7   * would otherwise be shorted together through the buttons.

8   */

9  #pragma once

10  #include "pins.h"

11

12  uint16_t keysHeld = 0;                   // debounced: bit (row * 4 + column) = key is down

13  uint16_t keysPressed = 0;                // keys that went down since the last picture

14

15  // Sets up the keypad pins: rows switched off (INPUT), columns with pull-up resistors.

16  void inputBegin() {

17    for (uint8_t r = 0; r < KEY_ROWS; r++) pinMode(KEY_ROW_PINS[r], INPUT);

18    for (uint8_t c = 0; c < KEY_COLS; c++) pinMode(KEY_COL_PINS[c], INPUT_PULLUP);

19  }

20

21  // The bit of a key (by its label, e.g. 'R') in keysHeld and keysPressed.

22  uint16_t keyBit(char key) {

23    for (uint8_t i = 0; i < KEY_ROWS * KEY_COLS; i++)

24      if (KEY_NAMES[i / KEY_COLS][i % KEY_COLS] == key) return 1u << i;

25    return 0;

26  }

27

28  // True while the key is held down.

29  bool keyHeld(char key) { return keysHeld & keyBit(key); }

30  // True only in the picture in which the key went down (one press = one action).

31  bool keyPressed(char key) { return keysPressed & keyBit(key); }

32  // Forgets this picture's key presses (called at the end of every picture).

33  void inputEndFrame() { keysPressed = 0; }

34

35  // Reads the whole keypad once: drives one row LOW at a time and reads the three columns.

36  // Returns one bit per key that is down (no debouncing here).

37  uint16_t readKeys() {

38    uint16_t mask = 0;

39    for (uint8_t r = 0; r < KEY_ROWS; r++) {

40      pinMode(KEY_ROW_PINS[r], OUTPUT);

41      digitalWrite(KEY_ROW_PINS[r], LOW);

42      delayMicroseconds(5);                // let the line settle

43      for (uint8_t c = 0; c < KEY_COLS; c++)

44        if (digitalRead(KEY_COL_PINS[c]) == LOW) mask |= 1u << (r * KEY_COLS + c);

45      pinMode(KEY_ROW_PINS[r], INPUT);

46    }

47    return mask;

48  }

49

50  // Reads the keypad every 5 ms. A change only counts once the keys read the same

51  // four times in a row: that hides the contact bounce that Wokwi simulates.

52  void inputUpdate(uint32_t now) {

53    static uint32_t lastScan = 0;

54    static uint16_t candidate = 0;

55    static uint8_t sameScans = 0;

56    if (now - lastScan < 5) return;

57    lastScan = now;

58    uint16_t raw = readKeys();

59    if (raw != candidate) {

60      candidate = raw;

61      sameScans = 0;

62      return;

63    }

64    if (sameScans < 3) sameScans++;

65    if (sameScans < 3 || candidate == keysHeld) return;

66    uint16_t changed = candidate ^ keysHeld;

67    keysPressed |= changed & candidate;

68    keysHeld = candidate;

69    for (uint8_t i = 0; i < KEY_ROWS * KEY_COLS; i++) {

70      if (!(changed & (1u << i))) continue;

71      Serial.print((keysHeld & (1u << i)) ? F("[KEY] DOWN ") : F("[KEY] UP "));

72      Serial.println(KEY_NAMES[i / KEY_COLS][i % KEY_COLS]);

73    }

74  }

## 4.6 sound.h – The sound engine

Drives the four buzzers with four 16-bit timers in CTC toggle mode, so each buzzer plays its note without using processor time. A small music engine plays songs written as note lists (one per voice, looping or once), and an effect engine on voice 3 plays the sound effects with priorities. Mute (M) silences everything.

Figure 23. Belongs to sound.h.

| Function | What it does |
| --- | --- |
| buzzerTone(uint8_t v, uint16_t hz) | Makes buzzer v play hz (0 = silent) by setting up its timer; does nothing if it already plays that frequency. |
| soundBegin() | Stops the timers Arduino starts by itself and makes the buzzer pins outputs. |
| noteHz(uint8_t pitch) | MIDI note number -> frequency: one octave of exact values, halved per octave down. |
| loadNote(Voice& v) | Copies the voice's current note from flash and works out its length in ms. |
| soundPlaySong(const Track* tracks, uint8_t tickMs, bool loop) | Starts a song: one track per buzzer (count 0 = that buzzer stays silent), tickMs = length of one tick, loop = start again at the end. |
| songTime() | ms since the song started; the logo uses it so its pictures land on the notes. |
| soundSfx(const SfxPart* parts, uint8_t count, uint8_t priority) | Starts a sound effect on buzzer 3, unless a more important one is playing. |
| soundStop() | Stops all music and effects and silences the four buzzers. |
| sfxUpdate(uint32_t now) | Plays the current effect on buzzer 3: moves on to the next part when one ends, slides the frequency, or jumps to a random frequency every 6 ms (noise for explosions). |
| soundEffect(uint8_t v, uint16_t hz) | An effect takes over buzzer v at hz until soundRelease (logo whoosh and crackle). |
| soundRelease(uint8_t v) | Gives buzzer v back to the music. |
| soundBeep(uint16_t hz, uint16_t ms) | A short beep on buzzer 3 (menus, alerts); it replaces any effect. |
| soundSetMuted(bool muted) | Turns all sound off or on (M key); the music keeps running silently. |
| soundUpdate(uint32_t now) | Call as often as possible: moves every voice on to its next note and drives the buzzers. |

### Source code

1  /*

2   * sound.h -- four buzzers = four voices.

3   *

4   * Each buzzer pin is the "A" output of its own 16-bit timer (pin 5 = Timer3, pin 6 =

5   * Timer4, pin 11 = Timer1, pin 46 = Timer5). In CTC mode with "toggle on compare match"

6   * the timer flips the pin by itself: a note costs no CPU time, and four notes can sound

7   * at once (Arduino's tone() can only play one). The Mega has no fifth free timer

8   * (Timer0 runs millis(), Timer2 scans the LED screen).

9   *

10   * Jobs: buzzer 1 = lead, buzzer 2 = bass, buzzer 4 = rhythm guitar / harmony, and

11   * buzzer 3 = every sound effect and menu sound (in the short jingles it plays music too). The timers run at 16 MHz / 256: notes up to

12   * about C7 stay within 1 % of their pitch, and Wokwi simulates this slow clock far

13   * faster than /8 (measured in Stage 0).

14   *

15   * Music is written as one list of notes per voice (MIDI note numbers, 60 = middle C)

16   * with lengths in "ticks". Effects (glides, beeps) take over a voice for a while.

17   */

18  #pragma once

19  #include "pins.h"

20

21  struct TimerRegs {

22    volatile uint8_t* tccrA;

23    volatile uint8_t* tccrB;

24    volatile uint16_t* ocrA;

25    volatile uint16_t* tcnt;

26  };

27  const TimerRegs VOICE_TIMER[BUZZER_COUNT] = {

28    { &TCCR3A, &TCCR3B, &OCR3A, &TCNT3 },  // pin 5  = OC3A

29    { &TCCR4A, &TCCR4B, &OCR4A, &TCNT4 },  // pin 6  = OC4A

30    { &TCCR1A, &TCCR1B, &OCR1A, &TCNT1 },  // pin 11 = OC1A

31    { &TCCR5A, &TCCR5B, &OCR5A, &TCNT5 },  // pin 46 = OC5A

32  };

33  const uint8_t SFX_VOICE = 2;             // buzzer 3 (pin 11): sound effects and beeps

34

35  struct Note { uint8_t pitch; uint8_t ticks; };   // pitch 0 = rest

36  struct Track { const Note* notes; uint8_t count; };

37

38  struct Voice {

39    const Note* notes;                     // track in flash (NULL = no music on this voice)

40    uint8_t count, index;

41    uint32_t noteStart;

42    uint16_t noteMs;

43    uint8_t pitch;

44    bool effect;                           // an effect has taken over the voice

45    uint16_t effectHz;

46    uint32_t effectEnd;                    // 0 = until released

47    uint16_t playingHz;                    // what the buzzer plays right now

48  };

49  Voice voices[BUZZER_COUNT];

50  uint8_t songTickMs = 50;

51  uint32_t songStart = 0;

52  bool songLoops = false;

53  bool soundMuted = false;

54  const uint8_t NOTE_GAP_MS = 12;          // short silence at the end of each note

55

56  // ================================================================= hardware

57  // Makes buzzer v play hz (0 = silent) by setting up its timer; does nothing if it already

58  // plays that frequency.

59  void buzzerTone(uint8_t v, uint16_t hz) {

60    if (soundMuted) hz = 0;

61    if (hz == voices[v].playingHz) return;

62    voices[v].playingHz = hz;

63    const TimerRegs& t = VOICE_TIMER[v];

64    *t.tccrB = 0;                          // stop the timer while changing it

65    if (hz == 0) {

66      *t.tccrA = 0;                        // disconnect the pin from the timer

67      digitalWrite(BUZZER_PINS[v], LOW);

68      return;

69    }

70    // Mode bits first: in Arduino's default PWM mode a new OCRnA value would wait for the

71    // end of a PWM cycle that a stopped timer never reaches, and the note would be lost.

72    *t.tccrA = _BV(COM1A0);                // toggle pin on match (same bit in timers 1, 3, 4, 5)

73    *t.tcnt = 0;

74    *t.ocrA = F_CPU / (2UL * 256 * hz) - 1;  // f = 16 MHz / (2 * 256 * (OCRnA + 1))

75    *t.tccrB = _BV(WGM12) | _BV(CS12);     // CTC mode, prescaler 256: start

76  }

77

78  // Stops the timers Arduino starts by itself and makes the buzzer pins outputs.

79  void soundBegin() {

80    // Arduino starts timers 1, 3, 4 and 5 even when nothing uses them, and Wokwi spends

81    // time on every timer count: stop them. A buzzer's timer restarts when it plays.

82    // Clearing the A registers too leaves Arduino's PWM mode (see buzzerTone).

83    TCCR1B = 0; TCCR1A = 0;

84    TCCR3B = 0; TCCR3A = 0;

85    TCCR4B = 0; TCCR4A = 0;

86    TCCR5B = 0; TCCR5A = 0;

87    for (uint8_t v = 0; v < BUZZER_COUNT; v++) pinMode(BUZZER_PINS[v], OUTPUT);

88  }

89

90  // MIDI note number -> frequency: one octave of exact values, halved per octave down.

91  uint16_t noteHz(uint8_t pitch) {

92    static const uint16_t OCTAVE8[12] PROGMEM = {4186, 4435, 4699, 4978, 5274, 5588,

93                                                 5920, 6272, 6645, 7040, 7459, 7902};

94    return pgm_read_word(&OCTAVE8[pitch % 12]) >> (9 - pitch / 12);

95  }

96

97  // ================================================================= music

98  // Copies the voice's current note from flash and works out its length in ms.

99  void loadNote(Voice& v) {

100    Note n;

101    memcpy_P(&n, &v.notes[v.index], sizeof(Note));

102    v.pitch = n.pitch;

103    v.noteMs = (uint16_t)n.ticks * songTickMs;

104  }

105

106  // Starts a song: one track per buzzer (count 0 = that buzzer stays silent), tickMs =

107  // length of one tick, loop = start again at the end.

108  void soundPlaySong(const Track* tracks, uint8_t tickMs, bool loop) {

109    uint32_t now = millis();

110    songTickMs = tickMs;

111    songStart = now;

112    songLoops = loop;

113    for (uint8_t i = 0; i < BUZZER_COUNT; i++) {

114      Voice& v = voices[i];

115      v.notes = tracks[i].count ? tracks[i].notes : NULL;

116      v.count = tracks[i].count;

117      v.index = 0;

118      v.noteStart = now;

119      if (v.notes) loadNote(v);

120    }

121  }

122

123  // ms since the song started; the logo uses it so its pictures land on the notes.

124  uint32_t songTime() {

125    return millis() - songStart;

126  }

127

128  // ================================================================= sound effects (buzzer 3)

129  // An effect is a short list of parts in flash. Each part slides from one frequency to

130  // another (fromHz == toHz: a steady tone, 0 Hz: a pause) or, with noise set, jumps to a

131  // random frequency in that range every few ms (explosions). A new effect only replaces

132  // the one playing if it is at least as important (priority).

133  struct SfxPart { uint16_t fromHz, toHz, ms; uint8_t noise; };

134

135  const SfxPart* sfxParts = NULL;

136  uint8_t sfxCount = 0, sfxIndex = 0, sfxPriority = 0;

137  SfxPart sfxPart;                         // copy of the part playing now

138  uint32_t sfxPartStart = 0, sfxNoiseAt = 0;

139  uint16_t sfxNoiseHz = 0;

140

141  // Starts a sound effect on buzzer 3, unless a more important one is playing.

142  void soundSfx(const SfxPart* parts, uint8_t count, uint8_t priority) {

143    if (sfxCount && priority < sfxPriority) return;

144    sfxParts = parts;

145    sfxCount = count;

146    sfxIndex = 0;

147    sfxPriority = priority;

148    sfxPartStart = millis();

149    memcpy_P(&sfxPart, &parts[0], sizeof(SfxPart));

150  }

151

152  // Stops all music and effects and silences the four buzzers.

153  void soundStop() {

154    sfxCount = 0;

155    for (uint8_t i = 0; i < BUZZER_COUNT; i++) {

156      voices[i].notes = NULL;

157      voices[i].effect = false;

158      buzzerTone(i, 0);

159    }

160  }

161

162  // Plays the current effect on buzzer 3: moves on to the next part when one ends, slides

163  // the frequency, or jumps to a random frequency every 6 ms (noise for explosions).

164  void sfxUpdate(uint32_t now) {

165    if (!sfxCount) return;

166    Voice& v = voices[SFX_VOICE];

167    while (now - sfxPartStart >= sfxPart.ms) {

168      sfxPartStart += sfxPart.ms;

169      if (++sfxIndex == sfxCount) {

170        sfxCount = 0;

171        v.effect = false;

172        return;

173      }

174      memcpy_P(&sfxPart, &sfxParts[sfxIndex], sizeof(SfxPart));

175    }

176    uint16_t hz;

177    if (sfxPart.noise) {

178      if ((int32_t)(now - sfxNoiseAt) >= 0) {

179        sfxNoiseHz = random(sfxPart.fromHz, sfxPart.toHz);

180        sfxNoiseAt = now + 6;

181      }

182      hz = sfxNoiseHz;

183    } else {

184      int32_t span = (int32_t)sfxPart.toHz - (int32_t)sfxPart.fromHz;

185      hz = sfxPart.fromHz + span * (int32_t)(now - sfxPartStart) / sfxPart.ms;

186    }

187    v.effect = true;

188    v.effectHz = hz;

189    v.effectEnd = 0;

190  }

191

192  // ================================================================= effects

193  // An effect takes over buzzer v at hz until soundRelease (logo whoosh and crackle).

194  void soundEffect(uint8_t v, uint16_t hz) {

195    voices[v].effect = true;

196    voices[v].effectHz = hz;

197    voices[v].effectEnd = 0;

198  }

199

200  // Gives buzzer v back to the music.

201  void soundRelease(uint8_t v) {

202    voices[v].effect = false;

203  }

204

205  // A short beep on buzzer 3 (menus, alerts); it replaces any effect.

206  void soundBeep(uint16_t hz, uint16_t ms) {

207    sfxCount = 0;                                    // a beep replaces any effect

208    Voice& v = voices[SFX_VOICE];

209    v.effect = true;

210    v.effectHz = hz;

211    v.effectEnd = millis() + ms;

212    if (v.effectEnd == 0) v.effectEnd = 1;

213  }

214

215  // Turns all sound off or on (M key); the music keeps running silently.

216  void soundSetMuted(bool muted) {

217    soundMuted = muted;

218    for (uint8_t i = 0; i < BUZZER_COUNT; i++) buzzerTone(i, 0);

219  }

220

221  // Call as often as possible: moves every voice on to its next note and drives the buzzers.

222  void soundUpdate(uint32_t now) {

223    sfxUpdate(now);

224    for (uint8_t i = 0; i < BUZZER_COUNT; i++) {

225      Voice& v = voices[i];

226      while (v.notes && now - v.noteStart >= v.noteMs) {   // next note (catch up if late)

227        v.noteStart += v.noteMs;

228        if (++v.index == v.count) {

229          if (!songLoops) {

230            v.notes = NULL;

231            break;

232          }

233          v.index = 0;

234        }

235        loadNote(v);

236      }

237      if (v.effect && v.effectEnd && (int32_t)(now - v.effectEnd) >= 0) v.effect = false;

238      uint16_t hz = 0;

239      if (v.effect) hz = v.effectHz;

240      else if (v.notes && v.pitch && now - v.noteStart < (uint32_t)v.noteMs - NOTE_GAP_MS)

241        hz = noteHz(v.pitch);

242      buzzerTone(i, hz);

243    }

244  }

## 4.7 tunes.h – Songs and sound effects

The note lists of every song and the parts of every sound effect, all stored in flash. The four in-game songs are generated by tools/compose_music.py from a short note notation; the jingles (fanfare, clear, victory, game over) and the effects are written here directly.

### Source code

1  /*

2   * tunes.h -- music and sound effects (all in flash).

3   *

4   * Songs: one note list per buzzer, MIDI note numbers (60 = middle C, 0 = rest) and

5   * lengths in ticks. The in-game songs are original metal tracks: buzzer 1 = lead guitar,

6   * buzzer 2 = bass, buzzer 4 = rhythm guitar (a fifth above the bass: a power chord).

7   * They leave buzzer 3 to the sound effects, so effects never cut the music. The songs are

8   * written in a short notation in tools/compose_music.py, which fills the block below.

9   * Effects: parts that slide between two frequencies (or noise for explosions).

10   * The start-up melody lives in logo.h.

11   */

12  #pragma once

13  #include "sound.h"

14

15  #define TRACK(notes) {notes, sizeof(notes) / sizeof(Note)}

16  #define NO_TRACK {NULL, 0}

17  #define SFX(parts) parts, sizeof(parts) / sizeof(SfxPart)

18

19  // ---- BEGIN songs generated by tools/compose_music.py (edit the songs there)

20  // ================================================================= Phase 1, ROCK BELT: thrash, E minor, galloping bass

21  // 8 bars, 1 tick = 80 ms, one loop = 10.2 s

22  const Note PHASE1_LEAD[] PROGMEM = {

23    {64, 4}, {67, 2}, {69, 2}, {71, 4}, {69, 2}, {67, 2}, {69, 4}, {67, 2},

24    {64, 2}, {62, 4}, {64, 4}, {64, 4}, {67, 2}, {69, 2}, {71, 4}, {74, 2},

25    {76, 2}, {70, 4}, {69, 4}, {67, 4}, {66, 4}, {76, 6}, {74, 2}, {71, 4},

26    {67, 4}, {69, 6}, {71, 2}, {67, 4}, {64, 4}, {76, 2}, {74, 2}, {71, 2},

27    {69, 2}, {67, 2}, {69, 2}, {71, 2}, {74, 2}, {66, 8}, {71, 8},

28  };

29  const Note PHASE1_BASS[] PROGMEM = {

30    {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1},

31    {40, 1}, {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1}, {40, 1}, {40, 2},

32    {40, 1}, {40, 1}, {43, 2}, {43, 1}, {43, 1}, {45, 2}, {45, 1}, {45, 1},

33    {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1},

34    {40, 1}, {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1}, {40, 1}, {40, 2},

35    {40, 1}, {40, 1}, {46, 4}, {45, 4}, {40, 2}, {40, 1}, {40, 1}, {40, 2},

36    {40, 1}, {40, 1}, {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1}, {40, 1},

37    {40, 2}, {40, 1}, {40, 1}, {40, 2}, {40, 1}, {40, 1}, {43, 2}, {43, 1},

38    {43, 1}, {45, 2}, {45, 1}, {45, 1}, {48, 2}, {48, 1}, {48, 1}, {48, 2},

39    {48, 1}, {48, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1},

40    {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 8},

41  };

42  const Note PHASE1_RHYTHM[] PROGMEM = {

43    {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1},

44    {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2},

45    {47, 1}, {47, 1}, {50, 2}, {50, 1}, {50, 1}, {52, 2}, {52, 1}, {52, 1},

46    {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1},

47    {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2},

48    {47, 1}, {47, 1}, {53, 4}, {52, 4}, {47, 2}, {47, 1}, {47, 1}, {47, 2},

49    {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1},

50    {47, 2}, {47, 1}, {47, 1}, {47, 2}, {47, 1}, {47, 1}, {50, 2}, {50, 1},

51    {50, 1}, {52, 2}, {52, 1}, {52, 1}, {55, 2}, {55, 1}, {55, 1}, {55, 2},

52    {55, 1}, {55, 1}, {54, 2}, {54, 1}, {54, 1}, {54, 2}, {54, 1}, {54, 1},

53    {54, 2}, {54, 1}, {54, 1}, {54, 2}, {54, 1}, {54, 1}, {54, 8},

54  };

55  const Track SONG_PHASE1[BUZZER_COUNT] = {TRACK(PHASE1_LEAD), TRACK(PHASE1_BASS), NO_TRACK, TRACK(PHASE1_RHYTHM)};

56  const uint8_t SONG_PHASE1_TICK_MS = 80;

57

58  // ================================================================= Phase 2, DEEP MIST: slow drop-D chugs, D minor

59  // 8 bars, 1 tick = 100 ms, one loop = 12.8 s

60  const Note PHASE2_LEAD[] PROGMEM = {

61    {0, 8}, {69, 4}, {65, 4}, {64, 8}, {62, 4}, {0, 4}, {0, 8}, {69, 4},

62    {72, 4}, {70, 8}, {69, 4}, {0, 4}, {74, 6}, {72, 2}, {69, 4}, {65, 4},

63    {67, 6}, {69, 2}, {64, 8}, {65, 4}, {64, 4}, {62, 4}, {61, 4}, {62, 12},

64    {0, 4},

65  };

66  const Note PHASE2_BASS[] PROGMEM = {

67    {38, 1}, {38, 1}, {0, 1}, {38, 1}, {38, 2}, {0, 2}, {38, 1}, {38, 1},

68    {0, 1}, {38, 1}, {41, 2}, {40, 2}, {38, 1}, {38, 1}, {0, 1}, {38, 1},

69    {38, 2}, {0, 2}, {45, 4}, {44, 2}, {43, 2}, {38, 1}, {38, 1}, {0, 1},

70    {38, 1}, {38, 2}, {0, 2}, {38, 1}, {38, 1}, {0, 1}, {38, 1}, {41, 2},

71    {40, 2}, {38, 1}, {38, 1}, {0, 1}, {38, 1}, {38, 2}, {0, 2}, {45, 4},

72    {44, 2}, {43, 2}, {38, 1}, {38, 1}, {0, 1}, {38, 1}, {38, 2}, {0, 2},

73    {38, 1}, {38, 1}, {0, 1}, {38, 1}, {41, 2}, {40, 2}, {38, 1}, {38, 1},

74    {0, 1}, {38, 1}, {38, 2}, {0, 2}, {45, 4}, {44, 2}, {43, 2}, {38, 1},

75    {38, 1}, {0, 1}, {38, 1}, {38, 2}, {0, 2}, {38, 1}, {38, 1}, {0, 1},

76    {38, 1}, {41, 2}, {40, 2}, {46, 8}, {45, 8},

77  };

78  const Note PHASE2_RHYTHM[] PROGMEM = {

79    {45, 1}, {45, 1}, {0, 1}, {45, 1}, {45, 2}, {0, 2}, {45, 1}, {45, 1},

80    {0, 1}, {45, 1}, {48, 2}, {47, 2}, {45, 1}, {45, 1}, {0, 1}, {45, 1},

81    {45, 2}, {0, 2}, {52, 4}, {51, 2}, {50, 2}, {45, 1}, {45, 1}, {0, 1},

82    {45, 1}, {45, 2}, {0, 2}, {45, 1}, {45, 1}, {0, 1}, {45, 1}, {48, 2},

83    {47, 2}, {45, 1}, {45, 1}, {0, 1}, {45, 1}, {45, 2}, {0, 2}, {52, 4},

84    {51, 2}, {50, 2}, {45, 1}, {45, 1}, {0, 1}, {45, 1}, {45, 2}, {0, 2},

85    {45, 1}, {45, 1}, {0, 1}, {45, 1}, {48, 2}, {47, 2}, {45, 1}, {45, 1},

86    {0, 1}, {45, 1}, {45, 2}, {0, 2}, {52, 4}, {51, 2}, {50, 2}, {45, 1},

87    {45, 1}, {0, 1}, {45, 1}, {45, 2}, {0, 2}, {45, 1}, {45, 1}, {0, 1},

88    {45, 1}, {48, 2}, {47, 2}, {53, 8}, {52, 8},

89  };

90  const Track SONG_PHASE2[BUZZER_COUNT] = {TRACK(PHASE2_LEAD), TRACK(PHASE2_BASS), NO_TRACK, TRACK(PHASE2_RHYTHM)};

91  const uint8_t SONG_PHASE2_TICK_MS = 100;

92

93  // ================================================================= Phase 3, THE VOID: syncopated groove, E minor

94  // 8 bars, 1 tick = 75 ms, one loop = 9.6 s

95  const Note PHASE3_LEAD[] PROGMEM = {

96    {71, 4}, {71, 2}, {67, 2}, {69, 4}, {64, 4}, {71, 4}, {71, 2}, {67, 2},

97    {72, 4}, {71, 4}, {71, 4}, {71, 2}, {67, 2}, {69, 4}, {64, 2}, {67, 2},

98    {66, 4}, {67, 4}, {69, 4}, {71, 4}, {76, 4}, {74, 2}, {71, 2}, {74, 4},

99    {76, 4}, {79, 4}, {78, 2}, {76, 2}, {74, 4}, {71, 4}, {76, 2}, {71, 2},

100    {67, 2}, {64, 2}, {76, 2}, {71, 2}, {67, 2}, {64, 2}, {76, 1}, {74, 1},

101    {71, 1}, {69, 1}, {67, 1}, {69, 1}, {71, 1}, {74, 1}, {76, 8},

102  };

103  const Note PHASE3_BASS[] PROGMEM = {

104    {40, 1}, {0, 1}, {40, 1}, {0, 1}, {40, 1}, {40, 1}, {0, 2}, {43, 2},

105    {40, 2}, {46, 2}, {45, 2}, {40, 1}, {0, 1}, {40, 1}, {0, 1}, {40, 1},

106    {40, 1}, {0, 2}, {48, 4}, {47, 4}, {40, 1}, {0, 1}, {40, 1}, {0, 1},

107    {40, 1}, {40, 1}, {0, 2}, {43, 2}, {40, 2}, {46, 2}, {45, 2}, {40, 1},

108    {0, 1}, {40, 1}, {0, 1}, {40, 1}, {40, 1}, {0, 2}, {48, 4}, {47, 4},

109    {40, 1}, {0, 1}, {40, 1}, {0, 1}, {40, 1}, {40, 1}, {0, 2}, {43, 2},

110    {40, 2}, {46, 2}, {45, 2}, {40, 1}, {0, 1}, {40, 1}, {0, 1}, {40, 1},

111    {40, 1}, {0, 2}, {48, 4}, {47, 4}, {40, 2}, {40, 2}, {40, 2}, {40, 2},

112    {40, 2}, {40, 2}, {40, 2}, {40, 2}, {40, 1}, {40, 1}, {40, 1}, {40, 1},

113    {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1},

114    {40, 1}, {40, 1}, {40, 1}, {40, 1},

115  };

116  const Note PHASE3_RHYTHM[] PROGMEM = {

117    {47, 1}, {0, 1}, {47, 1}, {0, 1}, {47, 1}, {47, 1}, {0, 2}, {50, 2},

118    {47, 2}, {53, 2}, {52, 2}, {47, 1}, {0, 1}, {47, 1}, {0, 1}, {47, 1},

119    {47, 1}, {0, 2}, {55, 4}, {54, 4}, {47, 1}, {0, 1}, {47, 1}, {0, 1},

120    {47, 1}, {47, 1}, {0, 2}, {50, 2}, {47, 2}, {53, 2}, {52, 2}, {47, 1},

121    {0, 1}, {47, 1}, {0, 1}, {47, 1}, {47, 1}, {0, 2}, {55, 4}, {54, 4},

122    {47, 1}, {0, 1}, {47, 1}, {0, 1}, {47, 1}, {47, 1}, {0, 2}, {50, 2},

123    {47, 2}, {53, 2}, {52, 2}, {47, 1}, {0, 1}, {47, 1}, {0, 1}, {47, 1},

124    {47, 1}, {0, 2}, {55, 4}, {54, 4}, {47, 2}, {47, 2}, {47, 2}, {47, 2},

125    {47, 2}, {47, 2}, {47, 2}, {47, 2}, {47, 1}, {47, 1}, {47, 1}, {47, 1},

126    {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1},

127    {47, 1}, {47, 1}, {47, 1}, {47, 1},

128  };

129  const Track SONG_PHASE3[BUZZER_COUNT] = {TRACK(PHASE3_LEAD), TRACK(PHASE3_BASS), NO_TRACK, TRACK(PHASE3_RHYTHM)};

130  const uint8_t SONG_PHASE3_TICK_MS = 75;

131

132  // ================================================================= Boss fights: tremolo bass, chromatic lead

133  // 4 bars, 1 tick = 70 ms, one loop = 4.5 s

134  const Note BOSS_LEAD[] PROGMEM = {

135    {76, 2}, {77, 2}, {76, 2}, {75, 2}, {76, 8}, {77, 2}, {79, 2}, {77, 2},

136    {76, 2}, {77, 8}, {79, 2}, {81, 2}, {82, 2}, {81, 2}, {79, 4}, {76, 4},

137    {70, 4}, {69, 4}, {68, 4}, {67, 4},

138  };

139  const Note BOSS_BASS[] PROGMEM = {

140    {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1},

141    {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1},

142    {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1},

143    {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1}, {41, 1},

144    {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1}, {40, 1},

145    {43, 1}, {43, 1}, {43, 1}, {43, 1}, {46, 1}, {46, 1}, {46, 1}, {46, 1},

146    {46, 1}, {46, 1}, {46, 1}, {46, 1}, {46, 1}, {46, 1}, {46, 1}, {46, 1},

147    {45, 1}, {45, 1}, {45, 1}, {45, 1}, {45, 1}, {45, 1}, {45, 1}, {45, 1},

148  };

149  const Note BOSS_RHYTHM[] PROGMEM = {

150    {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1},

151    {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1},

152    {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1},

153    {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1}, {48, 1},

154    {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1}, {47, 1},

155    {50, 1}, {50, 1}, {50, 1}, {50, 1}, {53, 1}, {53, 1}, {53, 1}, {53, 1},

156    {53, 1}, {53, 1}, {53, 1}, {53, 1}, {53, 1}, {53, 1}, {53, 1}, {53, 1},

157    {52, 1}, {52, 1}, {52, 1}, {52, 1}, {52, 1}, {52, 1}, {52, 1}, {52, 1},

158  };

159  const Track SONG_BOSS[BUZZER_COUNT] = {TRACK(BOSS_LEAD), TRACK(BOSS_BASS), NO_TRACK, TRACK(BOSS_RHYTHM)};

160  const uint8_t SONG_BOSS_TICK_MS = 70;

161  // ---- END songs generated by tools/compose_music.py

162

163  // ================================================================= short tunes (play once)

164  const Note STING_A[] PROGMEM = {{69, 2}, {76, 2}, {81, 4}};       // a phase begins

165  const Note STING_B[] PROGMEM = {{64, 2}, {69, 2}, {76, 4}};

166  const Note STING_C[] PROGMEM = {{45, 8}};

167  const Track SONG_STING[BUZZER_COUNT] = {TRACK(STING_A), TRACK(STING_B), TRACK(STING_C)};

168

169  const Note CLEAR_A[] PROGMEM = {{76, 2}, {79, 2}, {84, 6}};       // a phase is cleared

170  const Note CLEAR_B[] PROGMEM = {{72, 2}, {76, 2}, {79, 6}};

171  const Note CLEAR_C[] PROGMEM = {{48, 10}};

172  const Track SONG_CLEAR[BUZZER_COUNT] = {TRACK(CLEAR_A), TRACK(CLEAR_B), TRACK(CLEAR_C)};

173

174  const Note WIN_A[] PROGMEM = {{72, 2}, {76, 2}, {79, 2}, {84, 6}, {81, 2}, {83, 2}, {84, 8}};

175  const Note WIN_B[] PROGMEM = {{64, 2}, {67, 2}, {72, 2}, {76, 6}, {77, 2}, {79, 2}, {76, 8}};

176  const Note WIN_C[] PROGMEM = {{48, 6}, {55, 6}, {53, 4}, {48, 8}};

177  const Track SONG_VICTORY[BUZZER_COUNT] = {TRACK(WIN_A), TRACK(WIN_B), TRACK(WIN_C)};

178

179  const Note OVER_A[] PROGMEM = {{64, 4}, {63, 4}, {62, 4}, {61, 12}};

180  const Note OVER_B[] PROGMEM = {{57, 4}, {56, 4}, {55, 4}, {52, 12}};

181  const Note OVER_C[] PROGMEM = {{45, 12}, {40, 12}};

182  const Track SONG_GAMEOVER[BUZZER_COUNT] = {TRACK(OVER_A), TRACK(OVER_B), TRACK(OVER_C)};

183

184  // ================================================================= sound effects (buzzer 3)

185  // {from Hz, to Hz, ms, noise}; priorities: 1 shot ... 6 player hit

186  const SfxPart SFX_SHOT[] PROGMEM = {{2200, 1300, 35, 0}};

187  const SfxPart SFX_CHIRP[] PROGMEM = {{1500, 2600, 40, 0}};                  // auto-fire starts

188  const SfxPart SFX_EMPTY[] PROGMEM = {{2600, 2600, 8, 0}};                   // dry click

189  const SfxPart SFX_HIT[] PROGMEM = {{900, 650, 40, 0}};                      // enemy hit

190  const SfxPart SFX_BOOM[] PROGMEM = {{200, 900, 180, 1}};                    // enemy destroyed

191  const SfxPart SFX_BIG_BOOM[] PROGMEM = {{100, 600, 500, 1}, {600, 80, 300, 0}};

192  const SfxPart SFX_PLAYER_HIT[] PROGMEM = {{1200, 150, 350, 0}, {80, 300, 250, 1}};

193  const SfxPart SFX_PICKUP[] PROGMEM = {{800, 1600, 70, 0}, {1600, 2400, 70, 0}};

194  const SfxPart SFX_LIFE[] PROGMEM = {{660, 660, 80, 0}, {880, 880, 80, 0}, {1320, 1320, 140, 0}};

195  const SfxPart SFX_LOW_AMMO[] PROGMEM = {{1100, 1100, 60, 0}, {0, 0, 50, 0}, {1100, 1100, 60, 0}};

196  const SfxPart SFX_SIREN[] PROGMEM = {

197    {600, 1200, 250, 0}, {1200, 600, 250, 0}, {600, 1200, 250, 0},

198    {1200, 600, 250, 0}, {600, 1200, 250, 0}, {1200, 600, 250, 0},

199  };

200  const SfxPart SFX_ROAR[] PROGMEM = {{90, 400, 600, 1}};                     // boss changes form

201  const SfxPart SFX_TEST[] PROGMEM = {{400, 2000, 900, 0}};                   // LED test sweep

## 4.8 storage.h – EEPROM: settings and best score

Saves the chosen ship, the difficulty, the brightness, the sound setting and the best score (with the zone it reached and its difficulty) in the Mega's EEPROM, using the EEPROM registers directly. A check byte tells whether the EEPROM holds valid data.

Figure 24. What storage.h draws or uses.

| Function | What it does |
| --- | --- |
| eepromRead(uint16_t address) | Reads one byte of the EEPROM (first waits if a write is still busy). |
| eepromWrite(uint16_t address, uint8_t value) | Writes one byte to the EEPROM, only if it changed (saves time and EEPROM wear). |
| settingsSave() | Saves the settings and the best score: a marker byte, then the Settings struct. |
| settingsLoad() | Loads the settings at start-up. If the EEPROM holds none (new chip, new simulation) or a value is out of range, the defaults are used. |

### Source code

1  /*

2   * storage.h -- settings and the high score, kept in the Mega's EEPROM.

3   *

4   * Written with the EEPROM registers directly (no library):

5   *   EEAR = address, EEDR = data byte,

6   *   EECR: EERE starts a read; EEMPE then EEPE (within 4 clock cycles) starts a write.

7   * A write takes about 3.4 ms, so the next access waits while EEPE is still set.

8   * (Wokwi keeps the EEPROM while a simulation runs; a restart starts empty.)

9   */

10  #pragma once

11  #include "config.h"

12

13  struct Settings {

14    uint8_t brightness;                    // index into BRIGHTNESS_LEVELS

15    uint8_t muted;

16    uint8_t difficulty;

17    uint8_t ship;                          // chosen in the hangar

18    uint16_t best;                         // high score

19    uint8_t bestPhase;                     // phase reached with the high score (4 = won)

20    uint8_t bestDifficulty;                // difficulty the high score was made on

21  };

22  Settings settings;

23  const uint8_t EE_MAGIC = 0xA7;           // marks an EEPROM that holds our settings

24

25  // Reads one byte of the EEPROM (first waits if a write is still busy).

26  uint8_t eepromRead(uint16_t address) {

27    while (EECR & _BV(EEPE)) {}            // wait for a write in progress

28    EEAR = address;

29    EECR |= _BV(EERE);

30    return EEDR;

31  }

32

33  // Writes one byte to the EEPROM, only if it changed (saves time and EEPROM wear).

34  void eepromWrite(uint16_t address, uint8_t value) {

35    if (eepromRead(address) == value) return;   // saves EEPROM wear (and time)

36    EEAR = address;

37    EEDR = value;

38    noInterrupts();                        // EEPE must follow EEMPE within 4 cycles

39    EECR |= _BV(EEMPE);

40    EECR |= _BV(EEPE);

41    interrupts();

42  }

43

44  // Saves the settings and the best score: a marker byte, then the Settings struct.

45  void settingsSave() {

46    eepromWrite(0, EE_MAGIC);

47    const uint8_t* p = (const uint8_t*)&settings;

48    for (uint8_t i = 0; i < sizeof(Settings); i++) eepromWrite(1 + i, p[i]);

49  }

50

51  // Loads the settings at start-up. If the EEPROM holds none (new chip, new simulation) or

52  // a value is out of range, the defaults are used.

53  void settingsLoad() {

54    settings = {0, 0, DIFF_NORMAL, 0, 0, 0, DIFF_EASY};

55    if (eepromRead(0) == EE_MAGIC) {

56      uint8_t* p = (uint8_t*)&settings;

57      for (uint8_t i = 0; i < sizeof(Settings); i++) p[i] = eepromRead(1 + i);

58    }

59    if (settings.brightness >= sizeof(BRIGHTNESS_LEVELS)) settings.brightness = 0;

60    if (settings.difficulty > DIFF_HARD) settings.difficulty = DIFF_NORMAL;

61    if (settings.ship > 2) settings.ship = 0;

62    if (settings.bestDifficulty > DIFF_HARD) settings.bestDifficulty = DIFF_EASY;

63    settings.muted = settings.muted ? 1 : 0;

64  }

## 4.9 font.h – Fonts (generated)

Two pixel fonts: small 3 × 5 letters (four fit on a line) and big 5 × 7 letters and digits. They are drawn as ASCII art in tools/gen_font.py, which writes this file.

Figure 25. What font.h draws or uses.

### Source code

1  /*

2   * font.h -- GENERATED by tools/gen_font.py (the letters are drawn there as ASCII art).

3   * Each glyph is stored column by column, one byte per column, bit 0 = top row.

4   */

5  #pragma once

6  #include <avr/pgmspace.h>

7

8  // 3 x 5 font: glyph i is the i-th character of FONT3x5_CHARS

9  const char FONT3x5_CHARS[] PROGMEM = "ABCDEFGHIJKLMNOPQRSTUVWXYZ0123456789 -!:.>";

10  const uint8_t FONT3x5[] PROGMEM = {

11    0x1E, 0x05, 0x1E,  // 'A'

12    0x1F, 0x15, 0x0A,  // 'B'

13    0x0E, 0x11, 0x11,  // 'C'

14    0x1F, 0x11, 0x0E,  // 'D'

15    0x1F, 0x15, 0x11,  // 'E'

16    0x1F, 0x05, 0x01,  // 'F'

17    0x0E, 0x11, 0x1D,  // 'G'

18    0x1F, 0x04, 0x1F,  // 'H'

19    0x11, 0x1F, 0x11,  // 'I'

20    0x08, 0x10, 0x0F,  // 'J'

21    0x1F, 0x04, 0x1B,  // 'K'

22    0x1F, 0x10, 0x10,  // 'L'

23    0x1F, 0x06, 0x1F,  // 'M'

24    0x1F, 0x01, 0x1E,  // 'N'

25    0x0E, 0x11, 0x0E,  // 'O'

26    0x1F, 0x05, 0x02,  // 'P'

27    0x0E, 0x19, 0x16,  // 'Q'

28    0x1F, 0x05, 0x1A,  // 'R'

29    0x12, 0x15, 0x09,  // 'S'

30    0x01, 0x1F, 0x01,  // 'T'

31    0x1F, 0x10, 0x1F,  // 'U'

32    0x0F, 0x10, 0x0F,  // 'V'

33    0x1F, 0x0C, 0x1F,  // 'W'

34    0x1B, 0x04, 0x1B,  // 'X'

35    0x03, 0x1C, 0x03,  // 'Y'

36    0x19, 0x15, 0x13,  // 'Z'

37    0x1F, 0x11, 0x1F,  // '0'

38    0x12, 0x1F, 0x10,  // '1'

39    0x1D, 0x15, 0x17,  // '2'

40    0x15, 0x15, 0x1F,  // '3'

41    0x07, 0x04, 0x1F,  // '4'

42    0x17, 0x15, 0x1D,  // '5'

43    0x1F, 0x15, 0x1D,  // '6'

44    0x01, 0x1D, 0x03,  // '7'

45    0x1F, 0x15, 0x1F,  // '8'

46    0x17, 0x15, 0x1F,  // '9'

47    0x00, 0x00, 0x00,  // ' '

48    0x04, 0x04, 0x04,  // '-'

49    0x00, 0x17, 0x00,  // '!'

50    0x00, 0x0A, 0x00,  // ':'

51    0x00, 0x10, 0x00,  // '.'

52    0x1F, 0x0E, 0x04,  // '>'

53  };

54

55  // 5 x 7 font: glyph i is the i-th character of FONT5x7_CHARS

56  const char FONT5x7_CHARS[] PROGMEM = " !-.:0123456789ABCDEFGHIJKLMNOPQRSTUVWXYZ";

57  const uint8_t FONT5x7[] PROGMEM = {

58    0x00, 0x00, 0x00, 0x00, 0x00,  // ' '

59    0x00, 0x00, 0x5F, 0x00, 0x00,  // '!'

60    0x08, 0x08, 0x08, 0x08, 0x08,  // '-'

61    0x00, 0x60, 0x60, 0x00, 0x00,  // '.'

62    0x00, 0x36, 0x36, 0x00, 0x00,  // ':'

63    0x3E, 0x51, 0x49, 0x45, 0x3E,  // '0'

64    0x00, 0x42, 0x7F, 0x40, 0x00,  // '1'

65    0x42, 0x61, 0x51, 0x49, 0x46,  // '2'

66    0x21, 0x41, 0x45, 0x4B, 0x31,  // '3'

67    0x18, 0x14, 0x12, 0x7F, 0x10,  // '4'

68    0x27, 0x45, 0x45, 0x45, 0x39,  // '5'

69    0x3C, 0x4A, 0x49, 0x49, 0x30,  // '6'

70    0x01, 0x71, 0x09, 0x05, 0x03,  // '7'

71    0x36, 0x49, 0x49, 0x49, 0x36,  // '8'

72    0x06, 0x49, 0x49, 0x29, 0x1E,  // '9'

73    0x7E, 0x09, 0x09, 0x09, 0x7E,  // 'A'

74    0x7F, 0x49, 0x49, 0x49, 0x36,  // 'B'

75    0x3E, 0x41, 0x41, 0x41, 0x22,  // 'C'

76    0x7F, 0x41, 0x41, 0x41, 0x3E,  // 'D'

77    0x7F, 0x49, 0x49, 0x49, 0x41,  // 'E'

78    0x7F, 0x09, 0x09, 0x09, 0x01,  // 'F'

79    0x3E, 0x41, 0x49, 0x49, 0x7A,  // 'G'

80    0x7F, 0x08, 0x08, 0x08, 0x7F,  // 'H'

81    0x00, 0x41, 0x7F, 0x41, 0x00,  // 'I'

82    0x20, 0x40, 0x41, 0x3F, 0x01,  // 'J'

83    0x7F, 0x08, 0x14, 0x22, 0x41,  // 'K'

84    0x7F, 0x40, 0x40, 0x40, 0x40,  // 'L'

85    0x7F, 0x02, 0x0C, 0x02, 0x7F,  // 'M'

86    0x7F, 0x04, 0x08, 0x10, 0x7F,  // 'N'

87    0x3E, 0x41, 0x41, 0x41, 0x3E,  // 'O'

88    0x7F, 0x09, 0x09, 0x09, 0x06,  // 'P'

89    0x3E, 0x41, 0x51, 0x21, 0x5E,  // 'Q'

90    0x7F, 0x09, 0x19, 0x29, 0x46,  // 'R'

91    0x46, 0x49, 0x49, 0x49, 0x31,  // 'S'

92    0x01, 0x01, 0x7F, 0x01, 0x01,  // 'T'

93    0x3F, 0x40, 0x40, 0x40, 0x3F,  // 'U'

94    0x1F, 0x20, 0x40, 0x20, 0x1F,  // 'V'

95    0x7F, 0x20, 0x18, 0x20, 0x7F,  // 'W'

96    0x63, 0x14, 0x08, 0x14, 0x63,  // 'X'

97    0x03, 0x04, 0x78, 0x04, 0x03,  // 'Y'

98    0x61, 0x51, 0x49, 0x45, 0x43,  // 'Z'

99  };

## 4.10 art.h – Pixel art

Every picture of the game as binary numbers: one 16-bit word per row, the 1s are the lit pixels. The three ships, the enemies, the rocks, the pickups, the bosses, the warning sign and the letters of the logo.

Figure 26. What art.h draws or uses.

### Source code

1  /*

2   * art.h -- pixel art, stored in flash. One 16-bit word per row, the leftmost pixel is

3   * the top bit, so the 1s in each binary number draw the picture.

4   */

5  #pragma once

6  #include <avr/pgmspace.h>

7

8  // ================================================================= the player's ships (3 x 3)

9  const uint16_t SHIP_BOLT[3] PROGMEM = {        // BOLT: balanced

10    0b0100000000000000,                          //  .#.

11    0b1110000000000000,                          //  ###

12    0b1110000000000000,                          //  ###

13  };

14  const uint16_t SHIP_DART[3] PROGMEM = {        // DART: fast

15    0b0100000000000000,                          //  .#.

16    0b1110000000000000,                          //  ###

17    0b1010000000000000,                          //  #.#

18  };

19  const uint16_t SHIP_TWIN[3] PROGMEM = {        // TWIN: two guns

20    0b1010000000000000,                          //  #.#

21    0b1110000000000000,                          //  ###

22    0b1110000000000000,                          //  ###

23  };

24  const uint16_t SHIP_LEFT[3] PROGMEM = {        // BOLT flying left (logo)

25    0b0110000000000000,                          //  .##

26    0b1110000000000000,                          //  ###

27    0b0110000000000000,                          //  .##

28  };

29  const uint16_t* const SHIP_UP = SHIP_BOLT;

30

31  // ================================================================= enemies and rocks

32  const uint16_t ENEMY_DRONE[2] PROGMEM = {      // drone: slow, shoots straight down

33    0b1010000000000000,                          //  #.#

34    0b0100000000000000,                          //  .#.

35  };

36  const uint16_t ENEMY_WEAVER[2] PROGMEM = {     // weaver: zig-zags

37    0b1110000000000000,                          //  ###

38    0b1010000000000000,                          //  #.#

39  };

40  const uint16_t ENEMY_MINER[2] PROGMEM = {      // mine-layer: crosses the top, drops mines

41    0b1110000000000000,                          //  ###

42    0b0100000000000000,                          //  .#.

43  };

44  const uint16_t ENEMY_CRUISER[3] PROGMEM = {    // cruiser: tough, aims at you

45    0b0111000000000000,                          //  .###.

46    0b1101100000000000,                          //  ##.##

47    0b1010100000000000,                          //  #.#.#

48  };

49  // Rocks are hollow and lumpy (a crater in the middle), so they don't look like any ship.

50  const uint16_t ROCK_SMALL[3] PROGMEM = {

51    0b1100000000000000,                          //  ##.

52    0b1010000000000000,                          //  #.#

53    0b0110000000000000,                          //  .##

54  };

55  const uint16_t ROCK_BIG[3] PROGMEM = {         // breaks into two small rocks

56    0b0110000000000000,                          //  .##.

57    0b1101000000000000,                          //  ##.#

58    0b0110000000000000,                          //  .##.

59  };

60

61

62  // ================================================================= pickups (blinking)

63  const uint16_t PICKUP_AMMO[3] PROGMEM = {      // ammo crate: solid, unlike the hollow rocks

64    0b1110000000000000,                          //  ###

65    0b1110000000000000,                          //  ###

66    0b1110000000000000,                          //  ###

67  };

68  const uint16_t PICKUP_LIFE[3] PROGMEM = {      // extra life

69    0b0100000000000000,                          //  .#.

70    0b1110000000000000,                          //  ###

71    0b0100000000000000,                          //  .#.

72  };

73

74  // ================================================================= bosses

75  const uint16_t BOSS_WARDEN[4] PROGMEM = {      // phase 1: wide and slow, spread shots

76    0b0011111100000000,                          //  ..######..

77    0b0111111110000000,                          //  .########.

78    0b1101111011000000,                          //  ##.####.##

79    0b1001001001000000,                          //  #..#..#..#

80  };

81  const uint16_t BOSS_HUNTER[3] PROGMEM = {      // phase 2: small and fast, dashes and charges

82    0b1000100000000000,                          //  #...#

83    0b1111100000000000,                          //  #####

84    0b0101000000000000,                          //  .#.#.

85  };

86  const uint16_t BOSS_KING[5] PROGMEM = {        // phase 3: the Void King, first form

87    0b0011111111000000,                          //  ..########..

88    0b0111111111100000,                          //  .##########.

89    0b1110011001110000,                          //  ###..##..###

90    0b1111111111110000,                          //  ############

91    0b0101010101010000,                          //  .#.#.#.#.#.#

92  };

93  const uint16_t BOSS_KING_ANGRY[5] PROGMEM = {  // second form: the eye opens

94    0b1011111111010000,                          //  #.########.#

95    0b0111100111100000,                          //  .####..####.

96    0b1110100101110000,                          //  ###.#..#.###

97    0b1111111111110000,                          //  ############

98    0b1010100101010000,                          //  #.#.#..#.#.#

99  };

100

101  // ================================================================= signs

102  const uint16_t WARNING_SIGN[10] PROGMEM = {    // 11 x 10, shown before each boss

103    0b0000010000000000,                          //  .....#.....

104    0b0000101000000000,                          //  ....#.#....

105    0b0000101000000000,                          //  ....#.#....

106    0b0001000100000000,                          //  ...#...#...

107    0b0001010100000000,                          //  ...#.#.#...

108    0b0010010010000000,                          //  ..#..#..#..

109    0b0010010010000000,                          //  ..#..#..#..

110    0b0100000001000000,                          //  .#.......#.

111    0b0100010001000000,                          //  .#...#...#.

112    0b1111111111100000,                          //  ###########

113  };

114

115  // ================================================================= logo: "VOID", bold 10 x 5 letters

116  const uint8_t LOGO_W = 10, LOGO_H = 5, LOGO_X = 3;

117  const uint8_t LOGO_Y[4] = {0, 6, 12, 18};      // V in the red band ... D in the green band

118  const uint16_t LOGO_LETTERS[4][LOGO_H] PROGMEM = {

119    {                                            // V

120      0b1100000011000000,                        //  ##......##

121      0b0110000110000000,                        //  .##....##.

122      0b0011001100000000,                        //  ..##..##..

123      0b0001111000000000,                        //  ...####...

124      0b0000110000000000,                        //  ....##....

125    },

126    {                                            // O

127      0b0111111110000000,                        //  .########.

128      0b1100000011000000,                        //  ##......##

129      0b1100000011000000,                        //  ##......##

130      0b1100000011000000,                        //  ##......##

131      0b0111111110000000,                        //  .########.

132    },

133    {                                            // I

134      0b0011111100000000,                        //  ..######..

135      0b0000110000000000,                        //  ....##....

136      0b0000110000000000,                        //  ....##....

137      0b0000110000000000,                        //  ....##....

138      0b0011111100000000,                        //  ..######..

139    },

140    {                                            // D

141      0b1111111000000000,                        //  #######...

142      0b1100001100000000,                        //  ##....##..

143      0b1100000110000000,                        //  ##.....##.

144      0b1100001100000000,                        //  ##....##..

145      0b1111111000000000,                        //  #######...

146    },

147  };

## 4.11 logo.h – Start-up logo and welcome melody

The 5.6-second logo animation and its melody. The picture is driven by the song's own clock, so every movement lands on a note: warp, the ship, the VOID letters, "RUNNER" on a ribbon, the flash and the glowing sign.

Figure 27. What logo.h draws or uses.

| Function | What it does |
| --- | --- |
| drawLogoLetter(uint8_t i) | Draws letter i of the VOID sign (V, O, I, D from top to bottom). |
| logoBegin() | Starts the logo: scatters the warp stars and starts the welcome melody. |
| starSpeed(uint32_t t) | Star speed in 1/16 pixel per picture: speeds up, brakes, then drifts. |
| drawStars(uint32_t t) | Moves and draws the warp stars; the faster they fly, the longer their streaks. |
| drawShipUp(int16_t x, int16_t y, uint16_t frameNo) | Draws the ship flying up, with a flickering engine flame below it. |
| logoEffects(uint32_t t) | Sound effects that follow the picture: whoosh, glide and letter crackle. |
| logoUpdate(uint16_t frameNo) | Draws one picture of the logo. Returns true when the logo is over. |

### Source code

1  /*

2   * logo.h -- the start-up logo, played in sync with the welcome melody.

3   *

4   * The picture is driven by the song's own clock (songTime()), so every movement lands

5   * on its note. Timeline (ms):

6   *     0 - 1200  warp: star streaks rush past and speed up      rising "whoosh" + low drone

7   *  1200 - 2000  the ship rises from the bottom and brakes       3 falling braking notes

8   *  2000 - 2400  the ship shoots up and out of the screen        quick rising glide

9   *  2400 - 3600  V, O, I, D light up one by one, flickering on;  one note per letter

10   *               enemy drones fly past in the side margins       + crackle while it flickers

11   *  3600 - 4800  "RUNNER" crosses on a dark ribbon, towed by     melody, harmony and bass

12   *               the ship

13   *  4800 - 5600  the whole screen flashes, then the sign glows   final A-minor chord

14   * The status bars fill up during the logo like a start-up progress bar.

15   */

16  #pragma once

17  #include "display.h"

18  #include "sound.h"

19  #include "art.h"

20

21  // ---- welcome melody: one track per buzzer, 1 tick = 50 ms (MIDI note numbers)

22  const Note BOOT_MELODY[] PROGMEM = {

23    {0, 24},                                          // warp (whoosh is an effect)

24    {0, 16},                                          // ship arrives

25    {0, 8},                                           // ship leaves (glide is an effect)

26    {69, 6}, {72, 6}, {76, 6}, {81, 6},               // V O I D: A4 C5 E5 A5

27    {76, 3}, {74, 3}, {72, 3}, {74, 3}, {76, 3}, {76, 3}, {76, 6},   // RUNNER tune

28    {76, 8}, {0, 8},                                  // final chord, top note E5

29  };

30  const Note BOOT_HARMONY[] PROGMEM = {

31    {0, 24},

32    {64, 4}, {60, 4}, {57, 6}, {0, 2},                // braking: E4 C4 A3

33    {0, 32},

34    {72, 6}, {71, 6}, {69, 6}, {71, 6},               // C5 B4 A4 B4

35    {72, 8}, {0, 8},                                  // final chord, middle note C5

36  };

37  const Note BOOT_BASS[] PROGMEM = {

38    {45, 24},                                         // A2 drone under the warp

39    {0, 24},

40    {0, 24},                                          // (crackle effects)

41    {45, 3}, {45, 3}, {52, 3}, {45, 3}, {45, 3}, {45, 3}, {43, 3}, {45, 3},  // bass line

42    {57, 8}, {0, 8},                                  // final chord, low note A3

43  };

44  const Track BOOT_SONG[BUZZER_COUNT] = {

45    {BOOT_MELODY, sizeof(BOOT_MELODY) / sizeof(Note)},

46    {BOOT_HARMONY, sizeof(BOOT_HARMONY) / sizeof(Note)},

47    {BOOT_BASS, sizeof(BOOT_BASS) / sizeof(Note)},

48  };

49  const uint8_t BOOT_TICK_MS = 50;

50  const uint16_t LOGO_END_MS = 5600;

51  const uint16_t LETTER_START[4] = {2400, 2700, 3000, 3300};

52  const uint8_t LETTER_FLICKER_MS = 120;

53

54  // ---- warp stars: y is stored in 1/16 pixel so they can move smoothly

55  const uint8_t STAR_COUNT = 12;

56  int8_t starX[STAR_COUNT];

57  int16_t starY16[STAR_COUNT];

58

59  // Draws letter i of the VOID sign (V, O, I, D from top to bottom).

60  void drawLogoLetter(uint8_t i) {

61    drawBitmap(LOGO_X, LOGO_Y[i], LOGO_LETTERS[i], LOGO_W, LOGO_H, true);

62  }

63

64  // Starts the logo: scatters the warp stars and starts the welcome melody.

65  void logoBegin() {

66    for (uint8_t i = 0; i < STAR_COUNT; i++) {

67      starX[i] = random(SCREEN_W);

68      starY16[i] = random(SCREEN_H * 16);

69    }

70    soundPlaySong(BOOT_SONG, BOOT_TICK_MS, false);

71  }

72

73  // Star speed in 1/16 pixel per picture: speeds up, brakes, then drifts.

74  uint8_t starSpeed(uint32_t t) {

75    if (t < 1200) return 4 + 28 * t / 1200;

76    if (t < 2000) return 32 - 29 * (t - 1200) / 800;

77    return 3;

78  }

79

80  // Moves and draws the warp stars; the faster they fly, the longer their streaks.

81  void drawStars(uint32_t t) {

82    uint8_t speed = starSpeed(t);

83    uint8_t len = 1 + speed / 10;                    // faster = longer streak

84    for (uint8_t i = 0; i < STAR_COUNT; i++) {

85      starY16[i] += speed;

86      if (starY16[i] / 16 - len > SCREEN_H) {        // fell off the bottom: start again on top

87        starY16[i] = -16 * (int16_t)random(4);

88        starX[i] = random(SCREEN_W);

89      }

90      for (uint8_t k = 0; k < len; k++) setPixel(starX[i], starY16[i] / 16 - k, true);

91    }

92  }

93

94  // Draws the ship flying up, with a flickering engine flame below it.

95  void drawShipUp(int16_t x, int16_t y, uint16_t frameNo) {

96    drawBitmap(x, y, SHIP_UP, 3, 3, true);

97    if (frameNo & 1) setPixel(x + 1, y + 3, true);   // flickering engine flame

98  }

99

100  // Sound effects that follow the picture: whoosh, glide and letter crackle.

101  void logoEffects(uint32_t t) {

102    if (t < 1200) soundEffect(0, 150 + 1350 * t / 1200);

103    else if (t >= 2000 && t < 2300) soundEffect(0, 400 + 1200 * (t - 2000) / 300);

104    else soundRelease(0);

105    bool crackle = false;

106    for (uint8_t i = 0; i < 4; i++)

107      if (t >= LETTER_START[i] && t < LETTER_START[i] + LETTER_FLICKER_MS) crackle = true;

108    if (crackle) soundEffect(2, random(1500, 3500));

109    else soundRelease(2);

110  }

111

112  // Draws one picture of the logo. Returns true when the logo is over.

113  bool logoUpdate(uint16_t frameNo) {

114    uint32_t t = songTime();

115    if (t >= LOGO_END_MS) {

116      soundRelease(0);

117      soundRelease(2);

118      return true;

119    }

120    logoEffects(t);

121    clearScreen();

122    if (t < 2400) drawStars(t);

123    if (t >= 1200 && t < 2000) {                     // ship rises and brakes

124      uint32_t p = (t - 1200) * 100 / 800, rest = 100 - p;

125      drawShipUp(6, 19 + 5 * rest * rest / 10000, frameNo);

126    } else if (t >= 2000 && t < 2400) {              // ship shoots up and out

127      uint32_t q = (t - 2000) * 100 / 400;

128      int16_t y = 19 - (int16_t)(23 * q * q / 10000);

129      drawShipUp(6, y, frameNo);

130      setPixel(7, y + 4, true);                      // longer flame

131    }

132    if (t >= LETTER_START[0] && t < 4800) {

133      for (uint8_t i = 0; i < 4; i++) {              // letters light up, flickering at first

134        if (t < LETTER_START[i]) continue;

135        uint32_t age = t - LETTER_START[i];

136        if (age >= LETTER_FLICKER_MS || (age / 30) % 2 == 0) drawLogoLetter(i);

137      }

138      if (t < 3600) {                                // drones fly past in the side margins

139        drawBitmap(0, -2 + (int16_t)((t - 2400) * 26 / 1200), ENEMY_DRONE, 3, 2, true);

140        if (t >= 2800) drawBitmap(13, -2 + (int16_t)((t - 2800) * 26 / 1200), ENEMY_DRONE, 3, 2, true);

141      }

142    }

143    if (t >= 3600 && t < 4800) {                     // "RUNNER" on a dark ribbon, towed by the ship

144      fillRect(0, 8, SCREEN_W, 9, false);

145      for (uint8_t x = 0; x < SCREEN_W; x++)

146        if ((x + t / 60) % 3) {                      // ribbon edges move like a conveyor

147          setPixel(x, 8, true);

148          setPixel(x, 16, true);

149        }

150      int16_t x = 16 - (int16_t)((t - 3600) * 51 / 1200);

151      drawText(x, 9, PSTR("RUNNER"), true);

152      drawBitmap(x - 4, 11, SHIP_LEFT, 3, 3, true);

153    }

154    if (t >= 4800) {                                 // final hit: flash, then the sign glows

155      if (t < 4900) fillRect(0, 0, SCREEN_W, SCREEN_H, true);

156      else for (uint8_t i = 0; i < 4; i++) drawLogoLetter(i);

157    }

158    uint8_t level = t >= 4800 ? 16 : t * 16 / 4800;  // status bars: start-up progress

159    for (uint8_t b = 0; b < STATUS_ROWS; b++) setStatusBar(b, level);

160    return false;

161  }

## 4.12 title.h – Title screen

The VOID sign stays lit, ships keep flying past it, and every 7 seconds a banner shows the pages "PUSH", a big R, "TO", "PLAY". After 20 s without a key the demo starts.

Figure 28. What title.h draws or uses.

| Function | What it does |
| --- | --- |
| titleDraw(uint32_t t, uint16_t frameNo) | Draws one picture of the title screen at time t (ms since it opened). |

### Source code

1  /*

2   * title.h -- title screen: the VOID sign stays lit, ships keep flying past it, and every

3   * 7 seconds a banner shows the pages "PUSH", a big R, "TO", "PLAY".

4   */

5  #pragma once

6  #include "display.h"

7  #include "art.h"

8  #include "logo.h"

9

10  // Draws one picture of the title screen at time t (ms since it opened).

11  void titleDraw(uint32_t t, uint16_t frameNo) {

12    clearScreen();

13    for (uint8_t i = 0; i < 4; i++) drawLogoLetter(i);

14    drawBitmap(0, -2 + (int16_t)(t % 2400 * 26 / 2400), ENEMY_DRONE, 3, 2, true);   // drone down the left

15    int16_t shipY = 24 - (int16_t)(t % 2000 * 28 / 2000);                          // ship up the right

16    drawBitmap(13, shipY, SHIP_UP, 3, 3, true);

17    if (frameNo & 1) setPixel(14, shipY + 3, true);

18    uint32_t b = t % 7000;

19    if (b < 4 * PAGE_MS) {                           // the banner: PUSH, R, TO, PLAY

20      fillRect(0, 8, SCREEN_W, 9, false);

21      for (uint8_t x = 0; x < SCREEN_W; x++)

22        if ((x + t / 60) % 3) {

23          setPixel(x, 8, true);

24          setPixel(x, 16, true);

25        }

26      uint8_t page = b / PAGE_MS;

27      if (page == 1) drawChar(5, 9, 'R', true);     // the big R, 5 x 7

28      else drawTextCentered(10, page == 0 ? PSTR("PUSH") : page == 2 ? PSTR("TO") : PSTR("PLAY"), false);

29    }

30    for (uint8_t bar = 0; bar < STATUS_ROWS; bar++) {      // status bars: a slow chase

31      uint8_t head = (t / 120 + bar * 5) % 16;

32      setStatusBits(bar, (1u << head) | (1u << ((head + 15) % 16)));

33    }

34  }

## 4.13 menu.h – Main menu

The menu PLAY, SHIP, OPTS, BEST: W/S move the underline, R opens the item, Q goes back to the title.

Figure 29. What menu.h draws or uses.

| Function | What it does |
| --- | --- |
| menuItem(uint8_t i) | Text of menu item i (the list of texts is stored in flash). |
| menuUpdate() | Handles the keys; returns the item to open, MENU_BACK or MENU_NONE. |
| menuDraw() | Draws the four menu items, underlines the chosen one and switches the status bars off. |

### Source code

1  /*

2   * menu.h -- main menu: PLAY, SHIP, OPTS, BEST (W/S to choose, R to open, Q back).

3   * The chosen item is underlined.

4   */

5  #pragma once

6  #include "display.h"

7  #include "input.h"

8  #include "sound.h"

9

10  const char MENU_PLAY[] PROGMEM = "PLAY";

11  const char MENU_SHIP[] PROGMEM = "SHIP";

12  const char MENU_OPTS[] PROGMEM = "OPTS";

13  const char MENU_BEST[] PROGMEM = "BEST";

14  const char* const MENU_ITEMS[] PROGMEM = {MENU_PLAY, MENU_SHIP, MENU_OPTS, MENU_BEST};

15  enum MenuItem : int8_t { MENU_ITEM_PLAY, MENU_ITEM_SHIP, MENU_ITEM_OPTS, MENU_ITEM_BEST };

16  const uint8_t MENU_COUNT = 4;

17  uint8_t menuIndex = 0;

18

19  // Text of menu item i (the list of texts is stored in flash).

20  const char* menuItem(uint8_t i) {

21    return (const char*)pgm_read_ptr(&MENU_ITEMS[i]);

22  }

23

24  const int8_t MENU_NONE = -1, MENU_BACK = -2;

25

26  // Handles the keys; returns the item to open, MENU_BACK or MENU_NONE.

27  int8_t menuUpdate() {

28    if (keyPressed(KEY_UP)) {

29      menuIndex = (menuIndex + MENU_COUNT - 1) % MENU_COUNT;

30      soundBeep(1400, 25);

31    }

32    if (keyPressed(KEY_DOWN)) {

33      menuIndex = (menuIndex + 1) % MENU_COUNT;

34      soundBeep(1400, 25);

35    }

36    if (keyPressed(KEY_FIRE)) {

37      soundBeep(2000, 60);

38      return menuIndex;

39    }

40    if (keyPressed(KEY_BACK)) return MENU_BACK;

41    return MENU_NONE;

42  }

43

44  // Draws the four menu items, underlines the chosen one and switches the status bars off.

45  void menuDraw() {

46    clearScreen();

47    for (uint8_t i = 0; i < MENU_COUNT; i++) {

48      uint8_t y = i * 6;

49      if (i == menuIndex) fillRect(0, y + 5, 15, 1, true); // underline in the gap below

50      drawText(0, y, menuItem(i), false);

51    }

52    for (uint8_t bar = 0; bar < STATUS_ROWS; bar++) setStatusBits(bar, 0);

53  }

## 4.14 world.h – Everything on the game screen

The data of the game world in fixed-size pools: the player, bullets and mines, enemies and rocks, pickups, explosion particles and the boss, with the tables of the three ships and the enemy types and small helpers to create them. There is no malloc, so nothing can run out during a game.

Figure 30. What world.h draws or uses.

| Function | What it does |
| --- | --- |
| playerShip() | The table entry (speed, fire rate, ammo ...) of the ship in use. |
| shipPixel(int8_t x, int8_t y) | True if the player's ship has a lit pixel at screen position (x, y): exact hits. |
| addScore(uint16_t points) | Adds points to the score, which stops at 9999 (four digits fit on the screen). |
| playerShotCount() | Counts the player's bullets in the air (at most MAX_PLAYER_SHOTS). |
| newShot(int8_t x, int8_t y, uint8_t kind, uint16_t stepMs) | Puts a new bullet or mine in a free slot of the shot pool and returns it, or NULL when all slots are in use. |
| enemyShot(int8_t x, int8_t y, int8_t dx, uint8_t sideEvery, uint8_t stepMs) | Fires an enemy bullet: straight down, or with dx = -1 / 1 one step sideways every sideEvery rows (spread shots). |
| aimedShot(int8_t x, int8_t y, uint8_t stepMs) | A shot that heads for the player: it steps sideways once every few rows. |
| infoOf(const Enemy& e) | The table entry (sprite, size, health, points, speed) of an enemy's type. |
| fireDelay(uint16_t ms) | Scales an enemy fire interval by the difficulty (EASY slower, HARD faster). |
| spawnEnemy(uint8_t type, int8_t x, int8_t y) | Puts a new enemy or rock of a type at (x, y) in a free slot and returns it, or NULL when all slots are in use. |
| enemiesAlive() | Counts the enemies and rocks on the screen. |
| spawnPickup(uint8_t type, int8_t x, int8_t y) | Drops a pickup (ammo crate or extra life) at (x, y) if a slot is free. |
| pickupAlive(uint8_t type) | True if a pickup of this type is on the screen. |
| explode(int8_t cx, int8_t cy, uint8_t count, uint8_t speed) | Starts an explosion: up to `count` pixels fly out from (cx, cy) at random speeds. |
| bossSprite() | The picture of the current boss (the VOID KING has one per form). |
| shake(uint8_t frames) | Shakes the screen for some pictures (a longer shake already running is not cut short). |
| worldClear() | Empties the game screen: no shots, enemies, pickups, explosion pixels or boss. |

### Source code

1  /*

2   * world.h -- everything that lives on the game screen: the player, bullets, enemies and

3   * rocks, pickups, explosion particles and the boss.

4   *

5   * Only fixed-size arrays (no malloc): the Mega has 8 KB of RAM and nothing may run out in

6   * the middle of a game. Positions are in LED pixels, (0,0) = top-left of the game screen.

7   * All timing uses gameMs, a clock that advances one frame at a time and stops on pause.

8   */

9  #pragma once

10  #include "config.h"

11  #include "display.h"

12  #include "sound.h"

13  #include "tunes.h"

14  #include "art.h"

15  #include "storage.h"

16

17  uint32_t gameMs = 0;

18  const int8_t PLAYER_MIN_Y = 9, PLAYER_MAX_Y = SCREEN_H - 3;

19

20  // ================================================================= the three ships

21  struct ShipInfo {

22    const char* name;                      // in flash

23    const uint16_t* sprite;

24    uint8_t moveMs;                        // ms per pixel while gliding

25    uint16_t fireMs;                       // ms between shots while R is held

26    uint8_t ammoMax;

27    bool twin;                             // two guns: two bullets, two ammo per shot

28    uint8_t speedLevel, ammoLevel, fireLevel;   // 1..3, shown in the hangar

29  };

30  const char NAME_BOLT[] PROGMEM = "BOLT";

31  const char NAME_DART[] PROGMEM = "DART";

32  const char NAME_TWIN[] PROGMEM = "TWIN";

33  const ShipInfo SHIPS[3] = {

34    {NAME_BOLT, SHIP_BOLT, 70, 200, 60, false, 2, 2, 2},

35    {NAME_DART, SHIP_DART, 45, 220, 40, false, 3, 1, 2},

36    {NAME_TWIN, SHIP_TWIN, 95, 260, 80, true, 1, 3, 3},

37  };

38

39  // ================================================================= player

40  struct Player {

41    int8_t x, y;                           // top-left of the 3 x 3 ship

42    uint8_t ship, lives, ammo;

43    uint16_t score;

44    bool firing;                           // R was pressed and no move key cancelled it

45    uint8_t burst;                         // shots since R was pressed

46    char moveKey;                          // move key that is held (0 = none)

47    bool gliding;                          // the key was held long enough to glide

48    uint32_t nextShot, nextMove, shieldUntil;

49    bool lowAmmoWarned;

50  };

51  Player player;

52  bool testShield = false;                 // test hook: the player can't be hit

53  bool autopilot = false;                  // test hook: the game plays itself

54

55  // The table entry (speed, fire rate, ammo ...) of the ship in use.

56  const ShipInfo& playerShip() {

57    return SHIPS[player.ship];

58  }

59

60  // True if the player's ship has a lit pixel at screen position (x, y): exact hits.

61  bool shipPixel(int8_t x, int8_t y) {

62    int8_t c = x - player.x, r = y - player.y;

63    if (c < 0 || c > 2 || r < 0 || r > 2) return false;

64    return pgm_read_word(playerShip().sprite + r) & (0x8000u >> c);

65  }

66

67  // Adds points to the score, which stops at 9999 (four digits fit on the screen).

68  void addScore(uint16_t points) {

69    player.score = (player.score + points > 9999) ? 9999 : player.score + points;

70  }

71

72  // ================================================================= shots (player and enemy)

73  enum ShotKind : uint8_t { SHOT_PLAYER, SHOT_ENEMY, SHOT_MINE };

74  struct Shot {

75    int8_t x, y, dx;                       // dx: sideways step (-1, 0, 1)

76    uint8_t sideEvery, sideCount;          // sideways step every n down steps

77    uint16_t stepMs;                       // ms per pixel

78    uint8_t kind;

79    uint32_t next;

80    bool alive;

81  };

82  const uint8_t MAX_SHOTS = 28, MAX_PLAYER_SHOTS = 6;

83  Shot shots[MAX_SHOTS];

84

85  // Counts the player's bullets in the air (at most MAX_PLAYER_SHOTS).

86  uint8_t playerShotCount() {

87    uint8_t n = 0;

88    for (uint8_t i = 0; i < MAX_SHOTS; i++)

89      if (shots[i].alive && shots[i].kind == SHOT_PLAYER) n++;

90    return n;

91  }

92

93  // Puts a new bullet or mine in a free slot of the shot pool and returns it, or NULL when

94  // all slots are in use.

95  Shot* newShot(int8_t x, int8_t y, uint8_t kind, uint16_t stepMs) {

96    for (uint8_t i = 0; i < MAX_SHOTS; i++) {

97      Shot& s = shots[i];

98      if (s.alive) continue;

99      s = {x, y, 0, 1, 0, stepMs, kind, gameMs + stepMs, true};

100      return &s;

101    }

102    return NULL;                           // pool full: the shot is simply not fired

103  }

104

105  // Fires an enemy bullet: straight down, or with dx = -1 / 1 one step sideways every

106  // sideEvery rows (spread shots).

107  void enemyShot(int8_t x, int8_t y, int8_t dx, uint8_t sideEvery, uint8_t stepMs) {

108    Shot* s = newShot(x, y, SHOT_ENEMY, stepMs);

109    if (s) {

110      s->dx = dx;

111      s->sideEvery = sideEvery;

112    }

113  }

114

115  // A shot that heads for the player: it steps sideways once every few rows.

116  void aimedShot(int8_t x, int8_t y, uint8_t stepMs) {

117    int8_t cols = (player.x + 1) - x, rows = player.y - y;

118    if (rows < 1) rows = 1;

119    int8_t dx = (cols > 0) - (cols < 0);

120    uint8_t every = cols ? constrain(rows / abs(cols), 1, 6) : 1;

121    enemyShot(x, y, dx, every, stepMs);

122  }

123

124  // ================================================================= enemies and rocks

125  enum EnemyType : uint8_t { EN_DRONE, EN_WEAVER, EN_MINER, EN_CRUISER, EN_ROCK_S, EN_ROCK_B };

126  struct EnemyInfo {

127    const uint16_t* sprite;

128    uint8_t w, h, hp;

129    uint8_t score;

130    uint16_t stepMs;                       // ms per pixel of its main movement

131  };

132  const EnemyInfo ENEMY_INFO[6] = {

133    {ENEMY_DRONE, 3, 2, 1, 10, 450},

134    {ENEMY_WEAVER, 3, 2, 1, 15, 600},

135    {ENEMY_MINER, 3, 2, 2, 20, 280},

136    {ENEMY_CRUISER, 5, 3, 5, 40, 500},

137    {ROCK_SMALL, 3, 3, 1, 5, 360},

138    {ROCK_BIG, 4, 3, 2, 10, 520},

139  };

140  struct Enemy {

141    int8_t x, y, dir;

142    uint8_t type, hp, flash, steps;

143    uint32_t nextMove, nextSide, nextShot;

144    bool alive;

145  };

146  const uint8_t MAX_ENEMIES = 16;

147  Enemy enemies[MAX_ENEMIES];

148

149  // The table entry (sprite, size, health, points, speed) of an enemy's type.

150  const EnemyInfo& infoOf(const Enemy& e) {

151    return ENEMY_INFO[e.type];

152  }

153

154  // Scales an enemy fire interval by the difficulty (EASY slower, HARD faster).

155  uint16_t fireDelay(uint16_t ms) {

156    return (uint32_t)ms * DIFF_FIRE_SCALE[settings.difficulty] / 100;

157  }

158

159  // Puts a new enemy or rock of a type at (x, y) in a free slot and returns it, or NULL

160  // when all slots are in use.

161  Enemy* spawnEnemy(uint8_t type, int8_t x, int8_t y) {

162    for (uint8_t i = 0; i < MAX_ENEMIES; i++) {

163      Enemy& e = enemies[i];

164      if (e.alive) continue;

165      const EnemyInfo& info = ENEMY_INFO[type];

166      e = {x, y, 1, type, info.hp, 0, 0, gameMs + info.stepMs, gameMs + 300,

167           gameMs + fireDelay(1200 + random(1500)), true};

168      return &e;

169    }

170    return NULL;

171  }

172

173  // Counts the enemies and rocks on the screen.

174  uint8_t enemiesAlive() {

175    uint8_t n = 0;

176    for (uint8_t i = 0; i < MAX_ENEMIES; i++) n += enemies[i].alive;

177    return n;

178  }

179

180  // ================================================================= pickups

181  enum PickupType : uint8_t { PK_AMMO, PK_LIFE };

182  struct Pickup {

183    int8_t x, y;

184    uint8_t type;

185    uint32_t next;

186    bool alive;

187  };

188  const uint8_t MAX_PICKUPS = 4;

189  Pickup pickups[MAX_PICKUPS];

190

191  // Drops a pickup (ammo crate or extra life) at (x, y) if a slot is free.

192  void spawnPickup(uint8_t type, int8_t x, int8_t y) {

193    for (uint8_t i = 0; i < MAX_PICKUPS; i++)

194      if (!pickups[i].alive) {

195        pickups[i] = {(int8_t)constrain(x, 0, SCREEN_W - 3), y, type, gameMs + 350, true};

196        return;

197      }

198  }

199

200  // True if a pickup of this type is on the screen.

201  bool pickupAlive(uint8_t type) {

202    for (uint8_t i = 0; i < MAX_PICKUPS; i++)

203      if (pickups[i].alive && pickups[i].type == type) return true;

204    return false;

205  }

206

207  // ================================================================= explosion particles

208  // Only the boss and the player's ship explode into flying pixels; ordinary enemies just

209  // vanish, so the small screen does not fill up with debris that looks like bullets.

210  struct Particle {

211    int16_t x16, y16;                      // position in 1/16 pixel

212    int8_t vx, vy;                         // speed in 1/16 pixel per frame

213    uint8_t life;                          // frames left

214  };

215  const uint8_t MAX_PARTICLES = 40;

216  Particle particles[MAX_PARTICLES];

217

218  // Starts an explosion: up to `count` pixels fly out from (cx, cy) at random speeds.

219  void explode(int8_t cx, int8_t cy, uint8_t count, uint8_t speed) {

220    for (uint8_t i = 0; i < MAX_PARTICLES && count; i++) {

221      Particle& p = particles[i];

222      if (p.life) continue;

223      p = {(int16_t)(cx * 16 + 8), (int16_t)(cy * 16 + 8), (int8_t)random(-speed, speed + 1),

224           (int8_t)random(-speed, speed + 1), (uint8_t)random(8, 18)};

225      count--;

226    }

227  }

228

229  // ================================================================= boss

230  enum BossKind : uint8_t { BOSS_KIND_WARDEN, BOSS_KIND_HUNTER, BOSS_KIND_KING };

231  struct Boss {

232    bool alive;

233    uint8_t kind, w, h, flash, form, mode;

234    int8_t x, y, dir, targetX;

235    int16_t hp, maxHp;

236    uint32_t nextMove, nextShot, nextAction, modeUntil;

237    uint8_t burst;

238  };

239  Boss boss;

240

241  // The picture of the current boss (the VOID KING has one per form).

242  const uint16_t* bossSprite() {

243    if (boss.kind == BOSS_KIND_WARDEN) return BOSS_WARDEN;

244    if (boss.kind == BOSS_KIND_HUNTER) return BOSS_HUNTER;

245    return boss.form ? BOSS_KING_ANGRY : BOSS_KING;

246  }

247

248  // ================================================================= helpers

249  // True when two boxes overlap (the ship against enemies, rocks, the boss, pickups).

250  bool boxesTouch(int8_t ax, int8_t ay, uint8_t aw, uint8_t ah,

251                  int8_t bx, int8_t by, uint8_t bw, uint8_t bh) {

252    return ax < bx + bw && bx < ax + aw && ay < by + bh && by < ay + ah;

253  }

254

255  uint8_t shakeFrames = 0;

256  // Shakes the screen for some pictures (a longer shake already running is not cut short).

257  void shake(uint8_t frames) {

258    if (frames > shakeFrames) shakeFrames = frames;

259  }

260

261  // Empties the game screen: no shots, enemies, pickups, explosion pixels or boss.

262  void worldClear() {

263    for (uint8_t i = 0; i < MAX_SHOTS; i++) shots[i].alive = false;

264    for (uint8_t i = 0; i < MAX_ENEMIES; i++) enemies[i].alive = false;

265    for (uint8_t i = 0; i < MAX_PICKUPS; i++) pickups[i].alive = false;

266    for (uint8_t i = 0; i < MAX_PARTICLES; i++) particles[i].life = 0;

267    boss.alive = false;

268  }

## 4.15 player.h – The player's ship

The key rules for moving and firing (tap, hold, auto-fire, newest key wins), ammo and the low-ammo warning, getting hit (a life, explosion, shake, shield) and drawing the ship.

Figure 31. What player.h draws or uses.

| Function | What it does |
| --- | --- |
| playerStart(bool newGame) | Gets the ship ready: bottom centre, full ammo, not firing. newGame = also the ship from the hangar, the lives of the difficulty and score 0 (a new phase keeps those). |
| playerShielded() | True while the ship can't be hit (2 s after a hit, or the test hook !i). |
| movePlayer(char key) | Moves the ship one pixel in the direction of a move key, staying on the screen and in rows 9-23. |
| playerFire() | Fires one shot (TWIN: two bullets) if there is ammo and a free bullet slot. Plays the shot sound (a chirp when auto-fire starts) and warns once when ammo gets low. |
| playerKeys() | Keys: called once per picture (key presses are counted per picture). |
| playerStep() | Movement and fire: called once per game step (20 ms of game time). |
| playerHit(const __FlashStringHelper* cause) | The ship is hit (unless shielded): one life less, 2 s shield, explosion, shake and sound. cause says what hit it, for the Serial log, e.g. F("shot"). |
| drawPlayer(uint16_t frameNo) | Draws the ship and its engine flame; it blinks while shielded, and its nose blinks during auto-fire. |

### Source code

1  /*

2   * player.h -- the player's ship: moving, firing, ammo and lives.

3   *

4   * Key rules (agreed in the design):

5   *   move   tap = one pixel; hold = after a short delay the ship glides at its speed

6   *   fire   press R = one bullet; keep holding = auto-fire (a chirp, the nose blinks);

7   *          release R = stop

8   *   The newest key wins: a move key stops the fire (press R again to fire again), and

9   *   while R is held the ship does not glide.

10   * Every shot costs ammo (TWIN: two bullets, two ammo). At LOW_AMMO a warning beeps and

11   * the yellow bar blinks. A hit costs a life and gives two seconds of blinking shield.

12   */

13  #pragma once

14  #include "world.h"

15  #include "input.h"

16

17  const char MOVE_KEYS[4] = {KEY_UP, KEY_LEFT, KEY_DOWN, KEY_RIGHT};

18

19  // Gets the ship ready: bottom centre, full ammo, not firing. newGame = also the ship from

20  // the hangar, the lives of the difficulty and score 0 (a new phase keeps those).

21  void playerStart(bool newGame) {

22    if (newGame) {

23      player.ship = settings.ship;

24      player.lives = DIFF_LIVES[settings.difficulty];

25      player.score = 0;

26    }

27    player.x = (SCREEN_W - 3) / 2;

28    player.y = SCREEN_H - 4;

29    player.ammo = playerShip().ammoMax;    // every phase starts with a full magazine

30    player.firing = false;

31    player.moveKey = 0;

32    player.shieldUntil = 0;

33    player.lowAmmoWarned = false;

34  }

35

36  // True while the ship can't be hit (2 s after a hit, or the test hook !i).

37  bool playerShielded() {

38    return testShield || gameMs < player.shieldUntil;

39  }

40

41  // Moves the ship one pixel in the direction of a move key, staying on the screen and in

42  // rows 9-23.

43  void movePlayer(char key) {

44    int8_t dx = (key == KEY_RIGHT) - (key == KEY_LEFT);

45    int8_t dy = (key == KEY_DOWN) - (key == KEY_UP);

46    player.x = constrain(player.x + dx, 0, SCREEN_W - 3);

47    player.y = constrain(player.y + dy, PLAYER_MIN_Y, PLAYER_MAX_Y);

48  }

49

50  // Fires one shot (TWIN: two bullets) if there is ammo and a free bullet slot. Plays the

51  // shot sound (a chirp when auto-fire starts) and warns once when ammo gets low.

52  void playerFire() {

53    const ShipInfo& ship = playerShip();

54    uint8_t bullets = (ship.twin && player.ammo >= 2) ? 2 : 1;

55    if (player.ammo == 0) {

56      soundSfx(SFX(SFX_EMPTY), 2);

57      player.nextShot = gameMs + ship.fireMs;

58      return;

59    }

60    if (playerShotCount() + bullets > MAX_PLAYER_SHOTS) return;   // try again next step

61    if (bullets == 2) {

62      newShot(player.x, player.y - 1, SHOT_PLAYER, PLAYER_SHOT_MS);

63      newShot(player.x + 2, player.y - 1, SHOT_PLAYER, PLAYER_SHOT_MS);

64    } else {

65      newShot(player.x + 1, player.y - 1, SHOT_PLAYER, PLAYER_SHOT_MS);

66    }

67    player.ammo -= bullets;

68    player.nextShot = gameMs + ship.fireMs;

69    if (++player.burst == 2) soundSfx(SFX(SFX_CHIRP), 2);   // auto-fire has started

70    else soundSfx(SFX(SFX_SHOT), 1);

71    if (player.ammo <= LOW_AMMO && !player.lowAmmoWarned) {

72      player.lowAmmoWarned = true;

73      soundSfx(SFX(SFX_LOW_AMMO), 4);

74    }

75  }

76

77  // Keys: called once per picture (key presses are counted per picture).

78  void playerKeys() {

79    if (autopilot) return;

80    for (uint8_t i = 0; i < 4; i++)

81      if (keyPressed(MOVE_KEYS[i])) {      // newest move key takes over and stops the fire

82        player.moveKey = MOVE_KEYS[i];

83        player.gliding = false;

84        player.nextMove = gameMs;          // first pixel right away

85        player.firing = false;

86      }

87    if (keyPressed(KEY_FIRE)) {

88      player.firing = true;

89      player.burst = 0;

90      player.nextShot = gameMs;            // first bullet right away

91    }

92    if (player.moveKey && !keyHeld(player.moveKey)) {   // let go: another held move key?

93      player.moveKey = 0;

94      for (uint8_t i = 0; i < 4; i++)

95        if (keyHeld(MOVE_KEYS[i])) {

96          player.moveKey = MOVE_KEYS[i];

97          player.gliding = true;

98          player.nextMove = gameMs + playerShip().moveMs;

99        }

100    }

101  }

102

103  // Movement and fire: called once per game step (20 ms of game time).

104  void playerStep() {

105    if (player.lives == 0) return;

106    if (player.firing) {

107      if (!autopilot && !keyHeld(KEY_FIRE)) player.firing = false;   // R released

108      else if (gameMs >= player.nextShot) playerFire();

109    }

110    if (!player.firing && player.moveKey && gameMs >= player.nextMove) {

111      movePlayer(player.moveKey);

112      player.nextMove = gameMs + (player.gliding ? playerShip().moveMs : MOVE_REPEAT_DELAY);

113      player.gliding = true;

114    }

115  }

116

117  // The ship is hit (unless shielded): one life less, 2 s shield, explosion, shake and sound.

118  // cause says what hit it, for the Serial log, e.g. F("shot").

119  void playerHit(const __FlashStringHelper* cause) {

120    if (playerShielded() || player.lives == 0) return;

121    player.lives--;

122    player.shieldUntil = gameMs + HIT_SHIELD_MS;

123    player.firing = false;

124    explode(player.x + 1, player.y + 1, 12, 14);

125    shake(12);

126    soundSfx(SFX(SFX_PLAYER_HIT), 6);

127    Serial.print(F("[VR] hit by "));

128    Serial.print(cause);

129    Serial.print(F(", lives "));

130    Serial.println(player.lives);

131  }

132

133  // Draws the ship and its engine flame; it blinks while shielded, and its nose blinks

134  // during auto-fire.

135  void drawPlayer(uint16_t frameNo) {

136    if (player.lives == 0) return;

137    if (gameMs < player.shieldUntil && (frameNo & 8)) return;    // blinks while shielded

138    const uint16_t* sprite = playerShip().sprite;

139    drawBitmap(player.x, player.y, sprite, 3, 3, true);

140    if (player.firing && player.burst >= 2 && (frameNo & 8))      // auto-fire: the nose blinks

141      drawBitmap(player.x, player.y, sprite, 3, 1, false);

142    if (frameNo & 1) setPixel(player.x + 1, player.y + 3, true);  // engine flame

143  }

## 4.16 enemies.h – Enemies and rocks

How each enemy type moves and shoots, and the rule that keeps them apart: a move may not bring an enemy closer than one empty pixel to another one, so it steps aside, turns or waits. New enemies take the nearest free column.

Figure 32. What enemies.h draws or uses.

| Function | What it does |
| --- | --- |
| touchesOther(const Enemy& e, int8_t x, int8_t y, uint8_t gap) | True if enemy e, placed at (x, y), would touch another enemy: with gap = 1 also when they are only side by side with no empty pixel between them. |
| canMoveTo(const Enemy& e, int8_t x, int8_t y) | True if enemy e may move to (x, y). On 16 x 24 LEDs two enemies on the same pixels merge into one unreadable blob, so a move may not bring it closer than one empty pixel to another enemy (a move is still allowed if they were already that close, so two enemies side by side can't block each other forever). |
| moveDown(Enemy& e) | Moves an enemy one pixel down. If something is in the way it steps aside instead (its own direction first), or waits for this step. |
| moveSide(Enemy& e) | Moves an enemy one pixel sideways in its direction; if something is in the way (or the screen edge: only miners fly in and out at the sides) it turns around instead. Returns true if it moved. |
| spawnFromTable(uint8_t type, int8_t x) | Puts an enemy on the screen just above the top (miners come in from the side). |
| updateEnemy(Enemy& e) | Moves one enemy or rock for one game step and lets it shoot; each type has its own pattern (see the list at the top). Anything that leaves at the bottom is removed. |
| updateEnemies() | Updates every enemy and rock that is alive (once per game step). |
| drawEnemies() | Draws every enemy and rock; one that was just hit is hidden for a moment. |

### Source code

1  /*

2   * enemies.h -- how each enemy and rock moves and shoots. Every one moves slowly (the

3   * player is always faster); each type has its own shot:

4   *   drone     sinks, shoots straight down

5   *   weaver    sinks while zig-zagging, shoots straight down now and then

6   *   miner     crosses the top of the screen and drops mines

7   *   cruiser   stops near the top, slides sideways and fires shots aimed at the player

8   *   rocks     fall and drift; a big rock breaks into two small ones

9   */

10  #pragma once

11  #include "world.h"

12

13  const int8_t ENEMY_FIRE_MAX_Y = 9;

14

15  // True if enemy e, placed at (x, y), would touch another enemy: with gap = 1 also when they

16  // are only side by side with no empty pixel between them.

17  bool touchesOther(const Enemy& e, int8_t x, int8_t y, uint8_t gap) {

18    const EnemyInfo& info = infoOf(e);

19    for (uint8_t i = 0; i < MAX_ENEMIES; i++) {

20      const Enemy& o = enemies[i];

21      if (!o.alive || &o == &e) continue;

22      const EnemyInfo& oi = infoOf(o);

23      if (boxesTouch(x - gap, y - gap, info.w + 2 * gap, info.h + 2 * gap, o.x, o.y, oi.w, oi.h))

24        return true;

25    }

26    return false;

27  }

28

29  // True if enemy e may move to (x, y). On 16 x 24 LEDs two enemies on the same pixels merge

30  // into one unreadable blob, so a move may not bring it closer than one empty pixel to

31  // another enemy (a move is still allowed if they were already that close, so two enemies

32  // side by side can't block each other forever).

33  bool canMoveTo(const Enemy& e, int8_t x, int8_t y) {

34    if (touchesOther(e, x, y, 0) && !touchesOther(e, e.x, e.y, 0)) return false;

35    return !touchesOther(e, x, y, 1) || touchesOther(e, e.x, e.y, 1);

36  }

37

38  // Moves an enemy one pixel down. If something is in the way it steps aside instead (its

39  // own direction first), or waits for this step.

40  void moveDown(Enemy& e) {

41    if (canMoveTo(e, e.x, e.y + 1)) { e.y++; return; }

42    const int8_t maxX = SCREEN_W - infoOf(e).w;

43    int8_t d = e.dir ? e.dir : 1;

44    for (uint8_t i = 0; i < 2; i++, d = -d) {

45      int8_t x = e.x + d;

46      if (x >= 0 && x <= maxX && canMoveTo(e, x, e.y)) { e.x = x; return; }

47    }

48  }

49

50  // Moves an enemy one pixel sideways in its direction; if something is in the way (or the

51  // screen edge: only miners fly in and out at the sides) it turns around instead. Returns

52  // true if it moved.

53  bool moveSide(Enemy& e) {

54    int8_t x = e.x + e.dir;

55    bool inside = e.type == EN_MINER || (x >= 0 && x <= SCREEN_W - infoOf(e).w);

56    if (inside && canMoveTo(e, x, e.y)) { e.x = x; return true; }

57    e.dir = -e.dir;

58    return false;

59  }

60

61  // Puts an enemy on the screen just above the top (miners come in from the side).

62  void spawnFromTable(uint8_t type, int8_t x) {

63    const EnemyInfo& info = ENEMY_INFO[type];

64    if (type == EN_MINER) {

65      bool fromLeft = x < SCREEN_W / 2;

66      Enemy* e = spawnEnemy(type, fromLeft ? -3 : SCREEN_W, 1 + random(3));

67      if (!e) return;

68      e->dir = fromLeft ? 1 : -1;

69      while (e->x > -20 && e->x < SCREEN_W + 20 && touchesOther(*e, e->x, e->y, 1))

70        e->x -= e->dir;                    // another one is there: queue up further outside

71      return;

72    }

73    Enemy* e = spawnEnemy(type, constrain(x, 0, SCREEN_W - info.w), -info.h);

74    if (!e) return;

75    // Something is already entering there: take the nearest free column (+1, -1, +2, -2 ...),

76    // and if the whole row is busy, queue up higher above the screen.

77    for (int8_t d = 1; d < 2 * SCREEN_W && touchesOther(*e, e->x, e->y, 1); d++) {

78      int8_t tryX = e->x + ((d & 1) ? (d + 1) / 2 : -d / 2);

79      if (tryX >= 0 && tryX <= SCREEN_W - info.w && !touchesOther(*e, tryX, e->y, 1)) e->x = tryX;

80    }

81    while (e->y > -20 && touchesOther(*e, e->x, e->y, 1)) e->y--;

82    if (type == EN_ROCK_S || type == EN_ROCK_B) e->dir = random(3) - 1;   // drift

83  }

84

85  // Moves one enemy or rock for one game step and lets it shoot; each type has its own

86  // pattern (see the list at the top). Anything that leaves at the bottom is removed.

87  void updateEnemy(Enemy& e) {

88    const EnemyInfo& info = infoOf(e);

89    // Enemies only shoot from the upper part of the screen: a bullet fired right next to

90    // the ship could not be dodged. The ship may fly up to row 9 too, so a bullet must also

91    // start at least 3 rows above it.

92    bool canShoot = e.y >= 0 && e.y <= ENEMY_FIRE_MAX_Y && e.y + info.h + 3 <= player.y;

93    switch (e.type) {

94      case EN_DRONE:

95        if (gameMs >= e.nextMove) { moveDown(e); e.nextMove += info.stepMs; }

96        if (gameMs >= e.nextShot) {

97          if (canShoot) enemyShot(e.x + 1, e.y + 2, 0, 1, 110);

98          e.nextShot = gameMs + fireDelay(2600 + random(1800));

99        }

100        break;

101

102      case EN_WEAVER:

103        if (gameMs >= e.nextMove) { moveDown(e); e.nextMove += info.stepMs; }

104        if (gameMs >= e.nextSide) {              // zig-zag: three steps one way, then back

105          if (!moveSide(e)) e.steps = 0;         // blocked: zig-zags back early

106          else if (++e.steps >= 3 || e.x <= 0 || e.x >= SCREEN_W - info.w) {

107            e.dir = -e.dir;

108            e.steps = 0;

109          }

110          e.nextSide = gameMs + 220;

111        }

112        if (gameMs >= e.nextShot) {

113          if (canShoot) enemyShot(e.x + 1, e.y + 2, 0, 1, 100);

114          e.nextShot = gameMs + fireDelay(4000 + random(2000));

115        }

116        break;

117

118      case EN_MINER:

119        if (gameMs >= e.nextMove) { moveSide(e); e.nextMove += info.stepMs; }   // blocked: turns back

120        if (gameMs >= e.nextShot) {

121          if (e.x >= 0 && e.x <= SCREEN_W - info.w) newShot(e.x + 1, e.y + 2, SHOT_MINE, 300);

122          e.nextShot = gameMs + fireDelay(1300);

123        }

124        if ((e.dir > 0 && e.x > SCREEN_W) || (e.dir < 0 && e.x < -info.w)) e.alive = false;

125        break;

126

127      case EN_CRUISER:

128        if (e.y < 3) {

129          if (gameMs >= e.nextMove) { moveDown(e); e.nextMove += info.stepMs; }

130        } else if (gameMs >= e.nextSide) {

131          if (moveSide(e) && (e.x <= 0 || e.x >= SCREEN_W - info.w)) e.dir = -e.dir;

132          e.nextSide = gameMs + 650;

133        }

134        if (gameMs >= e.nextShot) {

135          if (e.y >= 0) aimedShot(e.x + 2, e.y + 3, 100);

136          e.nextShot = gameMs + fireDelay(1800);

137        }

138        break;

139

140      default:                                   // rocks

141        if (gameMs >= e.nextMove) { moveDown(e); e.nextMove += info.stepMs; }

142        if (e.dir && gameMs >= e.nextSide) {

143          if (e.x + e.dir < 0 || e.x + e.dir > SCREEN_W - info.w) e.dir = -e.dir;   // edge

144          else moveSide(e);

145          e.nextSide = gameMs + 1400;

146        }

147        break;

148    }

149    if (e.y >= SCREEN_H) e.alive = false;        // left the screen at the bottom

150    if (e.flash) e.flash--;

151  }

152

153  // Updates every enemy and rock that is alive (once per game step).

154  void updateEnemies() {

155    for (uint8_t i = 0; i < MAX_ENEMIES; i++)

156      if (enemies[i].alive) updateEnemy(enemies[i]);

157  }

158

159  // Draws every enemy and rock; one that was just hit is hidden for a moment.

160  void drawEnemies() {

161    for (uint8_t i = 0; i < MAX_ENEMIES; i++) {

162      const Enemy& e = enemies[i];

163      if (!e.alive || e.flash) continue;   // a hit enemy disappears for a moment (120 ms)

164      const EnemyInfo& info = infoOf(e);

165      drawBitmap(e.x, e.y, info.sprite, info.w, info.h, true);

166    }

167  }

## 4.17 levels.h – The three phases

The spawn list of each phase (wait, what, where), its enemy limit, its name, its song and its boss. The lists are stored in flash.

Figure 33. What levels.h draws or uses.

| Function | What it does |
| --- | --- |
| spawnAt(uint8_t phase, uint8_t i) | Copies entry i of a phase's spawn list from flash into RAM. |

### Source code

1  /*

2   * levels.h -- the three phases. Each phase is a list of spawns: wait (in 0.1 s after the

3   * previous spawn), what, and x. SP_CLEAR waits until the screen is empty; SP_AMMO and

4   * SP_LIFE drop a pickup. After the last entry (and an empty screen) the boss comes.

5   * The enemies are slow, so they would pile up: the list also waits while the phase's

6   * maximum number of enemies is on the screen (that keeps 16 x 24 LEDs readable).

7   */

8  #pragma once

9  #include "world.h"

10  #include "tunes.h"

11

12  struct Spawn { uint8_t wait, what; int8_t x; };

13  const uint8_t SP_AMMO = 10, SP_LIFE = 11, SP_CLEAR = 12;

14  #define DR EN_DRONE

15  #define WV EN_WEAVER

16  #define MN EN_MINER

17  #define CR EN_CRUISER

18  #define RS EN_ROCK_S

19  #define RB EN_ROCK_B

20

21  // ---- phase 1: Rock Belt (asteroids) -- drones and lots of rocks

22  const Spawn PHASE1[] PROGMEM = {

23    {15, RS, 3}, {12, DR, 2}, {8, DR, 11}, {12, RB, 6}, {12, DR, 6},

24    {0, DR, 10}, {12, RS, 12}, {10, SP_CLEAR, 0}, {5, SP_AMMO, 7},

25    {10, DR, 1}, {5, DR, 6}, {5, DR, 11}, {14, RB, 3}, {12, DR, 4},

26    {0, DR, 9}, {12, RS, 7}, {10, DR, 2}, {4, DR, 7}, {4, DR, 12},

27    {12, RB, 0}, {10, SP_CLEAR, 0}, {5, SP_AMMO, 3},

28    {8, DR, 0}, {3, DR, 4}, {3, DR, 8}, {3, DR, 12}, {14, RB, 6},

29    {12, RS, 12}, {10, SP_CLEAR, 0},

30  };

31

32  // ---- phase 2: Deep Mist (a nebula) -- weavers and mine-layers

33  const Spawn PHASE2[] PROGMEM = {

34    {15, WV, 3}, {10, WV, 10}, {14, MN, 0}, {12, DR, 6}, {10, WV, 1},

35    {6, WV, 12}, {14, DR, 7}, {10, MN, 15}, {12, DR, 2}, {0, DR, 11},

36    {12, SP_CLEAR, 0}, {5, SP_AMMO, 6},

37    {10, WV, 5}, {4, WV, 9}, {12, MN, 0}, {8, MN, 15}, {14, RB, 6},

38    {10, DR, 1}, {4, DR, 7}, {4, DR, 12}, {12, WV, 2}, {6, WV, 11},

39    {10, RS, 4}, {12, SP_CLEAR, 0}, {5, SP_LIFE, 9},

40    {10, MN, 0}, {6, WV, 6}, {6, WV, 9}, {10, MN, 15}, {10, DR, 3},

41    {4, DR, 10}, {12, SP_CLEAR, 0}, {5, SP_AMMO, 4},

42  };

43

44  // ---- phase 3: The Void -- cruisers, everything else, and rock showers

45  const Spawn PHASE3[] PROGMEM = {

46    {15, CR, 5}, {14, DR, 1}, {4, DR, 12}, {12, RS, 3}, {8, RS, 12},

47    {14, WV, 6}, {12, SP_CLEAR, 0}, {5, SP_AMMO, 5},

48    {10, CR, 1}, {16, CR, 10}, {10, RB, 6}, {8, RS, 14},

49    {12, WV, 2}, {4, WV, 11}, {12, MN, 0}, {12, SP_CLEAR, 0}, {5, SP_AMMO, 9},

50    {8, RS, 2}, {6, RS, 10}, {8, RB, 5},

51    {12, DR, 0}, {3, DR, 5}, {3, DR, 10}, {10, CR, 5}, {12, WV, 1},

52    {4, WV, 12}, {12, SP_CLEAR, 0}, {5, SP_LIFE, 7}, {5, SP_AMMO, 3},

53  };

54

55  #undef DR

56  #undef WV

57  #undef MN

58  #undef CR

59  #undef RS

60  #undef RB

61

62  struct PhaseInfo {

63    const Spawn* spawns;

64    uint8_t count;

65    uint8_t maxEnemies;                    // at most this many on the screen at once

66    const char* name1;                     // the phase name: two words of up to 4

67    const char* name2;                     // letters (in flash), shown as a page

68    const Track* song;

69    uint8_t songTickMs;

70    uint8_t boss;

71  };

72  const char NAME_ROCK[] PROGMEM = "ROCK";

73  const char NAME_BELT[] PROGMEM = "BELT";

74  const char NAME_DEEP[] PROGMEM = "DEEP";

75  const char NAME_MIST[] PROGMEM = "MIST";

76  const char NAME_THE[] PROGMEM = "THE";

77  const char NAME_VOID[] PROGMEM = "VOID";

78  const PhaseInfo PHASES[3] = {

79    {PHASE1, sizeof(PHASE1) / sizeof(Spawn), 4, NAME_ROCK, NAME_BELT, SONG_PHASE1, SONG_PHASE1_TICK_MS, BOSS_KIND_WARDEN},

80    {PHASE2, sizeof(PHASE2) / sizeof(Spawn), 5, NAME_DEEP, NAME_MIST, SONG_PHASE2, SONG_PHASE2_TICK_MS, BOSS_KIND_HUNTER},

81    {PHASE3, sizeof(PHASE3) / sizeof(Spawn), 6, NAME_THE, NAME_VOID, SONG_PHASE3, SONG_PHASE3_TICK_MS, BOSS_KIND_KING},

82  };

83

84  // Copies entry i of a phase's spawn list from flash into RAM.

85  Spawn spawnAt(uint8_t phase, uint8_t i) {

86    Spawn s;

87    memcpy_P(&s, &PHASES[phase].spawns[i], sizeof(Spawn));

88    return s;

89  }

## 4.18 boss.h – The bosses

The behaviour of the WARDEN, the HUNTER (a small state machine: dash, aim, charge) and the VOID KING (two forms), their health and their drawing.

Figure 34. What boss.h draws or uses.

| Function | What it does |
| --- | --- |
| bossStart(uint8_t kind) | Gets the boss of a phase ready: its size, its health (scaled by the difficulty), a start just above the screen so it slides in, and the times of its first shot and action. |
| bossAngry() | True when the boss has lost half its health or more (it then attacks harder). |
| bossSlide(uint16_t ms) | Moves the boss one pixel sideways every `ms`, turning around at the screen edges. |
| updateWarden() | WARDEN's turn: slides along the top, fires a 3-way spread (5-way when angry) and drops rocks every 7 seconds. |
| updateHunter() | HUNTER's turn, a small state machine (boss.mode): FIGHT picks the next move, DASH flies fast to a random column, AIM fires a pair of aimed shots, CHARGE_DOWN / CHARGE_UP dives at the player and climbs back. |
| updateKing() | VOID KING's turn. Below half health it changes form once (eye opens, roar, shake). Form 1: 5-way spread and calls in drones. Form 2: aimed triple volleys and rock rain. |
| updateBoss() | Moves the boss for one game step: first it slides in from the top, then it runs the behaviour of its kind. |
| drawBoss(uint16_t frameNo) | Draws the boss; while boss.flash counts down it blinks (before a dive, at the form change and while it burns). |

### Source code

1  /*

2   * boss.h -- the three bosses (health is shown on the red status bar).

3   *   WARDEN     (phase 1) wide and slow, slides along the top, 3-way spread shots,

4   *              drops rocks; gets faster below half health

5   *   HUNTER     (phase 2) small and FAST: dashes to a new spot, fires an aimed pair,

6   *              and every few seconds charges straight down at the player

7   *   VOID KING  (phase 3) huge; 5-way spread and calls in drones; below half health it

8   *              changes form (the eye opens), fires aimed volleys and makes rocks rain

9   */

10  #pragma once

11  #include "world.h"

12  #include "enemies.h"

13

14  enum BossMode : uint8_t { BM_ENTER, BM_FIGHT, BM_DASH, BM_AIM, BM_CHARGE_DOWN, BM_CHARGE_UP };

15  const uint16_t BOSS_SCORE[3] = {200, 300, 500};

16

17  // Gets the boss of a phase ready: its size, its health (scaled by the difficulty), a start

18  // just above the screen so it slides in, and the times of its first shot and action.

19  void bossStart(uint8_t kind) {

20    boss = {};

21    boss.alive = true;

22    boss.kind = kind;

23    const uint8_t W[3] = {10, 5, 12}, H[3] = {4, 3, 5}, HP[3] = {24, 22, 40};

24    boss.w = W[kind];

25    boss.h = H[kind];

26    boss.maxHp = (int16_t)HP[kind] * DIFF_BOSS_HP[settings.difficulty] / 100;

27    boss.hp = boss.maxHp;

28    boss.x = (SCREEN_W - boss.w) / 2;

29    boss.y = -boss.h;

30    boss.dir = 1;

31    boss.mode = BM_ENTER;

32    boss.nextMove = gameMs;

33    boss.nextShot = gameMs + 2500;

34    boss.nextAction = gameMs + 4000;

35  }

36

37  // True when the boss has lost half its health or more (it then attacks harder).

38  bool bossAngry() {

39    return boss.hp * 2 <= boss.maxHp;

40  }

41

42  // Moves the boss one pixel sideways every `ms`, turning around at the screen edges.

43  void bossSlide(uint16_t ms) {

44    if (gameMs < boss.nextMove) return;

45    boss.x += boss.dir;

46    if (boss.x <= 0 || boss.x >= SCREEN_W - boss.w) boss.dir = -boss.dir;

47    boss.nextMove = gameMs + ms;

48  }

49

50  // WARDEN's turn: slides along the top, fires a 3-way spread (5-way when angry) and drops

51  // rocks every 7 seconds.

52  void updateWarden() {

53    bossSlide(bossAngry() ? 90 : 150);

54    if (gameMs >= boss.nextShot) {       // 3-way spread from the three "guns"

55      int8_t y = boss.y + boss.h;

56      enemyShot(boss.x + 1, y, -1, 2, 90);

57      enemyShot(boss.x + 4, y, 0, 1, 90);

58      enemyShot(boss.x + 8, y, 1, 2, 90);

59      if (bossAngry()) {

60        enemyShot(boss.x, y, -1, 1, 90);

61        enemyShot(boss.x + 9, y, 1, 1, 90);

62      }

63      boss.nextShot = gameMs + fireDelay(bossAngry() ? 1000 : 1600);

64    }

65    if (gameMs >= boss.nextAction) {     // drops rocks

66      spawnFromTable(EN_ROCK_S, random(SCREEN_W - 3));

67      if (bossAngry()) spawnFromTable(EN_ROCK_S, random(SCREEN_W - 3));

68      boss.nextAction = gameMs + 7000;

69    }

70  }

71

72  // HUNTER's turn, a small state machine (boss.mode): FIGHT picks the next move, DASH flies

73  // fast to a random column, AIM fires a pair of aimed shots, CHARGE_DOWN / CHARGE_UP dives

74  // at the player and climbs back.

75  void updateHunter() {

76    switch (boss.mode) {

77      case BM_FIGHT:

78        if (gameMs < boss.modeUntil) break;        // short rest after shooting

79        if (gameMs >= boss.nextAction) {           // time for a charge

80          boss.mode = BM_CHARGE_DOWN;

81          boss.flash = 15;

82          boss.burst = 0;

83          boss.nextMove = gameMs + 300;            // short warning blink first

84        } else {

85          boss.targetX = random(SCREEN_W - boss.w + 1);

86          boss.mode = BM_DASH;

87        }

88        break;

89      case BM_DASH:                                // fast: one pixel every 40 ms

90        if (gameMs >= boss.nextMove) {

91          boss.x += (boss.targetX > boss.x) - (boss.targetX < boss.x);

92          boss.nextMove = gameMs + (bossAngry() ? 30 : 40);

93          if (boss.x == boss.targetX) {

94            boss.mode = BM_AIM;

95            boss.modeUntil = gameMs + 350;

96            boss.burst = 0;

97          }

98        }

99        break;

100      case BM_AIM:                                 // pause, then an aimed pair of shots

101        if (gameMs >= boss.modeUntil) {

102          aimedShot(boss.x + 2, boss.y + boss.h, 85);

103          boss.modeUntil = gameMs + 160;

104          if (++boss.burst >= (bossAngry() ? 3 : 2)) {

105            boss.mode = BM_FIGHT;

106            boss.modeUntil = gameMs + fireDelay(500);

107          }

108        }

109        break;

110      case BM_CHARGE_DOWN:                         // dives, steering toward the player

111        if (gameMs >= boss.nextMove) {

112          boss.y++;

113          if (++boss.burst % 3 == 0) boss.x += (player.x - 1 > boss.x) - (player.x - 1 < boss.x);

114          boss.x = constrain(boss.x, 0, SCREEN_W - boss.w);

115          boss.nextMove = gameMs + 35;

116          if (boss.y >= 15) boss.mode = BM_CHARGE_UP;

117        }

118        break;

119      case BM_CHARGE_UP:

120        if (gameMs >= boss.nextMove) {

121          boss.y--;

122          boss.nextMove = gameMs + 60;

123          if (boss.y <= 1) {

124            boss.mode = BM_FIGHT;

125            boss.nextAction = gameMs + (bossAngry() ? 5000 : 7000);

126          }

127        }

128        break;

129    }

130  }

131

132  // VOID KING's turn. Below half health it changes form once (eye opens, roar, shake).

133  // Form 1: 5-way spread and calls in drones. Form 2: aimed triple volleys and rock rain.

134  void updateKing() {

135    if (!boss.form && bossAngry()) {               // second form

136      boss.form = 1;

137      boss.flash = 20;

138      shake(25);

139      explode(boss.x + boss.w / 2, boss.y + 2, 14, 12);

140      soundSfx(SFX(SFX_ROAR), 6);

141      Serial.println(F("[VR] boss form 2"));

142    }

143    bossSlide(boss.form ? 200 : 260);

144    int8_t y = boss.y + boss.h;

145    if (gameMs >= boss.nextShot) {

146      if (!boss.form) {                            // 5-way spread

147        enemyShot(boss.x + 1, y, -1, 1, 95);

148        enemyShot(boss.x + 3, y, -1, 2, 95);

149        enemyShot(boss.x + 6, y, 0, 1, 95);

150        enemyShot(boss.x + 8, y, 1, 2, 95);

151        enemyShot(boss.x + 10, y, 1, 1, 95);

152        boss.nextShot = gameMs + fireDelay(2200);

153      } else {                                     // aimed volley from three points

154        aimedShot(boss.x + 2, y, 90);

155        aimedShot(boss.x + 6, y, 90);

156        aimedShot(boss.x + 9, y, 90);

157        boss.nextShot = gameMs + fireDelay(1700);

158      }

159    }

160    if (gameMs >= boss.nextAction) {

161      if (!boss.form) {                            // calls in drones

162        uint8_t drones = 0;

163        for (uint8_t i = 0; i < MAX_ENEMIES; i++)

164          drones += enemies[i].alive && enemies[i].type == EN_DRONE;

165        if (drones < 2) spawnFromTable(EN_DRONE, random(SCREEN_W - 3));

166        boss.nextAction = gameMs + 6000;

167      } else {                                     // rock rain

168        spawnFromTable(EN_ROCK_S, random(SCREEN_W - 3));

169        boss.nextAction = gameMs + 2200;

170      }

171    }

172  }

173

174  // Moves the boss for one game step: first it slides in from the top, then it runs the

175  // behaviour of its kind.

176  void updateBoss() {

177    if (!boss.alive) return;

178    if (boss.flash) boss.flash--;

179    if (boss.mode == BM_ENTER) {                   // slides in from the top

180      if (gameMs >= boss.nextMove) {

181        boss.y++;

182        boss.nextMove = gameMs + 120;

183        if (boss.y >= 1) {

184          boss.mode = BM_FIGHT;

185          boss.nextAction = gameMs + 4000;

186        }

187      }

188      return;

189    }

190    if (boss.kind == BOSS_KIND_WARDEN) updateWarden();

191    else if (boss.kind == BOSS_KIND_HUNTER) updateHunter();

192    else updateKing();

193  }

194

195  // Draws the boss; while boss.flash counts down it blinks (before a dive, at the form

196  // change and while it burns).

197  void drawBoss(uint16_t frameNo) {

198    if (!boss.alive || (boss.flash && (frameNo & 8))) return;   // blinks (warning, burning)

199    drawBitmap(boss.x, boss.y, bossSprite(), boss.w, boss.h, true);

200  }

## 4.19 game.h – One game

Runs one game: the stages of a phase (intro, waves, warning, boss, boss down, clear), the spawner, all collisions (bullets, enemies, rocks, mines, pickups, boss), the score, the messages on top of the game and the three status bars.

Figure 35. What game.h draws or uses.

| Function | What it does |
| --- | --- |
| stageTime() | ms of game time since the current stage began. |
| setStage(uint8_t stage) | Switches the phase to another stage and restarts the stage clock. |
| playStageMusic() | The music that belongs to the current stage (also used after a pause). |
| phaseBegin(uint8_t phase) | Starts a phase: empties the screen, puts the ship back with full ammo, plays the fanfare and begins the INTRO stage. |
| gameStart(uint8_t phase) | Starts a new game: game clock at 0, the chosen ship, lives and score 0, then the phase (normally phase 1; the test hooks !2 and !3 start later). |
| destroyEnemy(Enemy& e, bool byPlayer) | Removes an enemy or rock with a boom; a big rock breaks into two small ones. Destroyed by the player (byPlayer), it also gives points and maybe drops a pickup. |
| hitEnemy(Enemy& e) | A player bullet hits an enemy: it loses one health point and disappears for a moment, or it is destroyed when it had only one left. |
| pointIn(int8_t px, int8_t py, int8_t x, int8_t y, uint8_t w, uint8_t h) | True when the point (px, py) lies inside the box at (x, y) of size w x h. |
| playerShotHits(Shot& s) | A player bullet against enemies, the boss and mines. |
| shotsStep() | Moves every bullet and mine one pixel when its time comes (player bullets up, enemy bullets down and maybe sideways) and checks what each of them hits. |
| enemyTouchesShip(const Enemy& e) | True when a lit pixel of the enemy or rock lies on a lit pixel of the ship, so the empty corners of a hollow rock (or between a drone's wings) don't count as a crash. |
| contactsStep() | Flying into an enemy, a rock or the boss also costs a life. |
| collect(Pickup& p) | The ship caught a pickup: ammo crate = more ammo (up to the ship's maximum), plus = one extra life (up to 4). Both give 5 points. |
| pickupsStep() | Lets the pickups fall, checks whether the ship catches one, and sends an ammo crate when the player is about to run out. |
| particlesStep() | Moves the explosion pixels and counts down their life (one game step). |
| spawnerStep() | Works through the phase's spawn list: when the wait is over, puts the next enemy, rock or pickup on the screen. Waits while the screen is full, and at SP_CLEAR until it is empty. |
| bossDown() | The boss has no health left: points, its bullets and helpers vanish, big explosion and screen shake; begins the BOSS_DOWN stage. |
| phaseClear() | A phase is won: bonus points (100 x phase), one extra life and the 'clear' tune. |
| stageStep() | Runs the current stage for one game step and decides when the next one begins (INTRO -> WAVES -> WARNING -> BOSS -> BOSS_DOWN -> CLEAR -> next phase). Returns GAME_OVER or GAME_WON when the game ends. |
| gameStep() | One game step (20 ms of game time): the ship, enemies, bullets, hits, pickups and explosions each move once, then the stage logic runs. |
| drawGameStatus(uint16_t frameNo) | Sets the three status bars: red = phase number / wave progress / boss health, yellow = ammo (blinks when low), green = one block of 3 LEDs per life. |
| drawGameTexts() | The messages on top of the game, as still pages (see display.h): INTRO "ZONE" + the phase number big, then the phase name, then "GO!" WARNING "WARN" in the red zone and a blinking warning sign CLEAR "ZONE DONE", the bonus points, "LIFE UP" (if a life was added) demo every 9 s: "DEMO", then "ANY KEY" |
| drawGame(uint16_t frameNo) | Draws one picture of the game: pickups, enemies, boss, bullets, ship and explosions (shifted while the screen shakes), then the texts on top and the status bars. |
| gameFrame(uint16_t frameNo) | One picture of the game: keys, game steps, drawing. |
| gameSkipToBoss() | Test hook !b: removes the enemies and ends the spawn list, so the boss comes next. |
| gamePrintState() | Test hook !s: prints phase, stage, score, lives, ammo, ship position, enemies, boss health and game time over Serial. |

### Source code

1  /*

2   * game.h -- one game, from PLAY to game over or victory.

3   *

4   * Each of the three phases runs through these stages:

5   *   INTRO      the phase number and name as pages, a short fanfare

6   *   WAVES      enemies and rocks arrive from the phase's spawn list (levels.h)

7   *   WARNING    "WARN" and a blinking warning sign, a siren, the red bar charges up

8   *   BOSS       boss fight with its own music; the red bar shows the boss's health

9   *   BOSS_DOWN  the boss blows up, the screen shakes

10   *   CLEAR      bonus points and one extra life, then the next phase (after the last

11   *              boss the game goes straight to the victory screen)

12   * and DEAD when the last life is lost.

13   *

14   * Status bars: red = phase progress / boss health, yellow = ammo (blinks when low),

15   * green = lives (one block of three LEDs per life).

16   * The game runs in steps of 20 ms of game time (gameMs); a picture is drawn after each

17   * step. Pause simply stops calling it. Test hook !f runs several steps per picture.

18   */

19  #pragma once

20  #include "world.h"

21  #include "player.h"

22  #include "autopilot.h"

23  #include "enemies.h"

24  #include "boss.h"

25  #include "levels.h"

26

27  enum GameStage : uint8_t { GS_INTRO, GS_WAVES, GS_WARNING, GS_BOSS, GS_BOSS_DOWN, GS_CLEAR, GS_DEAD };

28  enum GameResult : uint8_t { GAME_ON, GAME_OVER, GAME_WON };

29

30  struct Game {

31    uint8_t phase, stage, spawnIndex;

32    uint32_t stageStart;                   // gameMs

33    uint32_t nextSpawn, nextAmmoHelp, nextBoom;

34    uint16_t bonus;

35    bool lifeUp;                           // the phase clear gave an extra life

36  };

37  Game game;

38  uint8_t gameSpeed = 1;                   // steps per picture (test hook !f: fast)

39  bool demoMode = false;                   // the title screen's demo: the autopilot plays

40

41  const uint16_t INTRO_MS = 3 * PAGE_MS, WARNING_MS = 3000, BOSS_DOWN_MS = 2500;

42  const uint16_t CLEAR_MS = 3200, DEAD_MS = 2500;

43  const uint16_t PICKUP_STEP_MS = 250;

44  const uint8_t DROP_CHANCE = 12;          // % chance that a destroyed enemy drops ammo

45

46  // ms of game time since the current stage began.

47  uint32_t stageTime() {

48    return gameMs - game.stageStart;

49  }

50

51  // Switches the phase to another stage and restarts the stage clock.

52  void setStage(uint8_t stage) {

53    game.stage = stage;

54    game.stageStart = gameMs;

55  }

56

57  // The music that belongs to the current stage (also used after a pause).

58  void playStageMusic() {

59    const PhaseInfo& ph = PHASES[game.phase];

60    if (game.stage == GS_WAVES) soundPlaySong(ph.song, ph.songTickMs, true);

61    else if (game.stage == GS_BOSS) soundPlaySong(SONG_BOSS, SONG_BOSS_TICK_MS, true);

62  }

63

64  // Starts a phase: empties the screen, puts the ship back with full ammo, plays the

65  // fanfare and begins the INTRO stage.

66  void phaseBegin(uint8_t phase) {

67    game.phase = phase;

68    game.spawnIndex = 0;

69    game.nextAmmoHelp = 0;

70    worldClear();

71    playerStart(false);

72    setStage(GS_INTRO);

73    soundStop();

74    soundPlaySong(SONG_STING, 100, false);

75    Serial.print(F("[VR] phase "));

76    Serial.println(phase + 1);

77  }

78

79  // Starts a new game: game clock at 0, the chosen ship, lives and score 0, then the phase

80  // (normally phase 1; the test hooks !2 and !3 start later).

81  void gameStart(uint8_t phase) {

82    gameMs = 0;

83    playerStart(true);

84    phaseBegin(phase);

85  }

86

87  // ================================================================= hits and pickups

88  // Removes an enemy or rock with a boom; a big rock breaks into two small ones. Destroyed

89  // by the player (byPlayer), it also gives points and maybe drops a pickup.

90  void destroyEnemy(Enemy& e, bool byPlayer) {

91    const EnemyInfo& info = infoOf(e);

92    int8_t x = e.x, y = e.y;

93    uint8_t type = e.type;

94    e.alive = false;                       // it just vanishes: flying debris only for the

95    soundSfx(SFX(SFX_BOOM), 3);            // boss and the player's ship (easier to read)

96    if (type == EN_ROCK_B) {               // a big rock breaks into two small ones

97      const int8_t w = ENEMY_INFO[EN_ROCK_S].w;          // side by side, also at the edges

98      const int8_t ax = constrain(x - 1, 0, SCREEN_W - 2 * w);

99      Enemy* a = spawnEnemy(EN_ROCK_S, ax, y);

100      Enemy* b = spawnEnemy(EN_ROCK_S, ax + w, y);

101      if (a) a->dir = -1;

102      if (b) b->dir = 1;

103    }

104    if (!byPlayer) return;

105    addScore(info.score);

106    if (type == EN_CRUISER && player.lives < MAX_LIVES) spawnPickup(PK_LIFE, x + 1, y);

107    else if (random(100) < DROP_CHANCE && !pickupAlive(PK_AMMO)) spawnPickup(PK_AMMO, x, y);

108  }

109

110  // A player bullet hits an enemy: it loses one health point and disappears for a moment,

111  // or it is destroyed when it had only one left.

112  void hitEnemy(Enemy& e) {

113    if (e.hp > 1) {

114      e.hp--;

115      e.flash = 6;

116      soundSfx(SFX(SFX_HIT), 2);

117    } else {

118      destroyEnemy(e, true);

119    }

120  }

121

122  // True when the point (px, py) lies inside the box at (x, y) of size w x h.

123  bool pointIn(int8_t px, int8_t py, int8_t x, int8_t y, uint8_t w, uint8_t h) {

124    return px >= x && px < x + w && py >= y && py < y + h;

125  }

126

127  // A player bullet against enemies, the boss and mines.

128  void playerShotHits(Shot& s) {

129    for (uint8_t i = 0; i < MAX_ENEMIES; i++) {

130      Enemy& e = enemies[i];

131      if (e.alive && pointIn(s.x, s.y, e.x, e.y, infoOf(e).w, infoOf(e).h)) {

132        s.alive = false;

133        hitEnemy(e);

134        return;

135      }

136    }

137    if (game.stage == GS_BOSS && boss.alive && boss.hp > 0 &&

138        pointIn(s.x, s.y, boss.x, boss.y, boss.w, boss.h)) {

139      s.alive = false;

140      boss.hp--;                           // (no flash: the red bar and the sound show it)

141      soundSfx(SFX(SFX_HIT), 2);

142      return;

143    }

144    for (uint8_t i = 0; i < MAX_SHOTS; i++) {   // mines can be shot too

145      Shot& m = shots[i];

146      if (m.alive && m.kind == SHOT_MINE && m.x == s.x && (m.y == s.y || m.y == s.y + 1)) {

147        m.alive = s.alive = false;

148        addScore(2);

149        soundSfx(SFX(SFX_HIT), 2);

150        return;

151      }

152    }

153  }

154

155  // Moves every bullet and mine one pixel when its time comes (player bullets up, enemy

156  // bullets down and maybe sideways) and checks what each of them hits.

157  void shotsStep() {

158    for (uint8_t i = 0; i < MAX_SHOTS; i++) {

159      Shot& s = shots[i];

160      if (!s.alive) continue;

161      if (gameMs >= s.next) {

162        s.next += s.stepMs;

163        s.y += (s.kind == SHOT_PLAYER) ? -1 : 1;

164        if (s.dx && ++s.sideCount >= s.sideEvery) {

165          s.x += s.dx;

166          s.sideCount = 0;

167        }

168        if (s.y < 0 || s.y >= SCREEN_H || s.x < 0 || s.x >= SCREEN_W) {

169          s.alive = false;

170          continue;

171        }

172      }

173      if (s.kind == SHOT_PLAYER) playerShotHits(s);

174      else if (shipPixel(s.x, s.y) && !playerShielded() && player.lives) {

175        s.alive = false;

176        playerHit(s.kind == SHOT_MINE ? F("mine") : F("shot"));

177      }

178    }

179  }

180

181  // True when a lit pixel of the enemy or rock lies on a lit pixel of the ship, so the empty

182  // corners of a hollow rock (or between a drone's wings) don't count as a crash.

183  bool enemyTouchesShip(const Enemy& e) {

184    const EnemyInfo& info = infoOf(e);

185    if (!boxesTouch(e.x, e.y, info.w, info.h, player.x, player.y, 3, 3)) return false;

186    for (uint8_t r = 0; r < info.h; r++) {

187      uint16_t bits = pgm_read_word(info.sprite + r);

188      for (uint8_t c = 0; c < info.w; c++)

189        if ((bits & (0x8000u >> c)) && shipPixel(e.x + c, e.y + r)) return true;

190    }

191    return false;

192  }

193

194  // Flying into an enemy, a rock or the boss also costs a life.

195  void contactsStep() {

196    if (playerShielded() || player.lives == 0) return;

197    for (uint8_t i = 0; i < MAX_ENEMIES; i++) {

198      Enemy& e = enemies[i];

199      if (e.alive && enemyTouchesShip(e)) {

200        bool rock = e.type >= EN_ROCK_S;

201        destroyEnemy(e, false);

202        playerHit(rock ? F("rock") : F("enemy"));

203        return;

204      }

205    }

206    if (game.stage == GS_BOSS && boss.alive &&

207        boxesTouch(boss.x, boss.y, boss.w, boss.h, player.x, player.y, 3, 3))

208      playerHit(F("boss"));

209  }

210

211  // The ship caught a pickup: ammo crate = more ammo (up to the ship's maximum), plus =

212  // one extra life (up to 4). Both give 5 points.

213  void collect(Pickup& p) {

214    p.alive = false;

215    addScore(5);

216    if (p.type == PK_AMMO) {

217      uint8_t room = playerShip().ammoMax - player.ammo, pack = DIFF_PACK_AMMO[settings.difficulty];

218      player.ammo += pack < room ? pack : room;

219      if (player.ammo > LOW_AMMO) player.lowAmmoWarned = false;

220      soundSfx(SFX(SFX_PICKUP), 4);

221      Serial.println(F("[VR] pickup ammo"));

222    } else {

223      if (player.lives < MAX_LIVES) player.lives++;

224      soundSfx(SFX(SFX_LIFE), 4);

225      Serial.println(F("[VR] pickup life"));

226    }

227  }

228

229  // Lets the pickups fall, checks whether the ship catches one, and sends an ammo crate

230  // when the player is about to run out.

231  void pickupsStep() {

232    for (uint8_t i = 0; i < MAX_PICKUPS; i++) {

233      Pickup& p = pickups[i];

234      if (!p.alive) continue;

235      if (gameMs >= p.next) {

236        p.next += PICKUP_STEP_MS;

237        if (++p.y >= SCREEN_H) p.alive = false;

238      }

239      if (p.alive && player.lives && boxesTouch(p.x, p.y, 3, 3, player.x, player.y, 3, 3)) collect(p);

240    }

241    // Running out of ammo: a crate is sent, so the game can always go on.

242    bool fighting = game.stage == GS_WAVES || game.stage == GS_BOSS;

243    if (fighting && player.ammo <= LOW_AMMO && !pickupAlive(PK_AMMO) && gameMs >= game.nextAmmoHelp) {

244      spawnPickup(PK_AMMO, random(SCREEN_W - 2), -3);

245      game.nextAmmoHelp = gameMs + 5000;

246    }

247  }

248

249  // Moves the explosion pixels and counts down their life (one game step).

250  void particlesStep() {

251    for (uint8_t i = 0; i < MAX_PARTICLES; i++) {

252      Particle& p = particles[i];

253      if (!p.life) continue;

254      p.x16 += p.vx;

255      p.y16 += p.vy;

256      p.life--;

257    }

258  }

259

260  // ================================================================= phase flow

261  // Works through the phase's spawn list: when the wait is over, puts the next enemy,

262  // rock or pickup on the screen. Waits while the screen is full, and at SP_CLEAR until

263  // it is empty.

264  void spawnerStep() {

265    const PhaseInfo& ph = PHASES[game.phase];

266    while (game.spawnIndex < ph.count && gameMs >= game.nextSpawn) {

267      Spawn s = spawnAt(game.phase, game.spawnIndex);

268      if (s.what == SP_CLEAR && enemiesAlive()) return;       // wait for an empty screen

269      if (s.what < SP_AMMO && enemiesAlive() >= ph.maxEnemies) return;   // screen is full

270      if (s.what == SP_AMMO) spawnPickup(PK_AMMO, s.x, -3);

271      else if (s.what == SP_LIFE) spawnPickup(PK_LIFE, s.x, -3);

272      else if (s.what < SP_AMMO) spawnFromTable(s.what, s.x);

273      if (++game.spawnIndex < ph.count)

274        game.nextSpawn = gameMs + spawnAt(game.phase, game.spawnIndex).wait * 100UL;

275    }

276  }

277

278  // The boss has no health left: points, its bullets and helpers vanish, big explosion and

279  // screen shake; begins the BOSS_DOWN stage.

280  void bossDown() {

281    addScore(BOSS_SCORE[boss.kind]);

282    for (uint8_t i = 0; i < MAX_SHOTS; i++)           // its bullets vanish ...

283      if (shots[i].kind != SHOT_PLAYER) shots[i].alive = false;

284    for (uint8_t i = 0; i < MAX_ENEMIES; i++)         // ... and so do its helpers

285      enemies[i].alive = false;

286    setStage(GS_BOSS_DOWN);

287    game.nextBoom = gameMs;

288    soundStop();

289    soundSfx(SFX(SFX_BIG_BOOM), 5);

290    shake(40);

291    Serial.println(F("[VR] boss down"));

292  }

293

294  // A phase is won: bonus points (100 x phase), one extra life and the 'clear' tune.

295  void phaseClear() {

296    game.bonus = PHASE_BONUS * (game.phase + 1);

297    addScore(game.bonus);

298    game.lifeUp = player.lives < MAX_LIVES;

299    if (game.lifeUp) player.lives++;

300    setStage(GS_CLEAR);

301    soundPlaySong(SONG_CLEAR, 90, false);

302    Serial.println(F("[VR] phase clear"));

303  }

304

305  // Runs the current stage for one game step and decides when the next one begins

306  // (INTRO -> WAVES -> WARNING -> BOSS -> BOSS_DOWN -> CLEAR -> next phase).

307  // Returns GAME_OVER or GAME_WON when the game ends.

308  GameResult stageStep() {

309    const PhaseInfo& ph = PHASES[game.phase];

310    switch (game.stage) {

311      case GS_INTRO:

312        if (stageTime() >= INTRO_MS) {

313          setStage(GS_WAVES);

314          playStageMusic();

315          game.nextSpawn = gameMs + spawnAt(game.phase, 0).wait * 100UL;

316        }

317        break;

318      case GS_WAVES:

319        spawnerStep();

320        if (game.spawnIndex >= ph.count && !enemiesAlive()) {

321          setStage(GS_WARNING);

322          soundStop();

323          soundSfx(SFX(SFX_SIREN), 5);

324          Serial.println(F("[VR] warning"));

325        }

326        break;

327      case GS_WARNING:

328        if (stageTime() >= WARNING_MS) {

329          bossStart(ph.boss);

330          setStage(GS_BOSS);

331          playStageMusic();

332          Serial.println(F("[VR] boss"));

333        }

334        break;

335      case GS_BOSS:

336        updateBoss();

337        if (boss.hp <= 0) bossDown();

338        break;

339      case GS_BOSS_DOWN:

340        boss.flash = 2;                                 // keeps blinking while it burns

341        if (stageTime() < 1500 && gameMs >= game.nextBoom) {

342          explode(boss.x + random(boss.w), boss.y + random(boss.h), 6, 12);

343          game.nextBoom = gameMs + 120;

344        }

345        if (boss.alive && stageTime() >= 1500) {        // final blast

346          boss.alive = false;

347          explode(boss.x + boss.w / 2, boss.y + boss.h / 2, 30, 16);

348          shake(20);

349          soundSfx(SFX(SFX_BIG_BOOM), 5);

350        }

351        if (stageTime() >= BOSS_DOWN_MS) {

352          if (game.phase == 2) {                        // last boss: bonus, then straight to

353            addScore(PHASE_BONUS * 3);                  // the victory screen (no ZONE DONE,

354            return GAME_WON;                            // no extra life)

355          }

356          phaseClear();

357        }

358        break;

359      case GS_CLEAR:

360        if (stageTime() >= CLEAR_MS) {

361          if (game.phase == 2) return GAME_WON;

362          phaseBegin(game.phase + 1);

363        }

364        break;

365      case GS_DEAD:

366        if (stageTime() >= DEAD_MS) return GAME_OVER;

367        break;

368    }

369    return GAME_ON;

370  }

371

372  // One game step (20 ms of game time): the ship, enemies, bullets, hits, pickups and

373  // explosions each move once, then the stage logic runs.

374  GameResult gameStep() {

375    gameMs += FRAME_MS;

376    if (autopilot) autopilotStep();

377    playerStep();

378    updateEnemies();

379    shotsStep();

380    contactsStep();

381    pickupsStep();

382    particlesStep();

383    if (player.lives == 0 && game.stage != GS_DEAD) {   // the last life is gone

384      setStage(GS_DEAD);

385      explode(player.x + 1, player.y + 1, 25, 16);

386      shake(30);

387      soundStop();

388      soundSfx(SFX(SFX_BIG_BOOM), 6);

389      Serial.print(F("[VR] game over score="));

390      Serial.println(player.score);

391    }

392    return stageStep();

393  }

394

395  // ================================================================= drawing

396  // Sets the three status bars: red = phase number / wave progress / boss health,

397  // yellow = ammo (blinks when low), green = one block of 3 LEDs per life.

398  void drawGameStatus(uint16_t frameNo) {

399    uint8_t red = 0;                       // red: phase progress / boss charging / boss health

400    if (game.stage == GS_WAVES) red = (uint16_t)game.spawnIndex * 16 / PHASES[game.phase].count;

401    if (game.stage == GS_WARNING && (frameNo & 8)) red = stageTime() * 16 / WARNING_MS;

402    if (game.stage == GS_BOSS && boss.hp > 0) red = (boss.hp * 16 + boss.maxHp - 1) / boss.maxHp;

403    if (game.stage == GS_CLEAR && (frameNo & 8)) red = 16;

404    if (game.stage == GS_INTRO) {          // phase number: one block per phase

405      uint16_t blocks = 0;

406      for (uint8_t i = 0; i <= game.phase; i++) blocks |= 0x7u << (i * 4);

407      setStatusBits(0, blocks);

408    } else {

409      setStatusBar(0, red);

410    }

411    uint8_t ammoMax = playerShip().ammoMax;   // yellow: ammo, blinks when low

412    uint8_t yellow = ((uint16_t)player.ammo * 16 + ammoMax - 1) / ammoMax;

413    if (player.ammo <= LOW_AMMO && (frameNo & 8)) yellow = 0;

414    setStatusBar(1, yellow);

415    uint16_t lives = 0;                    // green: one block of three LEDs per life

416    for (uint8_t i = 0; i < player.lives; i++) lives |= 0x7u << (i * 4);

417    setStatusBits(2, lives);

418  }

419

420  // The messages on top of the game, as still pages (see display.h):

421  //   INTRO    "ZONE" + the phase number big, then the phase name, then "GO!"

422  //   WARNING  "WARN" in the red zone and a blinking warning sign

423  //   CLEAR    "ZONE DONE", the bonus points, "LIFE UP" (if a life was added)

424  //   demo     every 9 s: "DEMO", then "ANY KEY"

425  void drawGameTexts() {

426    const PhaseInfo& ph = PHASES[game.phase];

427    uint32_t t = stageTime();

428    if (game.stage == GS_INTRO) {

429      uint8_t page = pageAt(t, 3);

430      if (page == 0) {

431        clearRows(0, 18);

432        drawTextCentered(0, PSTR("ZONE"), false);

433        drawChar(5, 7, '1' + game.phase, false, 2);      // 6 x 10 pixels

434      } else if (page == 1) {

435        drawPage(ph.name1, ph.name2);

436      } else {

437        drawPage(PSTR("GO!"), NULL);

438      }

439    } else if (game.stage == GS_WARNING) {

440      clearRows(0, 5);

441      drawTextCentered(0, PSTR("WARN"), false);

442      if ((t / 300) % 2) drawBitmap(2, 7, WARNING_SIGN, 11, 10, true);

443    } else if (game.stage == GS_CLEAR) {

444      uint8_t page = pageAt(t, 3);

445      if (page == 1) {

446        clearRows(6, 12);

447        drawNumberCentered(7, game.bonus, false);

448        drawTextCentered(13, PSTR("PTS"), false);

449      } else if (page == 2 && game.lifeUp) {

450        drawPage(PSTR("LIFE"), PSTR("UP"));

451      } else {

452        drawPage(PSTR("ZONE"), PSTR("DONE"));

453      }

454    } else if (demoMode && gameMs % 9000 < 2 * PAGE_MS) {

455      if (pageAt(gameMs % 9000, 2) == 0) drawPage(PSTR("DEMO"), NULL);

456      else drawPage(PSTR("ANY"), PSTR("KEY"));

457    }

458  }

459

460  // Draws one picture of the game: pickups, enemies, boss, bullets, ship and explosions

461  // (shifted while the screen shakes), then the texts on top and the status bars.

462  void drawGame(uint16_t frameNo) {

463    if (shakeFrames) {                     // screen shake: everything moves one pixel

464      shakeFrames--;

465      shakeX = random(-1, 2);

466      shakeY = random(-1, 2);

467    }

468    clearScreen();

469    for (uint8_t i = 0; i < MAX_PICKUPS; i++) {

470      const Pickup& p = pickups[i];

471      if (p.alive && (p.type == PK_AMMO || (frameNo & 8)))   // the life blinks

472        drawBitmap(p.x, p.y, p.type == PK_AMMO ? PICKUP_AMMO : PICKUP_LIFE, 3, 3, true);

473    }

474    drawEnemies();

475    drawBoss(frameNo);

476    for (uint8_t i = 0; i < MAX_SHOTS; i++) {

477      const Shot& s = shots[i];

478      if (!s.alive) continue;

479      if (s.kind == SHOT_PLAYER) {         // player bullet: a short streak

480        setPixel(s.x, s.y, true);

481        setPixel(s.x, s.y + 1, true);

482      } else {                             // enemy bullet or mine: one steady pixel

483        setPixel(s.x, s.y, true);

484      }

485    }

486    drawPlayer(frameNo);

487    for (uint8_t i = 0; i < MAX_PARTICLES; i++)

488      if (particles[i].life) setPixel(particles[i].x16 >> 4, particles[i].y16 >> 4, true);

489    shakeX = shakeY = 0;

490

491    drawGameTexts();

492    drawGameStatus(frameNo);

493  }

494

495  // One picture of the game: keys, game steps, drawing.

496  GameResult gameFrame(uint16_t frameNo) {

497    playerKeys();

498    GameResult result = GAME_ON;

499    for (uint8_t i = 0; i < gameSpeed && result == GAME_ON; i++) result = gameStep();

500    drawGame(frameNo);

501    return result;

502  }

503

504  // ================================================================= test hooks

505  // Test hook !b: removes the enemies and ends the spawn list, so the boss comes next.

506  void gameSkipToBoss() {

507    for (uint8_t i = 0; i < MAX_ENEMIES; i++) enemies[i].alive = false;

508    if (game.stage > GS_WAVES) return;

509    game.spawnIndex = PHASES[game.phase].count;

510    if (game.stage == GS_INTRO) setStage(GS_WAVES);

511  }

512

513  // Test hook !s: prints phase, stage, score, lives, ammo, ship position, enemies, boss

514  // health and game time over Serial.

515  void gamePrintState() {

516    Serial.print(F("[VR] state phase="));

517    Serial.print(game.phase + 1);

518    Serial.print(F(" stage="));

519    Serial.print(game.stage);

520    Serial.print(F(" score="));

521    Serial.print(player.score);

522    Serial.print(F(" lives="));

523    Serial.print(player.lives);

524    Serial.print(F(" ammo="));

525    Serial.print(player.ammo);

526    Serial.print(F(" x="));

527    Serial.print(player.x);

528    Serial.print(F(" y="));

529    Serial.print(player.y);

530    Serial.print(F(" enemies="));

531    Serial.print(enemiesAlive());

532    Serial.print(F(" boss="));

533    Serial.print(boss.alive ? boss.hp : 0);

534    Serial.print(F(" t="));

535    Serial.println(gameMs);

536  }

## 4.20 autopilot.h – Autopilot

A bot that plays like a careful player. It is used by the title screen's demo and by the automatic tests: for every column it works out how soon something would hit the ship there, flies to the safest column near its target and fires when lined up.

Figure 36. What autopilot.h draws or uses.

| Function | What it does |
| --- | --- |
| minMs(int32_t a, int32_t b) | Returns the smaller of two times (ms). |
| impactMs(int8_t c) | ms until something touches the ship if it stands with its left edge in column c. |
| autopilotTarget() | Chooses what the autopilot aims at: an ammo crate when ammo is low, otherwise the boss, otherwise the lowest enemy above the ship. Returns that column, or -1 if there is none. |
| autopilotStep() | The autopilot's turn (every game step while !x or the demo is on): when the ship may move again it picks the best reachable column and moves one pixel toward it, or fires when it is lined up under its target. |

### Source code

1  /*

2   * autopilot.h -- test hook (!x): the ship flies itself, like a careful player.

3   *

4   * Used by the automatic tests to play whole games and to check that every phase can be

5   * won without the shield. It follows the same rule as a player (it either moves or

6   * fires, never both) and stays on the bottom row.

7   *

8   * How it decides, each time the ship could move one pixel:

9   *   1. For every column it works out how many ms are left before a bullet, an enemy or

10   *      a rock would touch the ship standing there ("time to impact").

11   *   2. A column is reachable if the ship can fly there, one pixel per moveMs, without

12   *      passing a column where something hits it on the way.

13   *   3. Of the reachable columns that stay safe for a while, it takes the one nearest its

14   *      target (an ammo crate when ammo is low, else the boss or the lowest enemy); if

15   *      none is safe, the one where the danger arrives last.

16   *   4. Lined up under the target in a safe column, it fires.

17   */

18  #pragma once

19  #include "world.h"

20  #include "player.h"

21

22  const int32_t AUTO_NO_DANGER = 100000;   // "nothing will hit this column"

23  const int32_t AUTO_SAFE_MS = 450;        // a column must stay clear this long to stop in

24  #ifndef AUTOPILOT_REACT_MS

25  #define AUTOPILOT_REACT_MS 0             // PC tests: > 0 = rethinks only this often (slower)

26  #endif

27

28  // Returns the smaller of two times (ms).

29  int32_t minMs(int32_t a, int32_t b) {

30    return a < b ? a : b;

31  }

32

33  // ms until something touches the ship if it stands with its left edge in column c.

34  int32_t impactMs(int8_t c) {

35    int8_t top = player.y, bottom = player.y + 2;

36    int32_t first = AUTO_NO_DANGER;

37    for (uint8_t i = 0; i < MAX_SHOTS; i++) {               // enemy bullets and mines

38      const Shot& s = shots[i];

39      if (!s.alive || s.kind == SHOT_PLAYER || s.y > bottom) continue;

40      for (int8_t r = top; r <= bottom; r++) {

41        int8_t down = r - s.y;                               // rows still to fall

42        if (down < 0) continue;

43        int8_t col = s.x + (s.dx ? s.dx * ((down + s.sideCount) / s.sideEvery) : 0);

44        if (col < c || col > c + 2) continue;

45        int32_t t = down ? (int32_t)(s.next - gameMs) + (int32_t)(down - 1) * s.stepMs : 0;

46        first = minMs(first, t);

47      }

48    }

49    for (uint8_t i = 0; i < MAX_ENEMIES; i++) {             // enemies and rocks coming down

50      const Enemy& e = enemies[i];

51      bool staysUp = e.type == EN_MINER || (e.type == EN_CRUISER && e.y >= 3);   // no longer sink

52      if (!e.alive || staysUp || e.y > bottom) continue;

53      const EnemyInfo& info = infoOf(e);

54      int8_t margin = e.type == EN_WEAVER ? 2 : (e.type >= EN_ROCK_S ? 1 : 0);   // side moves

55      if (e.x - margin > c + 2 || e.x + info.w - 1 + margin < c) continue;

56      int8_t down = top - (e.y + info.h - 1);

57      int32_t wait = (int32_t)(e.nextMove - gameMs);        // until its next step down

58      int32_t t = down <= 0 ? 0 : (wait > 0 ? wait : 0) + (int32_t)(down - 1) * info.stepMs;

59      first = minMs(first, t);

60    }

61    if (boss.alive && boss.y + boss.h > top && boss.x <= c + 2 && boss.x + boss.w > c)

62      first = 0;                                             // the boss itself

63    return first;

64  }

65

66  // Chooses what the autopilot aims at: an ammo crate when ammo is low, otherwise the boss,

67  // otherwise the lowest enemy above the ship. Returns that column, or -1 if there is none.

68  int8_t autopilotTarget() {

69    if (player.ammo <= LOW_AMMO)

70      for (uint8_t i = 0; i < MAX_PICKUPS; i++)

71        if (pickups[i].alive && pickups[i].type == PK_AMMO) return pickups[i].x;

72    if (boss.alive) return boss.x + boss.w / 2 - 1;

73    int8_t target = -1, lowest = -100;

74    for (uint8_t i = 0; i < MAX_ENEMIES; i++) {

75      const Enemy& e = enemies[i];

76      if (e.alive && e.y > lowest && e.y < player.y - 6 && e.x >= 0 && e.x < SCREEN_W) {

77        lowest = e.y;

78        target = e.x + infoOf(e).w / 2 - 1;

79      }

80    }

81    return target < 0 ? -1 : constrain(target, 0, SCREEN_W - 3);

82  }

83

84  // The autopilot's turn (every game step while !x or the demo is on): when the ship may

85  // move again it picks the best reachable column and moves one pixel toward it, or fires

86  // when it is lined up under its target.

87  void autopilotStep() {

88    if (player.lives == 0 || gameMs < player.nextMove) return;

89    if (player.y < PLAYER_MAX_Y) {                           // go to the bottom row first

90      movePlayer(KEY_DOWN);

91      player.firing = false;

92      player.nextMove = gameMs + playerShip().moveMs;

93      return;

94    }

95    int32_t moveMs = playerShip().moveMs;

96    static int8_t plan = -1;                                 // column chosen last time

97    static uint32_t nextThink = 0;

98    if (AUTOPILOT_REACT_MS && gameMs < nextThink && plan >= 0) {   // still reacting

99      if (plan != player.x) {

100        movePlayer(plan > player.x ? KEY_RIGHT : KEY_LEFT);

101        player.firing = false;

102      }

103      player.nextMove = gameMs + moveMs;

104      return;

105    }

106    int32_t impact[SCREEN_W - 2];

107    for (int8_t c = 0; c <= SCREEN_W - 3; c++) impact[c] = impactMs(c);

108

109    int8_t target = autopilotTarget();

110    int8_t best = player.x;

111    int32_t bestSpare = -AUTO_NO_DANGER;                     // fallback: latest danger

112    int16_t bestDistance = 1000;                             // among safe columns

113    bool bestSafe = false;

114    for (int8_t dir = -1; dir <= 1; dir += 2) {

115      for (int8_t c = player.x, steps = 0; c >= 0 && c <= SCREEN_W - 3; c += dir, steps++) {

116        if (dir > 0 && steps == 0) continue;                 // the own column is checked once

117        int32_t arrive = steps * moveMs;

118        if (steps && impact[c] <= arrive + moveMs) break;    // would be hit on the way

119        int32_t spare = impact[c] - arrive;

120        bool safe = spare > AUTO_SAFE_MS;

121        int16_t distance = (target >= 0 ? abs(c - target) * 4 : 0) + steps;

122        if ((safe && (!bestSafe || distance < bestDistance)) || (!bestSafe && !safe && spare > bestSpare)) {

123          best = c;

124          bestSafe = safe;

125          bestSpare = spare;

126          bestDistance = distance;

127        }

128      }

129    }

130    plan = best;

131    nextThink = gameMs + AUTOPILOT_REACT_MS;

132    bool linedUp = target >= 0 && abs(target - player.x) <= 1 && player.ammo > 0;

133    if (best != player.x) {

134      movePlayer(best > player.x ? KEY_RIGHT : KEY_LEFT);   // moving: no fire

135      player.firing = false;

136    } else if (linedUp && impact[player.x] > AUTO_SAFE_MS) {

137      if (!player.firing) player.burst = 0;                  // like pressing R

138      player.firing = true;

139    } else {

140      player.firing = false;

141    }

142    player.nextMove = gameMs + moveMs;

143  }

## 4.21 screens.h – The other screens

The hangar, the options (with the LED test), the best score, pause, game over and victory. Each screen function handles its keys, draws one picture and returns the next screen.

Figure 37. What screens.h draws or uses.

| Function | What it does |
| --- | --- |
| applyBrightness() | Sets the LED brightness from the saved setting (100 / 50 / 20 %). |
| drawSideArrows(int8_t y, uint16_t frameNo) | Small arrows at the left and right edge: "A/D changes this". |
| drawPageDots(uint8_t count, uint8_t current) | One dot per page at the bottom; the current page is a bigger block. |
| clearStatusBars() | Switches off all three status bars. |
| hangarBegin() | Opens the hangar on the ship that is in use now. |
| hangarFrame(uint32_t now, uint16_t frameNo) | One picture of the hangar: A/D (or W/S) shows another ship, R takes it (saved), Q goes back. Draws name, big ship, arrows and page dots; the stats go on the status bars. |
| optionsFrame(uint16_t frameNo) | One picture of the options: W/S picks LITE, SND, DIFF or TEST; A/D or R changes it (saved in the EEPROM); R on TEST starts the LED test; Q goes back. |
| bestFrame(uint16_t frameNo) | One picture of the BEST page: the high score, then (bottom line, changing every 1.2 s) how far it got - "ZONE" and 1-3, or "WIN" - and the difficulty it was made on. |
| pauseDraw(uint32_t t, uint16_t frameNo) | Draws the pause screen: the score, a big pause sign, and the keys as a page that changes every 1.2 s ("P:GO" = P continues, "Q:EX" = Q exits), with the game's bars. |
| recordScore(uint8_t phaseReached) | Saves the score as the new best (with the phase reached, 4 = won, and the difficulty) if it beats the old one. The score counts first; the same score on a harder difficulty also takes the record. |
| gameOverFrame(uint32_t t, uint16_t frameNo) | One picture of GAME OVER: the score (blinking if it is a record) and the best score; after 1 s, R or Q goes to the menu. |
| victoryFrame(uint32_t t) | One picture of the victory screen: fireworks, pages "YOU WIN!", "VOID SAFE" and the score, and racing status bars; after 2 s, R or Q goes to the menu. |
| ledTestSetRow(uint8_t fromTop, uint16_t bits) | Sets one matrix row, counted from the top as built (status bars first, then the game screen). |
| ledTestFrame(uint32_t t) | One picture of the LED test: all on, then each row, each column and a checkerboard, with a rising beep per step. Any key (or the end) goes back to the options. |

### Source code

1  /*

2   * screens.h -- the screens around the game:

3   *   HANGAR    choose a ship (A/D), R = take it, Q = back.

4   *             Status bars: red = speed, yellow = ammo, green = fire power.

5   *   OPTIONS   W/S = item, A/D or R = change: LITE (brightness), SND (sound),

6   *             DIFF (difficulty), TEST (LED test). Saved in the EEPROM.

7   *   BEST      the high score and how far it got.

8   *   PAUSE     P or R = continue, Q = quit to the menu.

9   *   GAME OVER / VICTORY   the score; R or Q = menu.

10   *   LED TEST  every LED on, then row, column and checker patterns (hardware check).

11   * Each xxxFrame() handles the keys, draws one picture and returns the screen to show

12   * next (its own screen to stay).

13   */

14  #pragma once

15  #include "display.h"

16  #include "input.h"

17  #include "sound.h"

18  #include "storage.h"

19  #include "world.h"

20  #include "game.h"

21

22  // Sets the LED brightness from the saved setting (100 / 50 / 20 %).

23  void applyBrightness() {

24    displaySetBrightness(BRIGHTNESS_LEVELS[settings.brightness]);

25  }

26

27  // Small arrows at the left and right edge: "A/D changes this".

28  void drawSideArrows(int8_t y, uint16_t frameNo) {

29    if (frameNo & 16) return;

30    setPixel(1, y, true);

31    setPixel(0, y + 1, true);

32    setPixel(1, y + 2, true);

33    setPixel(14, y, true);

34    setPixel(15, y + 1, true);

35    setPixel(14, y + 2, true);

36  }

37

38  // One dot per page at the bottom; the current page is a bigger block.

39  void drawPageDots(uint8_t count, uint8_t current) {

40    int8_t x = (SCREEN_W - (count * 4 - 2)) / 2;

41    for (uint8_t i = 0; i < count; i++, x += 4) {

42      if (i == current) fillRect(x, 20, 2, 2, true);

43      else setPixel(x, 21, true);

44    }

45  }

46

47  // Switches off all three status bars.

48  void clearStatusBars() {

49    for (uint8_t bar = 0; bar < STATUS_ROWS; bar++) setStatusBits(bar, 0);

50  }

51

52  // ================================================================= hangar

53  uint8_t hangarShip = 0;

54  uint32_t hangarSwitched = 0;             // millis() when the ship on show changed

55

56  // Opens the hangar on the ship that is in use now.

57  void hangarBegin() {

58    hangarShip = settings.ship;

59    hangarSwitched = millis();

60  }

61

62  // One picture of the hangar: A/D (or W/S) shows another ship, R takes it (saved), Q goes

63  // back. Draws name, big ship, arrows and page dots; the stats go on the status bars.

64  Screen hangarFrame(uint32_t now, uint16_t frameNo) {

65    int8_t step = (keyPressed(KEY_RIGHT) || keyPressed(KEY_DOWN)) -

66                  (keyPressed(KEY_LEFT) || keyPressed(KEY_UP));

67    if (step) {

68      hangarShip = (hangarShip + 3 + step) % 3;

69      hangarSwitched = now;

70      soundBeep(1400, 25);

71    }

72    if (keyPressed(KEY_FIRE)) {

73      settings.ship = hangarShip;

74      settingsSave();

75      soundSfx(SFX(SFX_PICKUP), 4);

76      Serial.print(F("[VR] ship "));

77      Serial.println((const __FlashStringHelper*)SHIPS[hangarShip].name);

78      return SCR_MENU;

79    }

80    if (keyPressed(KEY_BACK)) return SCR_MENU;

81

82    const ShipInfo& ship = SHIPS[hangarShip];

83    clearScreen();

84    drawTextCentered(0, ship.name, false);

85    drawBitmapScaled(3, 6, ship.sprite, 3, 3, 3);   // the ship, 3 times bigger

86    if (frameNo & 1) fillRect(6, 15, 3, 2, true);   // engine flame

87    drawSideArrows(9, frameNo);

88    drawPageDots(3, hangarShip);

89    if (hangarShip == settings.ship) setPixel(SCREEN_W - 1, 23, true);   // the ship in use

90    // stats on the status bars, growing in after each switch: 5 LEDs per level

91    uint32_t grow = now - hangarSwitched;

92    uint8_t levels[3] = {ship.speedLevel, ship.ammoLevel, ship.fireLevel};

93    for (uint8_t bar = 0; bar < STATUS_ROWS; bar++) {

94      uint8_t full = levels[bar] * 5;

95      setStatusBar(bar, grow >= 300 ? full : full * grow / 300);

96    }

97    return SCR_HANGAR;

98  }

99

100  // ================================================================= options

101  enum OptionItem : uint8_t { OPT_LITE, OPT_SOUND, OPT_DIFF, OPT_TEST, OPTION_COUNT };

102  const char OPT_NAME_LITE[] PROGMEM = "LITE";

103  const char OPT_NAME_SND[] PROGMEM = "SND";

104  const char OPT_NAME_DIFF[] PROGMEM = "DIFF";

105  const char OPT_NAME_TEST[] PROGMEM = "TEST";

106  const char* const OPTION_NAMES[OPTION_COUNT] = {OPT_NAME_LITE, OPT_NAME_SND, OPT_NAME_DIFF, OPT_NAME_TEST};

107  const char DIFF_EASY_NAME[] PROGMEM = "EASY";

108  const char DIFF_NORMAL_NAME[] PROGMEM = "NORM";

109  const char DIFF_HARD_NAME[] PROGMEM = "HARD";

110  const char* const DIFF_NAMES[3] = {DIFF_EASY_NAME, DIFF_NORMAL_NAME, DIFF_HARD_NAME};

111  uint8_t optionItem = 0;

112

113  // One picture of the options: W/S picks LITE, SND, DIFF or TEST; A/D or R changes it

114  // (saved in the EEPROM); R on TEST starts the LED test; Q goes back.

115  Screen optionsFrame(uint16_t frameNo) {

116    if (keyPressed(KEY_UP) || keyPressed(KEY_DOWN)) {

117      optionItem = (optionItem + (keyPressed(KEY_UP) ? OPTION_COUNT - 1 : 1)) % OPTION_COUNT;

118      soundBeep(1400, 25);

119    }

120    int8_t change = keyPressed(KEY_RIGHT) - keyPressed(KEY_LEFT);

121    if (keyPressed(KEY_FIRE)) {

122      if (optionItem == OPT_TEST) return SCR_LEDTEST;

123      change = 1;

124    }

125    if (change && optionItem != OPT_TEST) {

126      if (optionItem == OPT_LITE) {

127        settings.brightness = (settings.brightness + 3 + change) % 3;

128        applyBrightness();

129      } else if (optionItem == OPT_SOUND) {

130        settings.muted = !settings.muted;

131        soundSetMuted(settings.muted);

132      } else {

133        settings.difficulty = (settings.difficulty + 3 + change) % 3;

134      }

135      settingsSave();

136      soundBeep(2000, 40);

137      Serial.print(F("[VR] option "));

138      Serial.print((const __FlashStringHelper*)OPTION_NAMES[optionItem]);

139      Serial.print(' ');

140      Serial.println(optionItem == OPT_LITE ? BRIGHTNESS_LEVELS[settings.brightness]

141                     : optionItem == OPT_SOUND ? !settings.muted : settings.difficulty);

142    }

143    if (keyPressed(KEY_BACK)) return SCR_MENU;

144

145    clearScreen();

146    clearStatusBars();

147    drawTextCentered(0, OPTION_NAMES[optionItem], false);

148    switch (optionItem) {

149      case OPT_LITE:

150        drawNumberCentered(9, BRIGHTNESS_LEVELS[settings.brightness], false);

151        setStatusBar(1, 16 * BRIGHTNESS_LEVELS[settings.brightness] / 100);   // a brightness gauge

152        break;

153      case OPT_SOUND:

154        drawTextCentered(9, settings.muted ? PSTR("OFF") : PSTR("ON"), false);

155        break;

156      case OPT_DIFF:

157        drawTextCentered(9, DIFF_NAMES[settings.difficulty], false);

158        setStatusBar(0, (settings.difficulty + 1) * 5);

159        break;

160      case OPT_TEST:

161        if (frameNo & 16) drawTextCentered(9, PSTR("R"), false);

162        break;

163    }

164    if (optionItem != OPT_TEST) drawSideArrows(15, frameNo);   // under the value

165    drawPageDots(OPTION_COUNT, optionItem);

166    return SCR_OPTIONS;

167  }

168

169  // ================================================================= best score

170  // One picture of the BEST page: the high score, then (bottom line, changing every 1.2 s)

171  // how far it got - "ZONE" and 1-3, or "WIN" - and the difficulty it was made on.

172  Screen bestFrame(uint16_t frameNo) {

173    if (keyPressed(KEY_BACK) || keyPressed(KEY_FIRE)) return SCR_MENU;

174    clearScreen();

175    clearStatusBars();

176    drawTextCentered(0, PSTR("BEST"), false);

177    if (settings.best) drawNumberCentered(9, settings.best, false);

178    else drawTextCentered(9, PSTR("---"), false);

179    if (settings.bestPhase) {

180      bool won = settings.bestPhase >= 4;

181      uint8_t page = (frameNo / 60) % (won ? 2 : 3);   // WIN, NORM  or  ZONE, 2, NORM

182      if (page == (won ? 1 : 2)) drawTextCentered(18, DIFF_NAMES[settings.bestDifficulty], false);

183      else if (won) drawTextCentered(18, PSTR("WIN"), false);

184      else if (page == 0) drawTextCentered(18, PSTR("ZONE"), false);

185      else drawNumberCentered(18, settings.bestPhase, false);

186    }

187    if (settings.best && (frameNo & 8)) {  // a little sparkle

188      setPixel(1, 7, true);

189      setPixel(14, 15, true);

190    } else if (settings.best) {

191      setPixel(14, 7, true);

192      setPixel(1, 15, true);

193    }

194    return SCR_BEST;

195  }

196

197  // ================================================================= pause

198  // Draws the pause screen: the score, a big pause sign, and the keys as a page that

199  // changes every 1.2 s ("P:GO" = P continues, "Q:EX" = Q exits), with the game's bars.

200  void pauseDraw(uint32_t t, uint16_t frameNo) {

201    clearScreen();

202    drawNumberCentered(0, player.score, false);

203    fillRect(4, 7, 3, 9, true);            // the pause sign: two bars

204    fillRect(9, 7, 3, 9, true);

205    drawTextCentered(18, (t / 1200) % 2 ? PSTR("Q:EX") : PSTR("P:GO"), false);

206    drawGameStatus(frameNo);

207  }

208

209  // ================================================================= game over and victory

210  bool newBest = false;

211

212  // Saves the score as the new best (with the phase reached, 4 = won, and the difficulty)

213  // if it beats the old one. The score counts first; the same score on a harder difficulty

214  // also takes the record.

215  void recordScore(uint8_t phaseReached) {

216    newBest = player.score > settings.best ||

217              (player.score == settings.best && player.score > 0 &&

218               settings.difficulty > settings.bestDifficulty);

219    if (!newBest) return;

220    settings.best = player.score;

221    settings.bestPhase = phaseReached;

222    settings.bestDifficulty = settings.difficulty;

223    settingsSave();

224    Serial.println(F("[VR] new best"));

225  }

226

227  // One picture of GAME OVER: the score (blinking if it is a record) and the best score;

228  // after 1 s, R or Q goes to the menu.

229  Screen gameOverFrame(uint32_t t, uint16_t frameNo) {

230    if (t > 1000 && (keyPressed(KEY_FIRE) || keyPressed(KEY_BACK))) return SCR_MENU;

231    clearScreen();

232    clearStatusBars();

233    drawTextCentered(0, PSTR("GAME"), false);

234    drawTextCentered(6, PSTR("OVER"), false);

235    if (!newBest || (frameNo & 8)) drawNumberCentered(12, player.score, false);   // blinks: record

236    if ((t / 1500) % 2 == 0) drawTextCentered(18, newBest ? PSTR("NEW") : PSTR("BEST"), false);

237    else drawNumberCentered(18, settings.best, false);

238    return SCR_GAMEOVER;

239  }

240

241  uint32_t nextFirework = 0;

242

243  // One picture of the victory screen: fireworks, pages "YOU WIN!", "VOID SAFE" and the

244  // score, and racing status bars; after 2 s, R or Q goes to the menu.

245  Screen victoryFrame(uint32_t t) {

246    if (t > 2000 && (keyPressed(KEY_FIRE) || keyPressed(KEY_BACK))) return SCR_MENU;

247    if (t >= nextFirework) {               // fireworks in the red and green zones

248      bool top = random(2);

249      explode(2 + random(SCREEN_W - 4), top ? 1 + random(4) : 18 + random(4), 10, 12);

250      if (t > 2600) soundSfx(SFX(SFX_BOOM), 3);   // after the fanfare

251      nextFirework = t + 350 + random(400);

252    }

253    particlesStep();

254    clearScreen();

255    for (uint8_t i = 0; i < MAX_PARTICLES; i++)

256      if (particles[i].life) setPixel(particles[i].x16 >> 4, particles[i].y16 >> 4, true);

257    uint8_t page = (t / 1200) % 3;

258    if (page == 0) {

259      drawPage(PSTR("YOU"), PSTR("WIN!"));

260    } else if (page == 1) {

261      drawPage(PSTR("VOID"), PSTR("SAFE"));

262    } else {

263      clearRows(6, 12);

264      drawNumberCentered(7, player.score, false);

265      drawTextCentered(13, PSTR("PTS"), false);

266    }

267    for (uint8_t bar = 0; bar < STATUS_ROWS; bar++) {   // status bars: a fast chase

268      uint8_t head = (t / 50 + bar * 4) % 16;

269      uint16_t bits = 0;

270      for (uint8_t k = 0; k < 3; k++) bits |= 1u << ((head + k) % 16);

271      setStatusBits(bar, bits);

272    }

273    return SCR_VICTORY;

274  }

275

276  // ================================================================= LED test

277  // Lights every LED, then one row at a time (status bars first), one column at a time,

278  // and a checkerboard -- a quick way to see that every LED and wire works.

279  const uint16_t TEST_ALL_MS = 1500, TEST_STEP_MS = 90;

280

281  // Sets one matrix row, counted from the top as built (status bars first, then the game

282  // screen).

283  void ledTestSetRow(uint8_t fromTop, uint16_t bits) {

284    frame[fromTop < STATUS_ROWS ? SCREEN_H + fromTop : fromTop - STATUS_ROWS] = bits;

285  }

286

287  // One picture of the LED test: all on, then each row, each column and a checkerboard,

288  // with a rising beep per step. Any key (or the end) goes back to the options.

289  Screen ledTestFrame(uint32_t t) {

290    static uint8_t lastStep = 255;

291    uint16_t skip = keysPressed & ~(keyBit(KEY_BRIGHTNESS) | keyBit(KEY_MUTE));

292    const uint32_t rowsEnd = TEST_ALL_MS + (uint32_t)TOTAL_ROWS * TEST_STEP_MS;

293    const uint32_t colsEnd = rowsEnd + (uint32_t)SCREEN_W * TEST_STEP_MS;

294    const uint32_t checkEnd = colsEnd + 2000;

295    if (skip || t >= checkEnd) {

296      lastStep = 255;

297      soundStop();

298      return SCR_OPTIONS;

299    }

300    uint8_t step = 0;

301    for (uint8_t r = 0; r < TOTAL_ROWS; r++) {

302      uint16_t bits = 0xFFFF;                        // all on

303      if (t >= colsEnd) {                            // checkerboard, swapping every 250 ms

304        bits = ((r + t / 250) & 1) ? 0xAAAA : 0x5555;

305      } else if (t >= rowsEnd) {                     // column sweep

306        step = (t - rowsEnd) / TEST_STEP_MS;

307        bits = 1u << step;

308      } else if (t >= TEST_ALL_MS) {                 // row sweep

309        step = (t - TEST_ALL_MS) / TEST_STEP_MS;

310        bits = (r == step) ? 0xFFFF : 0;

311      }

312      ledTestSetRow(r, bits);

313    }

314    if (t >= TEST_ALL_MS && t < colsEnd && step != lastStep) {   // a rising tick per step

315      lastStep = step;

316      soundBeep(t < rowsEnd ? 300 + step * 40 : 600 + step * 60, 30);

317    }

318    return SCR_LEDTEST;

319  }

## 4.22 Development tools and tests

These programs are not part of the Arduino sketch. They generate parts of it (the circuit, the pin list, the font, the music), make the pictures, and test the game on the PC.

| File | Job |
| --- | --- |
| tools/gen_circuit.py | Circuit generator: Generates `diagram.json` (432 LEDs, 9 buttons, 4 buzzers and every wire) and `pins.h`, routes the wires so they never cross, and checks every connection. |
| tools/gen_font.py | Font generator: Turns the ASCII-art letters into `font.h`. |
| tools/compose_music.py | Music composer: The in-game songs in a short note notation; writes their note lists into `tunes.h` and can render them as WAV files to listen to. |
| tools/render_dumps.py | Screen renderer: Draws screen dumps (the `!d` command) as PNG pictures or GIF animations in the real LED colours. |
| tools/draw_wiring.py | Wiring diagram: Draws the simplified wiring diagram. |
| tools/render_circuit.py | Circuit photo: Draws the whole circuit with Wokwi's own part pictures, lit with a game picture. |
| tools/make_doc.py | This document: Builds this document from the code and the game itself. |
| tests/host/Arduino.h | PC stand-in for Arduino: Registers as variables, a simulated clock and keypad, so the real game code runs on a PC. |
| tests/host/sim.cpp | PC test runner: Runs the game code from test scripts (keys, Serial, dumps). |
| tests/host/run_tests.py | Test runner: Builds the PC version and runs all tests and the balance check. |
| tests/host/controls.txt | Test: controls: The key rules. |
| tests/host/screens.txt | Test: screens: Every screen and the EEPROM. |
| tests/host/playthrough.txt | Test: whole game: All phases to the victory. |

### tools/gen_circuit.py – Circuit generator

Generates diagram.json (432 LEDs, 9 buttons, 4 buzzers and every wire) and pins.h, routes the wires so they never cross, and checks every connection.

1  #!/usr/bin/env python3

2  """

3  VOID RUNNER - circuit generator.

4

5  Builds the whole Wokwi circuit (diagram.json) from the tables below and writes the matching

6  pin table for the sketch (pins.h), so the circuit and the code can never disagree.

7

8    * 16 x 24 game screen + 3 status bars of single LEDs, wired as one row/column matrix

9    * 3 x 3 push-button keypad: exactly the 9 keys the game uses (every button also answers

10      to a PC keyboard key)

11    * 4 buzzers

12

13  Layout: LED panel on the left, the Mega (turned 180 degrees) on the right with its

14  double header facing the panel rows and its analog header facing up for the panel

15  columns, keypad and buzzers under the Mega. The wires inside the LED panel are hidden,

16  like a real LED panel; every wire from the panel, keypad and buzzers to the Mega is shown.

17

18  To change colours, size or key labels, edit the tables below and run:

19

20      python tools/gen_circuit.py <project_dir> <sketch_dir>

21

22  Writes <project_dir>/diagram.json, <sketch_dir>/pins.h and <project_dir>/circuit_preview.png,

23  then checks that every LED, button and buzzer reaches the right Mega pin and that no two

24  visible wires cross.

25  """

26  import json

27  import struct

28  import sys

29  import zlib

30  from pathlib import Path

31

32  # ============================================================== design tables (edit these)

33

34  SCREEN_W, SCREEN_H, STATUS_ROWS = 16, 24, 3

35

36  # Colour bands of the game screen, top to bottom: (first row, last row, light, body).

37  # The LED body is kept dark so an LED that is off almost disappears; the light colour is

38  # what you see when it is on (like the colour strips on the 1978 Space Invaders cabinet).

39  GAME_ZONES = [

40      (0, 5, '#ff3a3a', '#3a1818'),       # enemies and bosses enter here

41      (6, 17, '#ffffff', '#2c2c2c'),      # open space

42      (18, 23, '#3aff4a', '#183a1c'),     # your ship's zone

43  ]

44  # One (light, body) pair per status bar, top to bottom: boss health, ammo, lives.

45  STATUS_COLOURS = [('#ff3a3a', '#3a1818'), ('#ffd23a', '#3a3214'), ('#3aff4a', '#183a1c')]

46

47  # Brightness of a lit LED in Wokwi. The screen is multiplexed: each LED is on only 1/27 of

48  # the time, and Wokwi shows it with brightness = (on-time share) ^ (1 / gamma), measured on

49  # wokwi.com: default gamma 2.8 -> 32 % (too dim on the grey canvas), 30 -> 90 %. A real LED

50  # display gets the same effect by driving each LED harder during its short time slot.

51  LED_GAMMA = '30'

52  # How often Wokwi updates an LED's brightness. It averages over 8 updates, so the default

53  # 80 gives a 100 ms window and anything moving smears over several pixels; 300 gives

54  # 27 ms (about two full screen scans), which keeps moving things sharp.

55  LED_FPS = '300'

56

57  # Keypad, row by row: (label, PC keyboard key, button colour).

58  # Q back, R fire / OK; W A S D move (W above S, like a keyboard); P pause, M sound, B light.

59  BUTTONS = [

60      [('Q', 'q', 'white'), ('W', 'w', 'blue'), ('R', 'r', 'red')],

61      [('A', 'a', 'blue'), ('S', 's', 'blue'), ('D', 'd', 'blue')],

62      [('P', 'p', 'yellow'), ('M', 'm', 'yellow'), ('B', 'b', 'yellow')],

63  ]

64  KR, KC = len(BUTTONS), len(BUTTONS[0])               # keypad rows and columns

65

66  # Visible wire colours by job (wires inside the LED panel are hidden).

67  WIRE = {'row_bus': 'orange', 'col_bus': 'blue', 'key_row': 'purple', 'key_col': 'green',

68          'buzzer': 'violet', 'gnd': 'black'}

69

70  # ============================================================== part geometry (wokwi-elements)

71

72  MM = 96 / 25.4                                     # px per mm

73  LED_PINS = {'A': (25.0, 42.0), 'C': (15.0, 42.0)}  # wokwi-led is 40 x 50 px

74  BTN_PINS = {'1.l': (0.0, 13.0), '2.l': (0.0, 32.0), '1.r': (67.0, 13.0), '2.r': (67.0, 32.0)}

75  BUZ_W, BUZ_H = 17.0 * MM, 20.0 * MM                # wokwi-buzzer 17 x 20 mm, legs below

76  BUZ_PINS = {'1': (27.0, 84.0), '2': (37.0, 84.0)}

77  MEGA_W, MEGA_H = 102.66 * MM, 50.80 * MM

78  MEGA_PINS = {

79      'GND.1': (119, 9), '13': (129, 9), '12': (138, 9), '11': (148, 9), '10': (157.5, 9),

80      '9': (167.5, 9), '8': (177, 9), '7': (190, 9), '6': (200, 9), '5': (209.5, 9), '4': (219, 9),

81      '3': (228.5, 9), '2': (238, 9), '1': (247.5, 9), '0': (257.5, 9), '14': (270.5, 9),

82      '15': (280, 9), '16': (289.5, 9), '17': (299, 9), '18': (308.5, 9), '19': (318.5, 9),

83      '20': (328, 9), '21': (337.5, 9),

84  }

85  for _i, _x in enumerate([208.5, 218, 227.5, 237.25, 246.75, 256.25, 266, 275.5,

86                           290.25, 300, 309.5, 319.25, 328.75, 338.5, 348, 357.75]):

87      MEGA_PINS['A%d' % _i] = (_x, 184.5)

88  for _k, _y in enumerate([17.5, 27.25, 36.75, 46.25, 56, 65.5, 75, 84.5,

89                           94.25, 103.75, 113.5, 123, 132.75, 142.25, 152, 161.5]):

90      MEGA_PINS[str(22 + 2 * _k)] = (361, _y)       # double header: even pins inner column

91      MEGA_PINS[str(23 + 2 * _k)] = (371, _y)       #                odd pins outer column

92

93  # ============================================================== layout

94

95  TOTAL_ROWS = SCREEN_H + STATUS_ROWS

96  PX, PY = 44, 46                  # LED pitch: about square, so shapes are not stretched

97  SX, SY = 160, 240                # top-left LED of the panel (first status bar)

98  STATUS_GAP = 24                  # space between the status bars and the game screen

99  LANE = 6                         # spacing of parallel wires

100

101  # physical order of the matrix rows, top to bottom: status bars first, then the game screen

102  PHYS_ROWS = list(range(SCREEN_H, TOTAL_ROWS)) + list(range(SCREEN_H))

103

104  def led_xy(x, row):

105      """Top-left of the LED in column x of matrix row `row` (rows >= SCREEN_H are status bars)."""

106      if row >= SCREEN_H:

107          return SX + PX * x, SY + PY * (row - SCREEN_H)

108      return SX + PX * x, SY + PY * STATUS_ROWS + STATUS_GAP + PY * row

109

110  def led_id(x, row):

111      return 'led_%d_%d' % (x, row) if row < SCREEN_H else 'st_%d_%d' % (x, row - SCREEN_H)

112

113  def led_colours(row):

114      if row >= SCREEN_H:

115          return STATUS_COLOURS[row - SCREEN_H]

116      return next((light, body) for lo, hi, light, body in GAME_ZONES if lo <= row <= hi)

117

118  PANEL_RIGHT = SX + PX * (SCREEN_W - 1) + 40

119  MEGA_ROT = 180

120  MX = PANEL_RIGHT + 40 + LANE * TOTAL_ROWS + 30   # room for one lane per row wire

121  MY = SY - 10                                     # analog header level with the panel top

122

123  def rot(px, py, w, h, deg):

124      return (w - px, h - py) if deg == 180 else (px, py)

125

126  def mega_pin(name):

127      x, y = rot(*MEGA_PINS[name], MEGA_W, MEGA_H, MEGA_ROT)

128      return MX + x, MY + y

129

130  # ============================================================== pin assignment

131

132  # Rows: the double header, read top to bottom on the turned board. At each height the even

133  # pin sits further in, so its wire arrives 5 px above the odd pin's wire.

134  # Pin 46 is left out: it is OC5A, the output of Timer5, used by the fourth buzzer.

135  _row_pins = []

136  for k in range(15, -1, -1):

137      _row_pins += [str(22 + 2 * k), str(23 + 2 * k)]

138  _used = sorted([p for p in _row_pins if p != '46'], key=int)[:TOTAL_ROWS]

139  _row_pins = [p for p in _row_pins if p in _used]

140  ROW_PIN_OF_PHYS = _row_pins                          # physical row i (top to bottom) -> pin

141  ROW_PINS = [None] * TOTAL_ROWS

142  for i, row in enumerate(PHYS_ROWS):

143      ROW_PINS[row] = ROW_PIN_OF_PHYS[i]

144  COL_PINS = ['A%d' % x for x in range(SCREEN_W)]      # column x -> Ax (PORTF / PORTK bit order)

145  KEY_ROW_PINS = ['18', '19', '20', '21'][:KR]

146  KEY_COL_PINS = ['17', '16', '15', '14'][:KC]

147  BUZZER_PINS = ['5', '6', '11', '46']                  # OC3A, OC4A, OC1A, OC5A

148  BUZZER_TIMERS = ['Timer3 OC3A', 'Timer4 OC4A', 'Timer1 OC1A', 'Timer5 OC5A']

149

150  # ============================================================== diagram building

151

152  parts, wires = [], []

153

154  def part(ptype, pid, left, top, attrs=None, rotate=0):

155      p = {'type': ptype, 'id': pid, 'top': round(top, 2), 'left': round(left, 2), 'attrs': attrs or {}}

156      if rotate:

157          p['rotate'] = rotate

158      parts.append(p)

159

160  def mv(axis, d):

161      return '%s%g' % (axis, round(d, 2))

162

163  def wire(a, b, colour, route, kind):

164      """kind: 'hidden' (inside the panel), 'link' (short helper wire) or 'bus' (to the Mega)."""

165      wires.append({'a': a, 'b': b, 'colour': colour, 'route': route, 'kind': kind})

166

167  part('wokwi-arduino-mega', 'mega', MX, MY, {}, MEGA_ROT)

168

169  # --- LEDs and the hidden wires inside the panel

170  for row in range(TOTAL_ROWS):

171      light, body = led_colours(row)

172      for x in range(SCREEN_W):

173          lx, ly = led_xy(x, row)

174          part('wokwi-led', led_id(x, row), lx, ly, {'color': body, 'lightColor': light, 'gamma': LED_GAMMA, 'fps': LED_FPS})

175  for row in range(TOTAL_ROWS):                         # anodes of one row

176      for x in range(SCREEN_W - 1):

177          wire('%s:A' % led_id(x, row), '%s:A' % led_id(x + 1, row), '', [], 'hidden')

178  for x in range(SCREEN_W):                             # cathodes of one column

179      for a, b in zip(PHYS_ROWS, PHYS_ROWS[1:]):

180          wire('%s:C' % led_id(x, a), '%s:C' % led_id(x, b), '', [], 'hidden')

181

182  # --- column bus: from the top of each column, nested arcs over the panel into A0-A15.

183  # The leftmost column takes the highest arc and the rightmost pin, so no two cross.

184  top_row = PHYS_ROWS[0]

185  for x in range(SCREEN_W):

186      lx, ly = led_xy(x, top_row)

187      cx, cy = lx + LED_PINS['C'][0], ly + LED_PINS['C'][1]

188      lane_x = lx + 6                                   # just left of the LED body

189      h = SY - 16 - LANE * (SCREEN_W - 1 - x)

190      px, py = mega_pin(COL_PINS[x])

191      wire('%s:C' % led_id(x, top_row), 'mega:%s' % COL_PINS[x], WIRE['col_bus'],

192           [mv('h', lane_x - cx), mv('v', h - cy), '*', mv('v', h - py)], 'bus')

193

194  # --- row bus: from the right end of each row to its own lane, then to the double header.

195  approach = []

196  for i, row in enumerate(PHYS_ROWS):

197      px, py = mega_pin(ROW_PIN_OF_PHYS[i])

198      approach.append(py - 5 if int(ROW_PIN_OF_PHYS[i]) % 2 == 0 else py)

199  src_y = [led_xy(SCREEN_W - 1, row)[1] + LED_PINS['A'][1] for row in PHYS_ROWS]

200  down = [i for i in range(TOTAL_ROWS) if approach[i] > src_y[i]]      # rows above their pin

201  up = [i for i in range(TOTAL_ROWS) if approach[i] <= src_y[i]]       # rows at or below it

202  lane_rank = {}

203  for k, i in enumerate(sorted(down, reverse=True)):   # descending: upper rows take outer lanes

204      lane_rank[i] = k

205  for k, i in enumerate(sorted(up)):                   # ascending: lower rows take outer lanes

206      lane_rank[i] = k

207  for i, row in enumerate(PHYS_ROWS):

208      lx, ly = led_xy(SCREEN_W - 1, row)

209      ax, ay = lx + LED_PINS['A'][0], ly + LED_PINS['A'][1]

210      lane_x = PANEL_RIGHT + 20 + LANE * lane_rank[i]

211      pin = ROW_PIN_OF_PHYS[i]

212      route = [mv('h', lane_x - ax)]

213      if abs(approach[i] - ay) > 0.01:

214          route.append(mv('v', approach[i] - ay))

215      if int(pin) % 2 == 0:

216          route += ['*', 'v-5']

217      wire('%s:A' % led_id(SCREEN_W - 1, row), 'mega:%s' % pin, WIRE['row_bus'], route, 'bus')

218

219  # --- keypad and buzzers under the Mega (wires come up into its lower header)

220  BELOW = MY + MEGA_H                    # lower edge of the turned board

221  KX, KPX, KPY = MX + 40, 90, 70

222  BX, BPX = KX + KC * KPX + 30, 85

223

224  def fan_heights(items, top, step):

225      """items: (lane_x, pin_x, key) of wires coming up from below. Returns a turn height per

226      wire so that no two wires of the fan cross (outer wires turn higher)."""

227      right = sorted([i for i in items if i[1] >= i[0]], key=lambda i: i[0])

228      left = sorted([i for i in items if i[1] < i[0]], key=lambda i: -i[0])

229      heights = {}

230      for group in (right, left):

231          for rank, item in enumerate(group):

232              heights[item[2]] = top + step * rank

233      return heights

234

235  buz_top = BELOW + 16

236  key_top = buz_top + LANE * 4 + 12

237  KY = key_top + LANE * (KR + KC) + 30

238  BY = KY + 10

239

240  for r in range(KR):

241      for c in range(KC):

242          label, key, colour = BUTTONS[r][c]

243          part('wokwi-pushbutton', 'btn_%s' % label, KX + KPX * c, KY + KPY * r,

244               {'color': colour, 'label': label, 'key': key})

245  for r in range(KR):                   # contact 1 of each button = its row line

246      for c in range(KC - 1):

247          wire('btn_%s:1.r' % BUTTONS[r][c][0], 'btn_%s:1.l' % BUTTONS[r][c + 1][0],

248               WIRE['key_row'], [], 'link')

249  for c in range(KC):                   # contact 2 of each button = its column line

250      for r in range(KR - 1):

251          wire('btn_%s:2.r' % BUTTONS[r][c][0], 'btn_%s:2.r' % BUTTONS[r + 1][c][0],

252               WIRE['key_col'], ['h11', '*', 'h11'], 'link')

253

254  key_items = []                         # (lane, pin_x, key, src, pin, (x, y), first move, colour)

255  for r in range(KR):

256      sx_, sy_ = KX + BTN_PINS['1.l'][0], KY + KPY * r + BTN_PINS['1.l'][1]

257      lane = sx_ - 12 - 8 * r            # lower rows take outer lanes

258      key_items.append((lane, mega_pin(KEY_ROW_PINS[r])[0], 'r%d' % r, 'btn_%s:1.l' % BUTTONS[r][0][0],

259                        KEY_ROW_PINS[r], (sx_, sy_), mv('h', lane - sx_), WIRE['key_row']))

260  for c in range(KC):

261      sx_, sy_ = KX + KPX * c + BTN_PINS['2.r'][0], KY + BTN_PINS['2.r'][1]

262      key_items.append((sx_ + 11, mega_pin(KEY_COL_PINS[c])[0], 'c%d' % c, 'btn_%s:2.r' % BUTTONS[0][c][0],

263                        KEY_COL_PINS[c], (sx_, sy_), 'h11', WIRE['key_col']))

264  turn = fan_heights([(i[0], i[1], i[2]) for i in key_items], key_top, LANE)

265  for lane, _, key, src, pin, (sx_, sy_), first, colour in key_items:

266      _, py = mega_pin(pin)

267      wire(src, 'mega:%s' % pin, colour, [first, mv('v', turn[key] - sy_), '*', mv('v', turn[key] - py)], 'bus')

268

269  buz_items = []

270  for i in range(len(BUZZER_PINS)):

271      bx = BX + BPX * i

272      on_side = int(BUZZER_PINS[i]) >= 22               # its wire goes up beside the board:

273      turned = 0 if on_side else 180                    # not turned, so its signal leg is on the right

274      part('wokwi-buzzer', 'bz%d' % (i + 1), bx, BY, {'volume': '0.6'}, turned)

275      p2 = rot(*BUZ_PINS['2'], BUZ_W, BUZ_H, turned)

276      buz_items.append((bx + p2[0], mega_pin(BUZZER_PINS[i])[0], 'b%d' % i, 'bz%d:2' % (i + 1),

277                        BUZZER_PINS[i], (bx + p2[0], BY + p2[1]), WIRE['buzzer']))

278      if i < len(BUZZER_PINS) - 1:       # ground from buzzer to buzzer (hidden)

279          wire('bz%d:1' % (i + 1), 'bz%d:1' % (i + 2), '', [], 'hidden')

280  p1 = rot(*BUZ_PINS['1'], BUZ_W, BUZ_H, turned)

281  last = len(BUZZER_PINS) - 1

282  buz_items.append((BX + last * BPX + p1[0], mega_pin('GND.1')[0], 'gnd', 'bz%d:1' % (last + 1), 'GND.1',

283                    (BX + last * BPX + p1[0], BY + p1[1]), WIRE['gnd']))

284  # A buzzer on the double header (pin 46, the rightmost buzzer) goes straight up beside the

285  # board and across it at its pin's height; the others fan into the lower header.

286  side = [i for i in buz_items if i[4].isdigit() and int(i[4]) >= 22]

287  fan = [i for i in buz_items if i not in side]

288  turn = fan_heights([(i[0], i[1], i[2]) for i in fan], buz_top, LANE)

289  for lane, _, key, src, pin, (sx_, sy_), colour in fan:

290      _, py = mega_pin(pin)

291      wire(src, 'mega:%s' % pin, colour, [mv('v', turn[key] - sy_), '*', mv('v', turn[key] - py)], 'bus')

292  for lane, _, key, src, pin, (sx_, sy_), colour in side:

293      _, py = mega_pin(pin)

294      wire(src, 'mega:%s' % pin, colour, [mv('v', py - sy_)], 'bus')

295

296  # ============================================================== geometry helpers for checks

297

298  PIN_TABLE = {}

299  for p in parts:

300      if p['type'] == 'wokwi-led':

301          for n, (x, y) in LED_PINS.items():

302              PIN_TABLE['%s:%s' % (p['id'], n)] = (p['left'] + x, p['top'] + y)

303      elif p['type'] == 'wokwi-pushbutton':

304          for n, (x, y) in BTN_PINS.items():

305              PIN_TABLE['%s:%s' % (p['id'], n)] = (p['left'] + x, p['top'] + y)

306      elif p['type'] == 'wokwi-buzzer':

307          for n, (x, y) in BUZ_PINS.items():

308              rx, ry = rot(x, y, BUZ_W, BUZ_H, p.get('rotate', 0))

309              PIN_TABLE['%s:%s' % (p['id'], n)] = (p['left'] + rx, p['top'] + ry)

310  for n in MEGA_PINS:

311      PIN_TABLE['mega:%s' % n] = mega_pin(n)

312

313  def apply(pt, moves):

314      pts = [pt]

315      x, y = pt

316      for m in moves:

317          d = float(m[1:])

318          x, y = (x + d, y) if m[0] == 'h' else (x, y + d)

319          pts.append((x, y))

320      return pts

321

322  def polyline(w):

323      route = w['route']

324      star = route.index('*') if '*' in route else len(route)

325      src = apply(PIN_TABLE[w['a']], route[:star])

326      dst = apply(PIN_TABLE[w['b']], list(reversed(route[star + 1:])))

327      (x1, y1), (x2, y2) = src[-1], dst[-1]

328      mid = [] if (abs(x1 - x2) < 0.01 or abs(y1 - y2) < 0.01) else [(x2, y1)]

329      return src + mid + list(reversed(dst))

330

331  # ============================================================== checks

332

333  def check_nets():

334      parent = {}

335      def find(a):

336          parent.setdefault(a, a)

337          while parent[a] != a:

338              parent[a] = parent[parent[a]]

339              a = parent[a]

340          return a

341      for w in wires:

342          parent[find(w['a'])] = find(w['b'])

343      for p in parts:                     # a push button's left and right pins are one contact

344          if p['type'] == 'wokwi-pushbutton':

345              parent[find(p['id'] + ':1.l')] = find(p['id'] + ':1.r')

346              parent[find(p['id'] + ':2.l')] = find(p['id'] + ':2.r')

347      nets = {}

348      for pin in list(parent):

349          nets.setdefault(find(pin), set()).add(pin)

350      mega_of = {}

351      for members in nets.values():

352          megas = sorted(m for m in members if m.startswith('mega:'))

353          assert len(megas) <= 1, 'short between Mega pins: %s' % megas

354          for m in members:

355              mega_of[m] = megas[0][5:] if megas else None

356      errors = 0

357      for row in range(TOTAL_ROWS):

358          for x in range(SCREEN_W):

359              pid = led_id(x, row)

360              if mega_of.get(pid + ':A') != ROW_PINS[row] or mega_of.get(pid + ':C') != COL_PINS[x]:

361                  errors += 1

362      for r in range(KR):

363          for c in range(KC):

364              pid = 'btn_%s' % BUTTONS[r][c][0]

365              if mega_of.get(pid + ':1.l') != KEY_ROW_PINS[r] or mega_of.get(pid + ':2.l') != KEY_COL_PINS[c]:

366                  errors += 1

367      for i in range(len(BUZZER_PINS)):

368          if mega_of.get('bz%d:2' % (i + 1)) != BUZZER_PINS[i] or mega_of.get('bz%d:1' % (i + 1)) != 'GND.1':

369              errors += 1

370      return errors

371

372  def count_crossings():

373      segs = []

374      for wi, w in enumerate(wires):

375          if w['kind'] == 'hidden':

376              continue

377          pts = polyline(w)

378          for a, b in zip(pts, pts[1:]):

379              segs.append((wi, a, b))

380      def cross(s, t):

381          (ax1, ay1), (ax2, ay2) = s

382          (bx1, by1), (bx2, by2) = t

383          a_h, b_h = abs(ay1 - ay2) < 0.01, abs(by1 - by2) < 0.01

384          if a_h == b_h:

385              return False

386          h, v = (s, t) if a_h else (t, s)

387          (hx1, hy), (hx2, _) = h

388          (vx, vy1), (_, vy2) = v

389          return (min(hx1, hx2) < vx < max(hx1, hx2)) and (min(vy1, vy2) < hy < max(vy1, vy2))

390      found = []

391      for i in range(len(segs)):

392          for j in range(i + 1, len(segs)):

393              a, b = wires[segs[i][0]], wires[segs[j][0]]

394              if a is not b and a['kind'] == 'bus' and cross(segs[i][1:], segs[j][1:]):

395                  found.append((a['b'], b['b']))

396      return found

397

398  # ============================================================== outputs

399

400  def write_diagram(path):

401      doc = {'version': 1, 'author': 'VOID RUNNER team', 'editor': 'wokwi', 'parts': parts,

402             'connections': [[w['a'], w['b'], w['colour'], w['route']] for w in wires],

403             'dependencies': {}}

404      path.write_text(json.dumps(doc, indent=1), encoding='utf-8', newline='\n')

405

406  def write_pins_h(path):

407      def arr(items):

408          return ', '.join(str(i) for i in items)

409      keys = ',\n  '.join('{%s}' % ', '.join("'%s'" % b[0] for b in row) for row in BUTTONS)

410      text = f"""/*

411   * pins.h -- GENERATED by tools/gen_circuit.py together with diagram.json.

412   * Do not edit by hand: change the generator and run it again, so the circuit

413   * and the code always match.

414   */

415  #pragma once

416  #include <Arduino.h>

417

418  // ---- LED screen: {SCREEN_W} columns x {SCREEN_H} game rows + {STATUS_ROWS} status bars, one big matrix

419  const uint8_t SCREEN_W    = {SCREEN_W};

420  const uint8_t SCREEN_H    = {SCREEN_H};

421  const uint8_t STATUS_ROWS = {STATUS_ROWS};

422  const uint8_t TOTAL_ROWS  = SCREEN_H + STATUS_ROWS;

423

424  // Anode pin of every LED row: rows 0-{SCREEN_H - 1} = game screen top to bottom,

425  // rows {SCREEN_H}-{TOTAL_ROWS - 1} = status bars top to bottom (all on the double header).

426  const uint8_t ROW_PINS[TOTAL_ROWS] = {{

427    {arr(ROW_PINS)}

428  }};

429

430  // Cathode pin of every LED column, left to right: column x is wired to Ax,

431  // so columns 0-7 are PORTF bits 0-7 and columns 8-15 are PORTK bits 0-7.

432  const uint8_t COL_PINS[SCREEN_W] = {{

433    {arr(COL_PINS)}

434  }};

435

436  // ---- {KR} x {KC} push-button keypad: contact 1 = row line, contact 2 = column line

437  const uint8_t KEY_ROWS = {KR}, KEY_COLS = {KC};

438  const uint8_t KEY_ROW_PINS[KEY_ROWS] = {{ {arr(KEY_ROW_PINS)} }};

439  const uint8_t KEY_COL_PINS[KEY_COLS] = {{ {arr(KEY_COL_PINS)} }};

440  const char KEY_NAMES[KEY_ROWS][KEY_COLS] = {{

441    {keys}

442  }};

443

444  // ---- buzzers, each on the "A" output of its own 16-bit timer

445  const uint8_t BUZZER_COUNT = {len(BUZZER_PINS)};

446  const uint8_t BUZZER_PINS[BUZZER_COUNT] = {{ {arr(BUZZER_PINS)} }};   // {', '.join(BUZZER_TIMERS)}

447  """

448      path.write_text(text, encoding='utf-8', newline='\n')

449

450  def write_preview(path, scale=0.5):

451      xs, ys = zip(*(list(PIN_TABLE.values()) + [pt for w in wires for pt in polyline(w)]))

452      x0, y0 = min(xs) - 60, min(ys) - 60

453      W, H = int((max(xs) - x0 + 100) * scale), int((max(ys) - y0 + 100) * scale)

454      img = bytearray(b'\x30\x30\x30' * W * H)            # dark canvas, like Wokwi's dark theme

455      def put(x, y, c):

456          if 0 <= x < W and 0 <= y < H:

457              i = (y * W + x) * 3

458              img[i:i + 3] = bytes(c)

459      def line(p, q, c):

460          (x1, y1), (x2, y2) = [(int((a - x0) * scale), int((b - y0) * scale)) for a, b in (p, q)]

461          n = max(abs(x2 - x1), abs(y2 - y1), 1)

462          for k in range(n + 1):

463              put(x1 + (x2 - x1) * k // n, y1 + (y2 - y1) * k // n, c)

464      def rect(x, y, w, h, c):

465          for yy in range(int((y - y0) * scale), int((y + h - y0) * scale)):

466              for xx in range(int((x - x0) * scale), int((x + w - x0) * scale)):

467                  put(xx, yy, c)

468      def hexrgb(s, default=(120, 120, 120)):

469          named = {'blue': (40, 90, 230), 'black': (10, 10, 10), 'orange': (240, 140, 0), 'purple': (150, 60, 200),

470                   'violet': (200, 90, 220), 'green': (40, 190, 60), 'red': (230, 40, 40), 'white': (230, 230, 230),

471                   'yellow': (230, 200, 0)}

472          if s in named:

473              return named[s]

474          return tuple(int(s[i:i + 2], 16) for i in (1, 3, 5)) if s.startswith('#') else default

475      for p in parts:

476          if p['type'] == 'wokwi-led':                     # draw every LED "on" in its light colour

477              rect(p['left'] + 12, p['top'] + 4, 16, 22, hexrgb(p['attrs']['lightColor']))

478          elif p['type'] == 'wokwi-pushbutton':

479              rect(p['left'] + 12, p['top'], 45, 45, (70, 70, 70))

480              rect(p['left'] + 22, p['top'] + 10, 25, 25, hexrgb(p['attrs']['color']))

481          elif p['type'] == 'wokwi-buzzer':

482              rect(p['left'], p['top'], BUZ_W, BUZ_H, (10, 10, 10))

483          elif p['type'] == 'wokwi-arduino-mega':

484              rect(p['left'], p['top'], MEGA_W, MEGA_H, (0, 110, 150))

485      for w in wires:

486          if w['colour']:

487              pts = polyline(w)

488              for a, b in zip(pts, pts[1:]):

489                  line(a, b, hexrgb(w['colour']))

490      raw = b''.join(b'\x00' + bytes(img[y * W * 3:(y + 1) * W * 3]) for y in range(H))

491      def chunk(tag, data):

492          return struct.pack('>I', len(data)) + tag + data + struct.pack('>I', zlib.crc32(tag + data) & 0xFFFFFFFF)

493      png = b'\x89PNG\r\n\x1a\n' + chunk(b'IHDR', struct.pack('>IIBBBBB', W, H, 8, 2, 0, 0, 0))

494      path.write_bytes(png + chunk(b'IDAT', zlib.compress(bytes(raw), 9)) + chunk(b'IEND', b''))

495

496  if __name__ == '__main__':

497      project, sketch = Path(sys.argv[1]), Path(sys.argv[2])

498      write_diagram(project / 'diagram.json')

499      write_pins_h(sketch / 'pins.h')

500      write_preview(project / 'circuit_preview.png')

501      leds = sum(p['type'] == 'wokwi-led' for p in parts)

502      xs, ys = zip(*PIN_TABLE.values())

503      print('parts: %d (%d LEDs), wires: %d (%d visible)' % (

504          len(parts), leds, len(wires), sum(1 for w in wires if w['colour'])))

505      print('circuit size: about %d x %d px' % (max(xs) - min(xs), max(ys) - min(ys)))

506      print('wiring errors: %d' % check_nets())

507      crossings = count_crossings()

508      print('visible wire crossings: %d' % len(crossings))

509      for a, b in crossings[:20]:

510          print('   ', a, 'x', b)

### tools/gen_font.py – Font generator

Turns the ASCII-art letters into font.h.

1  #!/usr/bin/env python3

2  """

3  VOID RUNNER - font generator.

4

5  The two pixel fonts are drawn below as ASCII art ('#' = LED on). This script turns them

6  into font.h for the sketch, so the letters stay easy to read and edit here.

7

8    FONT3x5 - small font, 4 pixels per letter: four letters fit across the 16-LED screen

9    FONT5x7 - big font, 6 pixels per letter, used for scrolling banners

10

11  Each glyph is stored column by column, one byte per column, bit 0 = top row.

12  Usage: python tools/gen_font.py <sketch_dir>

13  """

14  import sys

15  from pathlib import Path

16

17  SMALL = {

18      'A': ['.#.', '#.#', '###', '#.#', '#.#'], 'B': ['##.', '#.#', '##.', '#.#', '##.'],

19      'C': ['.##', '#..', '#..', '#..', '.##'], 'D': ['##.', '#.#', '#.#', '#.#', '##.'],

20      'E': ['###', '#..', '##.', '#..', '###'], 'F': ['###', '#..', '##.', '#..', '#..'],

21      'G': ['.##', '#..', '#.#', '#.#', '.##'], 'H': ['#.#', '#.#', '###', '#.#', '#.#'],

22      'I': ['###', '.#.', '.#.', '.#.', '###'], 'J': ['..#', '..#', '..#', '#.#', '.#.'],

23      'K': ['#.#', '#.#', '##.', '#.#', '#.#'], 'L': ['#..', '#..', '#..', '#..', '###'],

24      'M': ['#.#', '###', '###', '#.#', '#.#'], 'N': ['##.', '#.#', '#.#', '#.#', '#.#'],

25      'O': ['.#.', '#.#', '#.#', '#.#', '.#.'], 'P': ['##.', '#.#', '##.', '#..', '#..'],

26      'Q': ['.#.', '#.#', '#.#', '##.', '.##'], 'R': ['##.', '#.#', '##.', '#.#', '#.#'],

27      'S': ['.##', '#..', '.#.', '..#', '##.'], 'T': ['###', '.#.', '.#.', '.#.', '.#.'],

28      'U': ['#.#', '#.#', '#.#', '#.#', '###'], 'V': ['#.#', '#.#', '#.#', '#.#', '.#.'],

29      'W': ['#.#', '#.#', '###', '###', '#.#'], 'X': ['#.#', '#.#', '.#.', '#.#', '#.#'],

30      'Y': ['#.#', '#.#', '.#.', '.#.', '.#.'], 'Z': ['###', '..#', '.#.', '#..', '###'],

31      '0': ['###', '#.#', '#.#', '#.#', '###'], '1': ['.#.', '##.', '.#.', '.#.', '###'],

32      '2': ['###', '..#', '###', '#..', '###'], '3': ['###', '..#', '###', '..#', '###'],

33      '4': ['#.#', '#.#', '###', '..#', '..#'], '5': ['###', '#..', '###', '..#', '###'],

34      '6': ['###', '#..', '###', '#.#', '###'], '7': ['###', '..#', '.#.', '.#.', '.#.'],

35      '8': ['###', '#.#', '###', '#.#', '###'], '9': ['###', '#.#', '###', '..#', '###'],

36      ' ': ['...'] * 5, '-': ['...', '...', '###', '...', '...'], '!': ['.#.', '.#.', '.#.', '...', '.#.'],

37      ':': ['...', '.#.', '...', '.#.', '...'], '.': ['...', '...', '...', '...', '.#.'],

38      '>': ['#..', '##.', '###', '##.', '#..'],

39  }

40

41  BIG = {

42      ' ': ['.....'] * 7,

43      '!': ['..#..', '..#..', '..#..', '..#..', '..#..', '.....', '..#..'],

44      '-': ['.....', '.....', '.....', '#####', '.....', '.....', '.....'],

45      '.': ['.....', '.....', '.....', '.....', '.....', '.##..', '.##..'],

46      ':': ['.....', '.##..', '.##..', '.....', '.##..', '.##..', '.....'],

47      '0': ['.###.', '#...#', '#..##', '#.#.#', '##..#', '#...#', '.###.'],

48      '1': ['..#..', '.##..', '..#..', '..#..', '..#..', '..#..', '.###.'],

49      '2': ['.###.', '#...#', '....#', '...#.', '..#..', '.#...', '#####'],

50      '3': ['#####', '...#.', '..#..', '...#.', '....#', '#...#', '.###.'],

51      '4': ['...#.', '..##.', '.#.#.', '#..#.', '#####', '...#.', '...#.'],

52      '5': ['#####', '#....', '####.', '....#', '....#', '#...#', '.###.'],

53      '6': ['..##.', '.#...', '#....', '####.', '#...#', '#...#', '.###.'],

54      '7': ['#####', '....#', '...#.', '..#..', '.#...', '.#...', '.#...'],

55      '8': ['.###.', '#...#', '#...#', '.###.', '#...#', '#...#', '.###.'],

56      '9': ['.###.', '#...#', '#...#', '.####', '....#', '...#.', '.##..'],

57      'A': ['.###.', '#...#', '#...#', '#####', '#...#', '#...#', '#...#'],

58      'B': ['####.', '#...#', '#...#', '####.', '#...#', '#...#', '####.'],

59      'C': ['.###.', '#...#', '#....', '#....', '#....', '#...#', '.###.'],

60      'D': ['####.', '#...#', '#...#', '#...#', '#...#', '#...#', '####.'],

61      'E': ['#####', '#....', '#....', '####.', '#....', '#....', '#####'],

62      'F': ['#####', '#....', '#....', '####.', '#....', '#....', '#....'],

63      'G': ['.###.', '#...#', '#....', '#.###', '#...#', '#...#', '.####'],

64      'H': ['#...#', '#...#', '#...#', '#####', '#...#', '#...#', '#...#'],

65      'I': ['.###.', '..#..', '..#..', '..#..', '..#..', '..#..', '.###.'],

66      'J': ['..###', '...#.', '...#.', '...#.', '...#.', '#..#.', '.##..'],

67      'K': ['#...#', '#..#.', '#.#..', '##...', '#.#..', '#..#.', '#...#'],

68      'L': ['#....', '#....', '#....', '#....', '#....', '#....', '#####'],

69      'M': ['#...#', '##.##', '#.#.#', '#.#.#', '#...#', '#...#', '#...#'],

70      'N': ['#...#', '#...#', '##..#', '#.#.#', '#..##', '#...#', '#...#'],

71      'O': ['.###.', '#...#', '#...#', '#...#', '#...#', '#...#', '.###.'],

72      'P': ['####.', '#...#', '#...#', '####.', '#....', '#....', '#....'],

73      'Q': ['.###.', '#...#', '#...#', '#...#', '#.#.#', '#..#.', '.##.#'],

74      'R': ['####.', '#...#', '#...#', '####.', '#.#..', '#..#.', '#...#'],

75      'S': ['.####', '#....', '#....', '.###.', '....#', '....#', '####.'],

76      'T': ['#####', '..#..', '..#..', '..#..', '..#..', '..#..', '..#..'],

77      'U': ['#...#', '#...#', '#...#', '#...#', '#...#', '#...#', '.###.'],

78      'V': ['#...#', '#...#', '#...#', '#...#', '#...#', '.#.#.', '..#..'],

79      'W': ['#...#', '#...#', '#...#', '#.#.#', '#.#.#', '##.##', '#...#'],

80      'X': ['#...#', '#...#', '.#.#.', '..#..', '.#.#.', '#...#', '#...#'],

81      'Y': ['#...#', '#...#', '.#.#.', '..#..', '..#..', '..#..', '..#..'],

82      'Z': ['#####', '....#', '...#.', '..#..', '.#...', '#....', '#####'],

83  }

84

85  def columns(rows):

86      out = []

87      for c in range(len(rows[0])):

88          out.append(sum(1 << r for r in range(len(rows)) if rows[r][c] == '#'))

89      return out

90

91  def table(name, glyphs, w, h):

92      chars = ''.join(glyphs)

93      lines = ['// %d x %d font: glyph i is the i-th character of %s_CHARS' % (w, h, name),

94               'const char %s_CHARS[] PROGMEM = "%s";' % (name, chars.replace('\\', '\\\\').replace('"', '\\"')),

95               'const uint8_t %s[] PROGMEM = {' % name]

96      for ch, rows in glyphs.items():

97          assert len(rows) == h and all(len(r) == w for r in rows), ch

98          lines.append('  ' + ', '.join('0x%02X' % b for b in columns(rows)) + ",  // '%s'" % ch)

99      lines.append('};')

100      return '\n'.join(lines)

101

102  if __name__ == '__main__':

103      out = Path(sys.argv[1]) / 'font.h'

104      out.write_text('/*\n * font.h -- GENERATED by tools/gen_font.py (the letters are drawn there as ASCII art).\n'

105                     ' * Each glyph is stored column by column, one byte per column, bit 0 = top row.\n */\n'

106                     '#pragma once\n#include <avr/pgmspace.h>\n\n'

107                     + table('FONT3x5', SMALL, 3, 5) + '\n\n' + table('FONT5x7', BIG, 5, 7) + '\n',

108                     encoding='utf-8', newline='\n')

109      print('wrote', out, '-', len(SMALL), 'small and', len(BIG), 'big glyphs')

### tools/compose_music.py – Music composer

The in-game songs in a short note notation; writes their note lists into tunes.h and can render them as WAV files to listen to.

1  #!/usr/bin/env python3

2  """

3  VOID RUNNER - the in-game music (original metal tracks), written in a short notation and

4  turned into the note lists of void_runner/tunes.h (the block between the BEGIN/END

5  markers is replaced; the rest of tunes.h is not touched).

6

7  Usage: python tools/compose_music.py

8

9  Notation: 'E2:2' = note E2 for 2 ticks (1 tick = a 16th note), 'r:4' = rest. Bars are 16

10  ticks. The rhythm guitar (buzzer 4) plays a fifth above the bass in the same rhythm, so

11  bass + rhythm sound like a power chord.

12  """

13  import re

14  from pathlib import Path

15

16  NAMES = {'C': 0, 'C#': 1, 'Db': 1, 'D': 2, 'D#': 3, 'Eb': 3, 'E': 4, 'F': 5, 'F#': 6, 'Gb': 6,

17           'G': 7, 'G#': 8, 'Ab': 8, 'A': 9, 'A#': 10, 'Bb': 10, 'B': 11}

18

19

20  def midi(name):

21      m = re.fullmatch(r'([A-G][#b]?)(\d)', name)

22      return 12 * (int(m.group(2)) + 1) + NAMES[m.group(1)]

23

24

25  def parse(text):

26      notes = []

27      for tok in text.split():

28          n, t = tok.split(':')

29          notes.append((0 if n == 'r' else midi(n), int(t)))

30      return notes

31

32

33  def gallop(n, beats=4):  # 8th + two 16ths per beat

34      return ' '.join(['%s:2 %s:1 %s:1' % (n, n, n)] * beats)

35

36

37  def fifths(notes):

38      return [(p + 7 if p else 0, t) for p, t in notes]

39

40

41  def bars(notes):

42      total = sum(t for _, t in notes)

43      return total / 16

44

45

46  SONGS = {}

47

48  # ---------------------------------------------------------------- phase 1: ROCK BELT

49  # Thrash, E minor, galloping bass (8th + two 16ths), tick 80 ms (about 190 BPM).

50  bass = ' '.join([

51      gallop('E2'),

52      gallop('E2', 2), 'G2:2 G2:1 G2:1 A2:2 A2:1 A2:1',

53      gallop('E2'),

54      gallop('E2', 2), 'Bb2:4 A2:4',

55      gallop('E2'),

56      gallop('E2', 2), 'G2:2 G2:1 G2:1 A2:2 A2:1 A2:1',

57      gallop('C3', 2), gallop('B2', 2),

58      gallop('B2', 2), 'B2:8',

59  ])

60  lead = '''

61  E4:4 G4:2 A4:2 B4:4 A4:2 G4:2

62  A4:4 G4:2 E4:2 D4:4 E4:4

63  E4:4 G4:2 A4:2 B4:4 D5:2 E5:2

64  Bb4:4 A4:4 G4:4 F#4:4

65  E5:6 D5:2 B4:4 G4:4

66  A4:6 B4:2 G4:4 E4:4

67  E5:2 D5:2 B4:2 A4:2 G4:2 A4:2 B4:2 D5:2

68  F#4:8 B4:8

69  '''

70  SONGS['PHASE1'] = (80, parse(lead), parse(bass), 'Phase 1, ROCK BELT: thrash, E minor, galloping bass')

71

72  # ---------------------------------------------------------------- phase 2: DEEP MIST

73  # Slow and heavy, drop-D chugs (low D2), D minor lead with long notes, tick 100 ms.

74  chug_a = 'D2:1 D2:1 r:1 D2:1 D2:2 r:2 D2:1 D2:1 r:1 D2:1 F2:2 E2:2'

75  chug_b = 'D2:1 D2:1 r:1 D2:1 D2:2 r:2 A2:4 G#2:2 G2:2'

76  bass = ' '.join([chug_a, chug_b, chug_a, chug_b, chug_a, chug_b, chug_a, 'Bb2:8 A2:8'])

77  lead = '''

78  r:8 A4:4 F4:4

79  E4:8 D4:4 r:4

80  r:8 A4:4 C5:4

81  Bb4:8 A4:4 r:4

82  D5:6 C5:2 A4:4 F4:4

83  G4:6 A4:2 E4:8

84  F4:4 E4:4 D4:4 C#4:4

85  D4:12 r:4

86  '''

87  SONGS['PHASE2'] = (100, parse(lead), parse(bass), 'Phase 2, DEEP MIST: slow drop-D chugs, D minor')

88

89  # ---------------------------------------------------------------- phase 3: THE VOID

90  # Syncopated heavy groove with a catchy lead, E minor, tick 75 ms.

91  groove_x = 'E2:1 r:1 E2:1 r:1 E2:1 E2:1 r:2 G2:2 E2:2 Bb2:2 A2:2'

92  groove_y = 'E2:1 r:1 E2:1 r:1 E2:1 E2:1 r:2 C3:4 B2:4'

93  bass = ' '.join([groove_x, groove_y, groove_x, groove_y, groove_x, groove_y,

94                   ' '.join(['E2:2'] * 8), ' '.join(['E2:1'] * 16)])

95  lead = '''

96  B4:4 B4:2 G4:2 A4:4 E4:4

97  B4:4 B4:2 G4:2 C5:4 B4:4

98  B4:4 B4:2 G4:2 A4:4 E4:2 G4:2

99  F#4:4 G4:4 A4:4 B4:4

100  E5:4 D5:2 B4:2 D5:4 E5:4

101  G5:4 F#5:2 E5:2 D5:4 B4:4

102  E5:2 B4:2 G4:2 E4:2 E5:2 B4:2 G4:2 E4:2

103  E5:1 D5:1 B4:1 A4:1 G4:1 A4:1 B4:1 D5:1 E5:8

104  '''

105  SONGS['PHASE3'] = (75, parse(lead), parse(bass), 'Phase 3, THE VOID: syncopated groove, E minor')

106

107  # ---------------------------------------------------------------- boss

108  # Tremolo-picked bass (16ths), chromatic wailing lead, tick 70 ms.

109  bass = ' '.join([' '.join(['E2:1'] * 16), ' '.join(['F2:1'] * 16),

110                   ' '.join(['E2:1'] * 8 + ['G2:1'] * 4 + ['Bb2:1'] * 4),

111                   ' '.join(['Bb2:1'] * 8 + ['A2:1'] * 8)])

112  lead = '''

113  E5:2 F5:2 E5:2 D#5:2 E5:8

114  F5:2 G5:2 F5:2 E5:2 F5:8

115  G5:2 A5:2 Bb5:2 A5:2 G5:4 E5:4

116  Bb4:4 A4:4 Ab4:4 G4:4

117  '''

118  SONGS['BOSS'] = (70, parse(lead), parse(bass), 'Boss fights: tremolo bass, chromatic lead')

119

120

121  def c_array(name, notes):

122      rows, line = [], []

123      for p, t in notes:

124          line.append('{%d, %d}' % (p, t))

125          if len(line) == 8:

126              rows.append('  ' + ', '.join(line) + ',')

127              line = []

128      if line:

129          rows.append('  ' + ', '.join(line) + ',')

130      return 'const Note %s[] PROGMEM = {\n%s\n};' % (name, '\n'.join(rows))

131

132

133  out = []

134  for key, (tick, lead, bass, title) in SONGS.items():

135      rhythm = fifths(bass)

136      assert bars(lead) == bars(bass) == bars(rhythm), (key, bars(lead), bars(bass))

137      secs = sum(t for _, t in bass) * tick / 1000

138      out.append('// ================================================================= %s' % title)

139      out.append('// %d bars, 1 tick = %d ms, one loop = %.1f s' % (bars(bass), tick, secs))

140      out.append(c_array(key + '_LEAD', lead))

141      out.append(c_array(key + '_BASS', bass))

142      out.append(c_array(key + '_RHYTHM', rhythm))

143      out.append('const Track SONG_%s[BUZZER_COUNT] = {TRACK(%s_LEAD), TRACK(%s_BASS), NO_TRACK, TRACK(%s_RHYTHM)};'

144                 % (key, key, key, key))

145      out.append('const uint8_t SONG_%s_TICK_MS = %d;\n' % (key, tick))

146

147  BEGIN = '// ---- BEGIN songs generated by tools/compose_music.py (edit the songs there)\n'

148  END = '// ---- END songs generated by tools/compose_music.py\n'

149  path = Path(__file__).resolve().parents[1] / 'void_runner' / 'tunes.h'

150  text = path.read_text(encoding='utf-8')

151  start, stop = text.index(BEGIN) + len(BEGIN), text.index(END)

152  path.write_text(text[:start] + '\n'.join(out) + text[stop:], encoding='utf-8', newline='\n')

153  print('wrote %d songs into %s' % (len(SONGS), path))

### tools/render_dumps.py – Screen renderer

Draws screen dumps (the !d command) as PNG pictures or GIF animations in the real LED colours.

1  #!/usr/bin/env python3

2  """

3  VOID RUNNER - turns screen dumps ("!d" command, or the PC harness's "record") into

4  pictures in the real LED colours of the circuit (from gen_circuit.py). Useful to review

5  animation frames without watching the simulation, and for the documentation.

6

7  Usage:

8      python tools/render_dumps.py <log> <out.png>               all screens side by side

9      python tools/render_dumps.py <log> <out.png> 3 7 12         only screens 3, 7 and 12

10      python tools/render_dumps.py <log> <out.gif> [ms]           animation (needs Pillow),

11                                                                  ms per frame (default 40)

12  """

13  import re

14  import struct

15  import sys

16  import zlib

17  from pathlib import Path

18

19  sys.path.insert(0, str(Path(__file__).parent))

20  import gen_circuit as gc                    # LED colours and screen size

21

22  BACKGROUND = (40, 40, 40)

23

24

25  def hexrgb(s):

26      return tuple(int(s[i:i + 2], 16) for i in (1, 3, 5))

27

28

29  def read_dumps(text):

30      dumps, cur = [], None

31      for line in re.sub(r'\[DUMP\]', '\n[DUMP]', text).splitlines():

32          line = line.strip()

33          if line.startswith('[DUMP] begin'):

34              cur = []

35          elif line.startswith('[DUMP] end') and cur is not None:

36              dumps.append(cur)

37              cur = None

38          elif cur is not None and len(line) == gc.SCREEN_W and set(line) <= set('.#'):

39              cur.append(line)

40      return [d for d in dumps if len(d) == gc.TOTAL_ROWS]

41

42

43  def draw_screen(img, dump, ox, oy, pitch):

44      """Paints one screen into img (a list of pixel rows) with its top-left at (ox, oy)."""

45      r_on, r_off = pitch * 5 // 14, pitch * 4 // 14

46      for i, line in enumerate(dump):                     # dump order: status bars, then game rows

47          row = gc.SCREEN_H + i if i < gc.STATUS_ROWS else i - gc.STATUS_ROWS

48          light, body = gc.led_colours(row)

49          gy = i if i < gc.STATUS_ROWS else i + 1          # one empty row under the status bars

50          for x, ch in enumerate(line):

51              c = hexrgb(light) if ch == '#' else hexrgb(body)

52              cx, cy = ox + x * pitch + pitch // 2, oy + gy * pitch + pitch // 2

53              r = r_on if ch == '#' else r_off

54              for y in range(cy - r, cy + r + 1):

55                  for xx in range(cx - r, cx + r + 1):

56                      if (xx - cx) ** 2 + (y - cy) ** 2 <= r * r:

57                          img[y][xx] = c

58

59

60  def screen_size(pitch):

61      return gc.SCREEN_W * pitch, (gc.TOTAL_ROWS + 1) * pitch

62

63

64  def render(dumps, path, pitch=14, gap=24):

65      fw, fh = screen_size(pitch)

66      W, H = len(dumps) * (fw + gap) + gap, fh + 2 * gap

67      img = [[BACKGROUND] * W for _ in range(H)]

68      for n, dump in enumerate(dumps):

69          draw_screen(img, dump, gap + n * (fw + gap), gap, pitch)

70      raw = b''.join(b'\x00' + bytes(v for px in row for v in px) for row in img)

71      def chunk(tag, data):

72          return struct.pack('>I', len(data)) + tag + data + struct.pack('>I', zlib.crc32(tag + data) & 0xFFFFFFFF)

73      png = b'\x89PNG\r\n\x1a\n' + chunk(b'IHDR', struct.pack('>IIBBBBB', W, H, 8, 2, 0, 0, 0))

74      Path(path).write_bytes(png + chunk(b'IDAT', zlib.compress(raw, 9)) + chunk(b'IEND', b''))

75

76

77  def render_gif(dumps, path, ms=40, pitch=10, gap=10):

78      from PIL import Image                           # Pillow: pip install pillow

79      fw, fh = screen_size(pitch)

80      W, H = fw + 2 * gap, fh + 2 * gap

81      frames = []

82      for dump in dumps:

83          img = [[BACKGROUND] * W for _ in range(H)]

84          draw_screen(img, dump, gap, gap, pitch)

85          frame = Image.new('RGB', (W, H))

86          frame.putdata([px for row in img for px in row])

87          frames.append(frame.convert('P', palette=Image.ADAPTIVE, colors=32))

88      frames[0].save(path, save_all=True, append_images=frames[1:], duration=ms, loop=0, optimize=True)

89

90

91  if __name__ == '__main__':

92      dumps = read_dumps(Path(sys.argv[1]).read_text(encoding='utf-8', errors='replace'))

93      out = sys.argv[2]

94      if out.endswith('.gif'):

95          render_gif(dumps, out, int(sys.argv[3]) if len(sys.argv) > 3 else 40)

96      else:

97          if len(sys.argv) > 3:                       # pick screens by number (1 = first)

98              dumps = [dumps[int(n) - 1] for n in sys.argv[3:]]

99          render(dumps, out)

100      print('rendered %d screens to %s' % (len(dumps), out))

### tools/draw_wiring.py – Wiring diagram

Draws the simplified wiring diagram.

1  #!/usr/bin/env python3

2  """

3  VOID RUNNER - draws docs/media/wiring.png: a simplified wiring diagram of the circuit (what

4  connects to which Mega pin), in the wire colours of diagram.json. The pin lists come from

5  gen_circuit.py, so the picture always matches the real circuit.

6

7  Usage: python tools/draw_wiring.py     (needs Pillow)

8  """

9  import sys

10  from pathlib import Path

11

12  from PIL import Image, ImageDraw, ImageFont

13

14  sys.path.insert(0, str(Path(__file__).parent))

15  import contextlib, io

16  with contextlib.redirect_stdout(io.StringIO()):

17      import gen_circuit as gc                  # pin lists and colours

18

19  W, H = 1500, 960

20  BG, FG, DIM = (40, 40, 40), (230, 230, 230), (150, 150, 150)

21  ORANGE, BLUE, VIOLET, GREEN, GREY, BLACK = (240, 150, 30), (70, 120, 255), (200, 90, 230), (60, 200, 80), (170, 170, 170), (15, 15, 15)

22  MEGA = (0, 110, 150)

23  img = Image.new('RGB', (W, H), BG)

24  d = ImageDraw.Draw(img)

25

26

27  def font(size, bold=False):

28      for name in (('arialbd.ttf' if bold else 'arial.ttf'), 'DejaVuSans.ttf'):

29          try:

30              return ImageFont.truetype(name, size)

31          except OSError:

32              pass

33      return ImageFont.load_default()

34

35

36  F_TITLE, F_HEAD, F, F_SMALL = font(30, True), font(19, True), font(16), font(14)

37

38

39  def text(xy, s, f=F, fill=FG, anchor='la'):

40      d.text(xy, s, font=f, fill=fill, anchor=anchor)

41

42

43  def line(points, fill, width=3):

44      d.line(points, fill=fill, width=width, joint='curve')

45

46

47  def pin_range(pins):

48      """'22-45, 47-49' from a list of pin numbers."""

49      nums = sorted(int(p) for p in pins)

50      parts, start = [], nums[0]

51      for a, b in zip(nums, nums[1:] + [None]):

52          if b != a + 1:

53              parts.append('%d' % start if start == a else '%d-%d' % (start, a))

54              start = b

55      return ', '.join(parts)

56

57

58  text((30, 20), 'VOID RUNNER - wiring', F_TITLE)

59  text((30, 60), 'Simplified: every LED, key and buzzer of the real circuit (diagram.json) is wired this way.', F, DIM)

60

61  # ---------------------------------------------------------------- Mega

62  MX, MY, MW, MH = 560, 150, 360, 560

63  d.rounded_rectangle((MX, MY, MX + MW, MY + MH), 14, fill=MEGA)

64  text((MX + MW / 2, MY + 22), 'Arduino Mega 2560', F_HEAD, anchor='ma')

65  headers = [  # (y, label, colour)

66      (MY + 90, 'A0 - A15  -  16 LED columns (-)', BLUE),

67      (MY + 200, 'pins %s  -  27 LED rows (+)' % pin_range(gc.ROW_PINS), ORANGE),

68      (MY + 330, 'pins %s-%s  -  keypad rows' % (gc.KEY_ROW_PINS[0], gc.KEY_ROW_PINS[-1]), VIOLET),

69      (MY + 390, 'pins %s-%s  -  keypad columns' % (gc.KEY_COL_PINS[-1], gc.KEY_COL_PINS[0]), GREEN),

70      (MY + 480, 'pins %s  -  buzzers' % ', '.join(gc.BUZZER_PINS), VIOLET),

71      (MY + 545, 'GND  -  buzzers (-)', GREY),

72  ]

73  for y, label, colour in headers:

74      d.rectangle((MX + 12, y - 9, MX + 24, y + 9), fill=colour)

75      text((MX + 34, y), label, F_SMALL, anchor='lm')

76

77  # ---------------------------------------------------------------- LED matrix (corner shown)

78  LX, LY, P = 60, 170, 70                       # 5 x 5 LEDs drawn, pitch P

79  rows_shown = [('row 1 (red bar)', (255, 60, 60)), ('row 2 (yellow bar)', (255, 210, 60)),

80                ('row 3 (green bar)', (60, 255, 80)), ('row 4 (game, red)', (255, 60, 60)),

81                ('...', DIM)]

82  text((LX, LY - 85), 'LED screen: 27 rows x 16 columns = 432 LEDs', F_HEAD)

83  for r, (name, colour) in enumerate(rows_shown):

84      y = LY + r * P

85      line([(LX + 10, y), (LX + 380, y)], ORANGE, 2)                     # row line (anodes)

86  for c in range(5):

87      x = LX + 40 + c * P

88      line([(x + 12, LY - 20), (x + 12, LY + 5 * P - 30)], BLUE, 2)     # column line (cathodes)

89  for r in range(4):

90      for c in range(5):

91          x, y = LX + 40 + c * P, LY + r * P

92          colour = rows_shown[r][1]

93          d.ellipse((x - 9, y + 4, x + 9, y + 22), fill=colour if (r + c) % 3 == 0 else (70, 70, 70), outline=FG)

94          line([(x, y), (x, y + 4)], ORANGE, 2)                          # anode leg to the row

95          line([(x + 6, y + 18), (x + 12, y + 18)], BLUE, 2)             # cathode leg to the column

96  text((LX + 40, LY + 5 * P - 25), 'col 1   col 2   col 3   col 4   ...  col 16', F_SMALL, DIM)

97  # the bundles to the Mega

98  line([(LX + 380, LY), (LX + 380, MY + 200), (MX, MY + 200)], ORANGE, 6)

99  text((LX + 392, MY + 170), '27 row wires', F_SMALL, ORANGE)

100  line([(LX + 52, LY - 20), (LX + 52, LY - 40), (LX + 460, LY - 40), (LX + 460, MY + 90), (MX, MY + 90)], BLUE, 6)

101  text((LX + 300, LY - 62), '16 column wires', F_SMALL, BLUE)

102  text((LX, LY + 5 * P + 10),

103       'Each LED: + leg on its row wire, - leg on its column wire.\n'

104       'The Mega switches ONE row on at a time (71 times a second for\n'

105       'the whole screen) and pulls the columns of the LEDs that\n'

106       'should light LOW. Your eye sees a steady picture.', F, FG)

107

108  # ---------------------------------------------------------------- keypad

109  KX, KY, KP = 1000, 150, 70

110  text((KX, KY - 45), 'Keypad: %d push buttons, %d x %d' % (gc.KR * gc.KC, gc.KR, gc.KC), F_HEAD)

111  labels = [b[0] for row in gc.BUTTONS for b in row]

112  for r in range(gc.KR):

113      y = KY + 20 + r * KP

114      line([(KX - 30, y), (KX + gc.KC * KP - 20, y)], VIOLET, 2)

115      text((KX + gc.KC * KP - 10, y), 'row %d -> pin %s' % (r + 1, gc.KEY_ROW_PINS[r]), F_SMALL, VIOLET, 'lm')

116  for c in range(gc.KC):

117      x = KX + 20 + c * KP

118      line([(x + 18, KY), (x + 18, KY + gc.KR * KP + 10)], GREEN, 2)

119      text((x + 18, KY + gc.KR * KP + 14), 'pin %s' % gc.KEY_COL_PINS[c], F_SMALL, GREEN, 'ma')

120  for r in range(gc.KR):

121      for c in range(gc.KC):

122          x, y = KX + 20 + c * KP, KY + 20 + r * KP

123          d.rounded_rectangle((x - 16, y - 16, x + 16, y + 16), 5, fill=(70, 70, 70), outline=FG)

124          text((x, y), labels[r * gc.KC + c], F_HEAD, anchor='mm')

125  line([(KX - 30, KY + 20), (KX - 30, MY + 330), (MX + MW, MY + 330)], VIOLET, 6)

126  line([(KX + 38, KY + gc.KR * KP + 40), (KX + 38, MY + 390), (MX + MW, MY + 390)], GREEN, 6)

127  text((KX + 60, KY + gc.KR * KP + 45),

128       'A button connects its row to its column.\n'

129       'The Mega drives one row LOW at a time and\n'

130       'reads the columns: a LOW column = that key\n'

131       'is pressed.', F, FG)

132

133  # ---------------------------------------------------------------- buzzers

134  BXp, BYp = 1000, 700

135  jobs = ['lead', 'bass', 'effects', 'rhythm']

136  text((BXp, BYp - 110), 'Buzzers: 4, one hardware timer each', F_HEAD)

137  for i, pin in enumerate(gc.BUZZER_PINS):

138      x = BXp + 20 + i * 115

139      y = BYp - 18 - i * 12                       # each buzzer has its OWN wire to its own pin

140      line([(x, BYp), (x, y), (MX + MW, y)], VIOLET, 3)

141      d.ellipse((x - 26, BYp, x + 26, BYp + 52), fill=BLACK, outline=FG)

142      text((x, BYp + 26), 'B%d' % (i + 1), F_HEAD, anchor='mm')

143      line([(x, BYp + 52), (x, BYp + 80)], GREY, 3)                      # other leg: ground

144      text((x + 6, BYp + 92), 'pin %s' % pin, F_SMALL, VIOLET)

145      text((x + 6, BYp + 110), jobs[i], F_SMALL, DIM)

146  line([(BXp + 20, BYp + 80), (BXp + 20 + 3 * 115, BYp + 80)], GREY, 3)

147  line([(BXp + 20, BYp + 80), (MX + MW + 20, BYp + 80), (MX + MW + 20, MY + 545), (MX + MW, MY + 545)], GREY, 6)

148  text((BXp, BYp + 140),

149       'Signal leg -> its own pin; each pin is the output of a timer\n'

150       '(Timer3, Timer4, Timer1, Timer5). Other legs -> GND together.', F, FG)

151

152  # ---------------------------------------------------------------- legend

153  text((60, 660), 'Thick line = a bundle of separate wires side by side\n'

154                  '(one wire per LED row, LED column, key row or key column).', F, DIM)

155

156  text((30, H - 40), 'Parts: 1 Arduino Mega, 432 LEDs, 9 push buttons, 4 buzzers. No resistors, no modules, no libraries.',

157       F, DIM)

158  out = Path(__file__).resolve().parents[1] / 'docs' / 'media' / 'wiring.png'

159  img.save(out)

160  print('wrote', out)

### tools/render_circuit.py – Circuit photo

Draws the whole circuit with Wokwi's own part pictures, lit with a game picture.

1  """

2  render_circuit.py -- a "photo" of the whole circuit as Wokwi draws it: the parts of the

3  generated circuit (tools/gen_circuit.py) with Wokwi's own part drawings (@wokwi/elements),

4  every visible wire, and the LEDs lit with one real game picture from a screen dump.

5

6  Usage (from the project folder):

7      python tools/render_circuit.py <dump log> <dump number> <out.png> [zoom]

8  e.g. python tools/render_circuit.py build/rec_phase1.txt 120 docs/media/circuit_photo.png 2

9

10  Writes a web page next to the picture and lets Microsoft Edge (headless) take the screenshot;

11  the page loads @wokwi/elements from the internet.

12  """

13  import importlib.util

14  import json

15  import subprocess

16  import sys

17  from pathlib import Path

18

19  ROOT = Path(__file__).resolve().parent.parent

20  EDGE = r'C:\Program Files (x86)\Microsoft\Edge\Application\msedge.exe'

21  SCREEN_W, SCREEN_H = 16, 24

22

23  spec = importlib.util.spec_from_file_location('gen_circuit', ROOT / 'tools' / 'gen_circuit.py')

24  gen = importlib.util.module_from_spec(spec)

25  spec.loader.exec_module(gen)               # builds parts, wires and PIN_TABLE (no files written)

26

27

28  def read_dump(log, number):

29      """Lit pixels of dump `number` (1 = first): {led id: True}."""

30      blocks = Path(log).read_text(encoding='utf-8').split('[DUMP] begin')[1:]

31      rows = [r for r in blocks[number - 1].splitlines()[1:] if r and not r.startswith('[')]

32      bars, game = rows[:3], [r for r in rows[3:] if not r.startswith('-')]

33      lit = {}

34      for k, r in enumerate(bars):

35          for x, c in enumerate(r):

36              if c == '#':

37                  lit['st_%d_%d' % (x, k)] = True

38      for y, r in enumerate(game[:SCREEN_H]):

39          for x, c in enumerate(r):

40              if c == '#':

41                  lit['led_%d_%d' % (x, y)] = True

42      return lit

43

44

45  def page(lit, zoom):

46      pts = list(gen.PIN_TABLE.values()) + [pt for w in gen.wires for pt in gen.polyline(w)]

47      xs, ys = zip(*pts)

48      x0, y0 = min(xs) - 80, min(ys) - 80

49      w, h = max(xs) - x0 + 120, max(ys) - y0 + 120

50      parts = []

51      for p in gen.parts:

52          a = dict(p.get('attrs', {}))

53          tag = p['type']

54          attrs = ''.join(' %s="%s"' % (k, v) for k, v in a.items() if k not in ('gamma', 'fps', 'key'))

55          if tag == 'wokwi-led' and lit.get(p['id']):

56              attrs += ' data-lit="1"'

57          rot = p.get('rotate', 0)

58          style = 'left:%.1fpx;top:%.1fpx;%s' % (p['left'] - x0, p['top'] - y0,

59                                                  'transform:rotate(%ddeg);' % rot if rot else '')

60          parts.append('<%s class="p" style="%s"%s></%s>' % (tag, style, attrs, tag))

61      lines = []

62      for wire in gen.wires:

63          if not wire['colour']:

64              continue

65          d = ' '.join('%.1f,%.1f' % (x - x0, y - y0) for x, y in gen.polyline(wire))

66          lines.append('<polyline points="%s" stroke="%s"/>' % (d, wire['colour']))

67      return f'''<!doctype html><html><head><meta charset="utf-8">

68  <style>html,body{{margin:0;background:#2b2b2b}}#c{{position:relative;width:{w:.0f}px;height:{h:.0f}px;

69  transform:scale({zoom});transform-origin:0 0}}.p{{position:absolute}}

70  svg{{position:absolute;left:0;top:0;pointer-events:none}}

71  polyline{{fill:none;stroke-width:2.2;stroke-linejoin:round;opacity:.9}}</style>

72  <script type="module">import 'https://cdn.jsdelivr.net/npm/@wokwi/elements@1.9.2/+esm';

73  customElements.whenDefined('wokwi-led').then(() =>

74    document.querySelectorAll('[data-lit]').forEach(e => e.value = true));</script>

75  </head><body><div id="c">{''.join(parts)}

76  <svg width="{w:.0f}" height="{h:.0f}">{''.join(lines)}</svg></div></body></html>''', w * zoom, h * zoom

77

78

79  def main():

80      log, number, out = sys.argv[1], int(sys.argv[2]), Path(sys.argv[3]).resolve()

81      zoom = float(sys.argv[4]) if len(sys.argv) > 4 else 2

82      html, w, h = page(read_dump(log, number), zoom)

83      page_file = out.with_suffix('.html')

84      page_file.write_text(html, encoding='utf-8', newline='\n')

85      subprocess.run([EDGE, '--headless=new', '--disable-gpu', '--hide-scrollbars',

86                      '--virtual-time-budget=15000', '--window-size=%d,%d' % (w, h),

87                      '--screenshot=%s' % out, page_file.as_uri()], check=True,

88                     stdout=subprocess.DEVNULL, stderr=subprocess.DEVNULL)

89      page_file.unlink()

90      print('wrote', out, '%dx%d' % (w, h))

91

92

93  if __name__ == '__main__':

94      main()

### tools/make_doc.py – This document

Builds this document from the code and the game itself.

1  """

2  make_doc.py -- builds the project documentation docs/VOID_RUNNER_documentation.docx

3  (open it in Google Docs: Drive -> New -> File upload, then "Open with Google Docs").

4

5  Parts: 1 cover page, 2 the hardware (a picture of the whole circuit), 3 project performance

6  (how the device works, with pictures of every screen), 4 the full code with explanations.

7

8  All pictures come from the real code: the PC harness plays the game (tools/doc_record.script)

9  and its screen dumps are drawn in the real LED colours (render_dumps.py); the circuit is

10  drawn with Wokwi's own part drawings (render_circuit.py, needs Edge and the internet).

11

12  Usage (from the project folder, after `python tests/host/run_tests.py` built build/sim.exe):

13      python tools/make_doc.py

14  Needs python-docx and Pillow (pip install python-docx pillow).

15  """

16  import re

17  import subprocess

18  import sys

19  from pathlib import Path

20

21  from docx import Document

22  from docx.enum.section import WD_SECTION

23  from docx.enum.table import WD_TABLE_ALIGNMENT

24  from docx.enum.text import WD_ALIGN_PARAGRAPH, WD_BREAK

25  from docx.oxml import OxmlElement

26  from docx.oxml.ns import qn

27  from docx.shared import Cm, Inches, Pt, RGBColor

28  from PIL import Image, ImageDraw

29

30  ROOT = Path(__file__).resolve().parent.parent

31  sys.path.insert(0, str(ROOT / 'tools'))

32  import render_dumps as rd                                    # noqa: E402

33

34  BUILD = ROOT / 'build'

35  MEDIA = BUILD / 'doc_media'

36  OUT = ROOT / 'docs' / 'VOID_RUNNER_documentation.docx'

37

38  # ---- cover page data (fill in)

39  COVER = {

40      'university': '[University name]',

41      'faculty': '[Faculty / Department]',

42      'course': 'Digital Systems 2 (Microprocessors)',

43      'project': 'Project 1',

44      'student': '[Student name]',

45      'student_id': '[Student ID]',

46      'teacher': '[Teacher name]',

47      'semester': '[Semester, e.g. Fall 2026]',

48      'date': '[Submission date]',

49      'repo': 'https://github.com/Arash1Kazemi/void-runner',

50  }

51

52  ACCENT = RGBColor(0x1F, 0x3A, 0x5F)

53  GREY = RGBColor(0x60, 0x60, 0x60)

54

55  # ================================================================= pictures

56  DUMP_NAMES = [

57      'title', 'title_push', 'menu', 'hangar_bolt', 'hangar_dart', 'hangar_twin',

58      'opt_lite', 'opt_snd', 'opt_diff', 'opt_test', 'led1', 'led2', 'led3', 'led4',

59      'best_empty', 'zone1', 'rockbelt', 'go', 'play1', 'play2', 'pause_score', 'pause_sign',

60      'warn1', 'warn2', 'warden1', 'warden2', 'bossdown', 'zonedone', 'pts', 'lifeup',

61      'phase2a', 'phase2b', 'hunter1', 'hunter2', 'phase3a', 'phase3b', 'king', 'king2',

62      'vic_a', 'vic_b', 'vic_c', 'vic_d', 'over1', 'over2', 'best1', 'best2', 'best3',

63  ]

64

65

66  def screen_png(dump, name, pitch=14):

67      path = MEDIA / (name + '.png')

68      rd.render([dump], path, pitch=pitch, gap=10)

69      return path

70

71

72  def make_media():

73      MEDIA.mkdir(parents=True, exist_ok=True)

74      sim = BUILD / ('sim.exe' if sys.platform == 'win32' else 'sim')

75      log = subprocess.run([str(sim), 'tools/doc_record.script'], cwd=ROOT, capture_output=True,

76                           text=True, errors='replace').stdout

77      dumps = rd.read_dumps(log)

78      assert len(dumps) == len(DUMP_NAMES), 'got %d dumps' % len(dumps)

79      pics = {n: screen_png(d, n) for n, d in zip(DUMP_NAMES, dumps)}

80      logo = rd.read_dumps((BUILD / 'doc_logo.txt').read_text(encoding='utf-8'))

81      for name, t in [('logo_warp', 600), ('logo_ship', 1600), ('logo_letters', 3100),

82                      ('logo_runner', 4200), ('logo_flash', 4900), ('logo_glow', 5400)]:

83          pics[name] = screen_png(logo[min(t // 100, len(logo) - 1)], name)

84      # busy moments from the clips of docs/media (tools/media_record.script)

85      subprocess.run([str(sim), 'tools/media_record.script'], cwd=ROOT, capture_output=True)

86      for clip, picks in [('phase1', [44, 120, 200, 300]), ('warden', [66, 108, 148]),

87                          ('king', [60, 140, 213])]:

88          clipdumps = rd.read_dumps((BUILD / ('rec_%s.txt' % clip)).read_text(encoding='utf-8'))

89          for k, i in enumerate(picks):

90              pics['%s_%d' % (clip, k)] = screen_png(clipdumps[min(i, len(clipdumps)) - 1],

91                                                     '%s_%d' % (clip, k))

92      # the whole circuit, lit with a boss-fight picture, and close-ups of its parts

93      photo = MEDIA / 'circuit_photo.png'

94      subprocess.run([sys.executable, 'tools/render_circuit.py', 'build/rec_warden.txt', '66',

95                      str(photo), '2'], cwd=ROOT, check=True)

96      pics['circuit'] = photo

97      pics.update(circuit_closeups(photo))

98      pics['wiring'] = ROOT / 'docs' / 'media' / 'wiring.png'

99      pics.update(sprite_sheets())

100      return pics

101

102

103  def circuit_closeups(photo):

104      """Crops the LED panel, the Mega, the keypad and the buzzers out of the circuit photo."""

105      import render_circuit as rc

106      gen = rc.gen

107      pts = list(gen.PIN_TABLE.values()) + [pt for w in gen.wires for pt in gen.polyline(w)]

108      xs, ys = zip(*pts)

109      x0, y0, zoom = min(xs) - 80, min(ys) - 80, 2

110      im = Image.open(photo)

111      boxes = {}

112      for kind, key in [('wokwi-led', 'cu_leds'), ('wokwi-pushbutton', 'cu_keypad'),

113                        ('wokwi-buzzer', 'cu_buzzers'), ('wokwi-arduino-mega', 'cu_mega')]:

114          ps = [p for p in gen.parts if p['type'] == kind]

115          size = {'wokwi-led': (40, 50), 'wokwi-pushbutton': (70, 50), 'wokwi-buzzer': (70, 80),

116                  'wokwi-arduino-mega': (400, 200)}[kind]

117          l = min(p['left'] for p in ps) - 30

118          t = min(p['top'] for p in ps) - 30

119          r = max(p['left'] for p in ps) + size[0] + 30

120          b = max(p['top'] for p in ps) + size[1] + 30

121          box = tuple(int((v - o) * zoom) for v, o in ((l, x0), (t, y0), (r, x0), (b, y0)))

122          path = MEDIA / (key + '.png')

123          im.crop(box).save(path)

124          boxes[key] = path

125      return boxes

126

127

128  def parse_art():

129      """The sprites of art.h: {NAME: list of row strings ('#' / '.')}."""

130      text = (ROOT / 'void_runner' / 'art.h').read_text(encoding='utf-8')

131      art = {}

132      for m in re.finditer(r'const uint16_t (\w+)\[\d+\] PROGMEM = \{(.*?)\};', text, re.S):

133          rows = [int(b, 2) for b in re.findall(r'0b([01]{16})', m.group(2))]

134          if not rows:

135              continue

136          width = max(16 - (r & -r).bit_length() + 1 for r in rows if r)

137          art[m.group(1)] = [''.join('#' if r & (0x8000 >> x) else '.' for x in range(width))

138                             for r in rows]

139      return art

140

141

142  def draw_sprite(rows, colour, pitch=26):

143      w, h = len(rows[0]), len(rows)

144      pad = pitch // 2

145      im = Image.new('RGB', (w * pitch + 2 * pad, h * pitch + 2 * pad), (40, 40, 40))

146      d = ImageDraw.Draw(im)

147      for y, row in enumerate(rows):

148          for x, c in enumerate(row):

149              cx, cy = pad + x * pitch + pitch // 2, pad + y * pitch + pitch // 2

150              r = pitch * 5 // 14 if c == '#' else pitch * 4 // 14

151              d.ellipse((cx - r, cy - r, cx + r, cy + r), fill=colour if c == '#' else (58, 58, 58))

152      return im

153

154

155  def sprite_sheets():

156      art = parse_art()

157      out = {}

158      red, white, green = (255, 70, 70), (240, 240, 240), (70, 255, 90)

159      for name, colour in [('SHIP_BOLT', green), ('SHIP_DART', green), ('SHIP_TWIN', green),

160                           ('ENEMY_DRONE', red), ('ENEMY_WEAVER', red), ('ENEMY_MINER', red),

161                           ('ENEMY_CRUISER', red), ('ROCK_SMALL', white), ('ROCK_BIG', white),

162                           ('PICKUP_AMMO', white), ('PICKUP_LIFE', white), ('BOSS_WARDEN', red),

163                           ('BOSS_HUNTER', red), ('BOSS_KING', red), ('BOSS_KING_ANGRY', red),

164                           ('WARNING_SIGN', white)]:

165          path = MEDIA / ('sprite_%s.png' % name.lower())

166          draw_sprite(art[name], colour).save(path)

167          out['sprite_' + name.lower()] = path

168      return out

169

170

171  # ================================================================= document helpers

172  def shade(paragraph_or_cell, fill):

173      el = paragraph_or_cell._p if hasattr(paragraph_or_cell, '_p') else paragraph_or_cell._tc

174      pr = el.get_or_add_pPr() if hasattr(el, 'get_or_add_pPr') else el.get_or_add_tcPr()

175      shd = OxmlElement('w:shd')

176      shd.set(qn('w:val'), 'clear')

177      shd.set(qn('w:color'), 'auto')

178      shd.set(qn('w:fill'), fill)

179      pr.append(shd)

180

181

182  class Doc:

183      def __init__(self):

184          self.d = Document()

185          sec = self.d.sections[0]

186          sec.page_width, sec.page_height = Cm(21), Cm(29.7)               # A4

187          sec.left_margin = sec.right_margin = Cm(2)

188          sec.top_margin = sec.bottom_margin = Cm(2)

189          normal = self.d.styles['Normal']

190          normal.font.name = 'Arial'

191          normal.font.size = Pt(10.5)

192          normal.paragraph_format.space_after = Pt(5)

193          normal.paragraph_format.line_spacing = 1.15

194          for lvl, size in [(1, 20), (2, 14), (3, 12)]:

195              st = self.d.styles['Heading %d' % lvl]

196              st.font.name = 'Arial'

197              st.font.size = Pt(size)

198              st.font.bold = True

199              st.font.color.rgb = ACCENT

200              st.paragraph_format.space_before = Pt(14 if lvl > 1 else 0)

201              st.paragraph_format.space_after = Pt(6)

202              st.paragraph_format.keep_with_next = True

203          self.width = Cm(17)

204          self.fig = 0

205          self.tab = 0

206

207      def h(self, text, level):

208          return self.d.add_heading(text, level)

209

210      def p(self, text='', bold_lead=None, size=None, align=None, italic=False, colour=None):

211          para = self.d.add_paragraph()

212          if bold_lead:

213              r = para.add_run(bold_lead)

214              r.bold = True

215              if size:

216                  r.font.size = Pt(size)

217          r = para.add_run(text)

218          r.italic = italic

219          if size:

220              r.font.size = Pt(size)

221          if colour:

222              r.font.color.rgb = colour

223          if align:

224              para.alignment = align

225          return para

226

227      def rich(self, text):

228          """A paragraph where **bold** and `code` parts are formatted."""

229          para = self.d.add_paragraph()

230          for part in re.split(r'(\*\*.+?\*\*|`.+?`)', text):

231              if part.startswith('**'):

232                  para.add_run(part[2:-2]).bold = True

233              elif part.startswith('`'):

234                  r = para.add_run(part[1:-1])

235                  r.font.name = 'Consolas'

236                  r.font.size = Pt(9.5)

237              elif part:

238                  para.add_run(part)

239          return para

240

241      def bullets(self, items):

242          for it in items:

243              para = self.rich(it)

244              para.style = self.d.styles['List Bullet']

245              para.paragraph_format.space_after = Pt(2)

246

247      def caption(self, text):

248          self.fig += 1

249          para = self.p('Figure %d. %s' % (self.fig, text), size=9, align=WD_ALIGN_PARAGRAPH.CENTER,

250                        italic=True, colour=GREY)

251          para.paragraph_format.space_after = Pt(10)

252

253      def picture(self, path, width_cm, caption=None):

254          self.d.add_picture(str(path), width=Cm(width_cm))

255          para = self.d.paragraphs[-1]

256          para.alignment = WD_ALIGN_PARAGRAPH.CENTER

257          para.paragraph_format.keep_with_next = bool(caption)

258          para.paragraph_format.space_after = Pt(2)

259          if caption:

260              self.caption(caption)

261

262      def gallery(self, items, per_row, width_cm, caption=None):

263          """A row of pictures, each with a short label under it (items: (path, label))."""

264          rows = (len(items) + per_row - 1) // per_row

265          t = self.d.add_table(rows=rows * 2, cols=per_row)

266          t.alignment = WD_TABLE_ALIGNMENT.CENTER

267          for i, (path, label) in enumerate(items):

268              r, c = divmod(i, per_row)

269              cell = t.cell(r * 2, c)

270              para = cell.paragraphs[0]

271              para.alignment = WD_ALIGN_PARAGRAPH.CENTER

272              para.add_run().add_picture(str(path), width=Cm(width_cm))

273              lab = t.cell(r * 2 + 1, c).paragraphs[0]

274              lab.alignment = WD_ALIGN_PARAGRAPH.CENTER

275              run = lab.add_run(label)

276              run.font.size = Pt(8.5)

277              run.font.color.rgb = GREY

278          if caption:

279              self.caption(caption)

280          else:

281              self.p('')

282

283      def table(self, header, rows, widths=None, caption=None, size=9.5):

284          t = self.d.add_table(rows=1 + len(rows), cols=len(header))

285          t.style = self.d.styles['Table Grid']

286          t.alignment = WD_TABLE_ALIGNMENT.CENTER

287          for c, text in enumerate(header):

288              cell = t.cell(0, c)

289              shade(cell, '1F3A5F')

290              run = cell.paragraphs[0].add_run(text)

291              run.bold = True

292              run.font.size = Pt(size)

293              run.font.color.rgb = RGBColor(0xFF, 0xFF, 0xFF)

294          for r, row in enumerate(rows, 1):

295              for c, text in enumerate(row):

296                  cell = t.cell(r, c)

297                  if r % 2 == 0:

298                      shade(cell, 'EEF2F7')

299                  para = cell.paragraphs[0]

300                  for part in re.split(r'(\*\*.+?\*\*)', str(text)):

301                      if not part:

302                          continue

303                      run = para.add_run(part.strip('*') if part.startswith('**') else part)

304                      run.bold = part.startswith('**')

305                      run.font.size = Pt(size)

306          if widths:

307              for row in t.rows:

308                  for c, w in enumerate(widths):

309                      row.cells[c].width = Cm(w)

310          if caption:

311              self.tab += 1

312              para = self.p('Table %d. %s' % (self.tab, caption), size=9,

313                            align=WD_ALIGN_PARAGRAPH.CENTER, italic=True, colour=GREY)

314              para.paragraph_format.space_after = Pt(10)

315          else:

316              self.p('')

317          return t

318

319      def code(self, text, lang):

320          """Full source, one paragraph per line: line numbers, comments in green."""

321          lines = text.replace('\t', '    ').split('\n')

322          if lines and lines[-1] == '':

323              lines.pop()

324          in_block = False

325          digits = len(str(len(lines)))

326          for n, line in enumerate(lines, 1):

327              para = self.d.add_paragraph()

328              pf = para.paragraph_format

329              pf.space_after = pf.space_before = Pt(0)

330              pf.line_spacing = 1.0

331              shade(para, 'F6F8FA')

332              num = para.add_run(str(n).rjust(digits) + '  ')

333              num.font.name = 'Consolas'

334              num.font.size = Pt(7.5)

335              num.font.color.rgb = RGBColor(0xA0, 0xA0, 0xA0)

336              for chunk, kind in split_code(line, lang, in_block):

337                  r = para.add_run(chunk)

338                  r.font.name = 'Consolas'

339                  r.font.size = Pt(7.5)

340                  r.font.color.rgb = {'comment': RGBColor(0x2E, 0x7D, 0x32),

341                                      'pre': RGBColor(0x8E, 0x24, 0xAA),

342                                      'string': RGBColor(0xB2, 0x3A, 0x1D),

343                                      'code': RGBColor(0x1A, 0x1A, 0x1A)}[kind]

344              in_block = block_state(line, lang, in_block)

345          self.p('')

346

347      def page_break(self):

348          self.d.add_paragraph().add_run().add_break(WD_BREAK.PAGE)

349

350

351  def block_state(line, lang, in_block):

352      if lang != 'c':

353          return False

354      s = line

355      while True:

356          if in_block:

357              end = s.find('*/')

358              if end < 0:

359                  return True

360              s, in_block = s[end + 2:], False

361          else:

362              s = re.sub(r'"(\\.|[^"\\])*"', '""', s)

363              start, lc = s.find('/*'), s.find('//')

364              if start < 0 or (0 <= lc < start):

365                  return False

366              s, in_block = s[start + 2:], True

367

368

369  def split_code(line, lang, in_block):

370      """Splits one line into (text, kind) parts: comment, pre(processor), string or code."""

371      if lang == 'c' and in_block:

372          end = line.find('*/')

373          if end < 0:

374              return [(line, 'comment')]

375          return [(line[:end + 2], 'comment')] + split_code(line[end + 2:], lang, False)

376      if lang == 'c' and line.lstrip().startswith('#'):

377          return [(line, 'pre')]

378      comment = '//' if lang == 'c' else '#'

379      out, i, start = [], 0, 0

380      while i < len(line):

381          ch = line[i]

382          if ch in '"\'' and not (lang == 'c' and ch == "'" and False):

383              j = i + 1

384              while j < len(line) and line[j] != ch:

385                  j += 2 if line[j] == '\\' else 1

386              out += [(line[start:i], 'code'), (line[i:j + 1], 'string')]

387              i = start = j + 1

388              continue

389          if line.startswith(comment, i) or (lang == 'c' and line.startswith('/*', i)):

390              out.append((line[start:i], 'code'))

391              if lang == 'c' and line.startswith('/*', i):

392                  end = line.find('*/', i + 2)

393                  if end >= 0:

394                      out.append((line[i:end + 2], 'comment'))

395                      i = start = end + 2

396                      continue

397              out.append((line[i:], 'comment'))

398              return [p for p in out if p[0]]

399          i += 1

400      out.append((line[start:], 'code'))

401      return [p for p in out if p[0]]

402

403

404  def functions_of(text):

405      """(signature, description) of every function: the // comment lines just above it."""

406      lines = text.split('\n')

407      found = []

408      for i, line in enumerate(lines):

409          m = re.match(r'^(?!\s)(?:static\s+)?[\w:<>\*&]+(?:\s+[\w:<>\*&]+)*\s+\**(\w+)\(([^)]*)\)\s*\{', line)

410          if not m or m.group(1) in ('if', 'for', 'while', 'switch'):

411              continue

412          desc, k = [], i - 1

413          while k >= 0 and lines[k].startswith('//'):

414              line_text = lines[k][2:].strip()

415              if re.match(r'^[=\-]{3,}', line_text):          # a section divider, not a description

416                  break

417              desc.insert(0, line_text)

418              k -= 1

419          found.append(('%s(%s)' % (m.group(1), m.group(2).strip()), ' '.join(desc)))

420      return found

421

422

423  # ================================================================= the document

424  def cover(doc):

425      d = doc.d

426      for _ in range(3):

427          doc.p('')

428      doc.p(COVER['university'], size=16, align=WD_ALIGN_PARAGRAPH.CENTER).runs[-1].bold = True

429      doc.p(COVER['faculty'], size=12, align=WD_ALIGN_PARAGRAPH.CENTER)

430      doc.p('')

431      doc.p(COVER['course'], size=13, align=WD_ALIGN_PARAGRAPH.CENTER)

432      doc.p(COVER['project'], size=12, align=WD_ALIGN_PARAGRAPH.CENTER, colour=GREY)

433      for _ in range(2):

434          doc.p('')

435      t = doc.p('VOID RUNNER', size=40, align=WD_ALIGN_PARAGRAPH.CENTER)

436      t.runs[-1].bold = True

437      t.runs[-1].font.color.rgb = ACCENT

438      doc.p('A vertical space shooter on an Arduino Mega', size=14,

439            align=WD_ALIGN_PARAGRAPH.CENTER, italic=True)

440      doc.p('432 single LEDs · a 9-button keypad · 4 buzzers · no modules, no libraries', size=10.5,

441            align=WD_ALIGN_PARAGRAPH.CENTER, colour=GREY)

442      for _ in range(3):

443          doc.p('')

444      info = [('Student', COVER['student']), ('Student ID', COVER['student_id']),

445              ('Teacher', COVER['teacher']), ('Course', COVER['course']),

446              ('Semester', COVER['semester']), ('Date', COVER['date']),

447              ('Platform', 'Arduino Mega 2560, simulated in Wokwi'), ('Source code', COVER['repo'])]

448      t = d.add_table(rows=len(info), cols=2)

449      t.alignment = WD_TABLE_ALIGNMENT.CENTER

450      for r, (k, v) in enumerate(info):

451          a, b = t.cell(r, 0), t.cell(r, 1)

452          a.width, b.width = Cm(4), Cm(10)

453          ra = a.paragraphs[0].add_run(k)

454          ra.bold = True

455          ra.font.size = Pt(11)

456          ra.font.color.rgb = ACCENT

457          b.paragraphs[0].add_run(v).font.size = Pt(11)

458      doc.page_break()

459

460

461  def hardware_page(doc, pics):

462      doc.h('The hardware', 1)

463      doc.picture(pics['circuit'], 17, 'The whole circuit in Wokwi: 432 LEDs (left), Arduino Mega 2560 '

464                  '(top), 3 × 3 push-button keypad and 4 buzzers (right). The LEDs show a real '

465                  'moment of the game: the WARDEN boss fight.')

466      doc.page_break()

467

468

469  def performance(doc, pics):

470      P, R = doc.p, doc.rich

471      doc.h('3. Project performance', 1)

472      R('**VOID RUNNER** is a complete arcade game built only from the parts the course allows: '

473        'one Arduino Mega 2560, single LEDs, single push buttons and buzzers. There are no ready-made '

474        'modules (no LED matrix module, no keypad module) and no libraries; the screen, the keypad and '

475        'the four-voice music are all driven by our own code. The player flies a small ship at the '

476        'bottom of a 16 × 24 LED screen, shoots enemies and space rocks, catches ammo crates and '

477        'extra lives, and fights a boss at the end of each of the three phases. The project runs in '

478        'the Wokwi simulator (Wokwi for VS Code and wokwi.com).')

479

480      doc.h('3.1 Parts and connections', 2)

481      doc.table(['Part', 'Count', 'Connection to the Mega'], [

482          ['Arduino Mega 2560', '1', '–'],

483          ['Single LEDs (wokwi-led), one matrix', '432', '16 × 24 game screen + 3 status bars of 16. '

484           '27 row wires (anodes) on pins 22–45 and 47–49, 16 column wires (cathodes) on A0–A15'],

485          ['Push buttons (wokwi-pushbutton), 3 × 3 matrix', '9', 'rows on pins 18–20, columns on 15–17'],

486          ['Buzzers (wokwi-buzzer)', '4', 'pins 5, 6, 11, 46 (outputs of Timer3, Timer4, Timer1, Timer5)'],

487      ], [5, 1.5, 10.5], 'The parts of the device.')

488      R('53 of the Mega\'s pins are used, and every header has one job: the double header drives the '

489        'LED rows, the analog header the LED columns, the communication header the keypad and the PWM '

490        'header the buzzers. No resistors are needed in the simulation. The circuit file '

491        '(`diagram.json`) and the pin list of the code (`pins.h`) are both generated by one program '

492        '(`tools/gen_circuit.py`), which also checks every wire, so the circuit and the code always '

493        'agree.')

494      doc.picture(pics['wiring'], 17, 'Simplified wiring: how every LED, key and buzzer is connected.')

495

496      doc.h('3.2 The LED screen', 2)

497      R('All 432 LEDs form **one matrix** of 27 rows × 16 columns: the + leg of every LED is on its '

498        'row wire, the – leg on its column wire. The Mega switches on **one row at a time** and pulls '

499        'the columns of the LEDs that should light LOW. A Timer2 interrupt moves to the next row about '

500        '1900 times per second, so the whole screen is redrawn 71 times per second and the eye sees a '

501        'steady picture. The columns are written with two port registers (`PORTF`, `PORTK`) at once: '

502        'this costs about 3 % of the processor, where `digitalWrite` would cost about 35 %.')

503      R('The game draws into a picture in memory (`frame`, one 16-bit word per row) and copies it to '

504        'the screen when it is finished, so a half-drawn picture is never shown.')

505      doc.table(['Area', 'LED colour', 'Use in the game'], [

506          ['Status bar 1', 'red', 'phase progress, then the boss\'s health'],

507          ['Status bar 2', 'yellow', 'ammo (blinks when low)'],

508          ['Status bar 3', 'green', 'lives (one block of 3 LEDs per life)'],

509          ['Game rows 0–5', 'red', 'where enemies and bosses enter, "WARN", "ZONE"'],

510          ['Game rows 6–17', 'white', 'open space, message pages'],

511          ['Game rows 18–23', 'green', 'the player\'s zone'],

512      ], [4, 3, 10], 'LED colours. Each LED has a fixed colour, like the colour strips of the 1978 '

513         'Space Invaders cabinet.')

514      doc.picture(pics['cu_leds'], 6.5, 'The LED panel in Wokwi during a boss fight.')

515      R('**Brightness.** Each LED is lit only 1/27 of the time. Wokwi shows brightness as '

516        '(share of time on)^(1/gamma); with the default gamma a lit LED looked only 32 % bright, which '

517        'was hard to see. Every LED therefore has gamma = 30 (90 %), which is what a real multiplexed '

518        'display does by driving each LED harder in its short time slot. Every LED also has fps = 300, '

519        'so moving objects stay sharp, and everything that blinks stays on and off for at least '

520        '160 ms so the blink is visible.')

521      R('**Text** never scrolls (scrolling letters smeared into a white block in the simulator). '

522        'Every message is a still **page** of short words, at most four small letters per line, '

523        'shown for 0.9 s, with big digits and signs where a word is not needed.')

524

525      doc.h('3.3 The keypad and the controls', 2)

526      R('The 9 push buttons form a 3 × 3 matrix, one button for each key the game uses. The code drives one row LOW at a time and reads the '

527        'four columns with the internal pull-ups, so a pressed key reads LOW. The rows that are not '

528        'being read are switched to inputs, so two pressed keys can never short a HIGH and a LOW '

529        'output together. Every key is debounced. The buttons are labelled like keyboard keys and also '

530        'react to the PC keyboard in Wokwi.')

531      doc.picture(pics['cu_keypad'], 8, 'The keypad in Wokwi.')

532      doc.table(['Key', 'Action'], [

533          ['W A S D', 'move (tap = one pixel, hold = glide)'],

534          ['R', 'fire (tap = one bullet, hold = auto-fire) · OK in the menus'],

535          ['Q', 'back · pause in the game'],

536          ['P', 'pause / continue'],

537          ['M', 'sound on / off (any screen, saved)'],

538          ['B', 'brightness 100 / 50 / 20 % (any screen, saved)'],

539      ], [3, 14], 'The keys.')

540      R('**Firing:** press R for one bullet; keep holding and another follows every 0.2 s (a chirp '

541        'when auto-fire starts, and the ship\'s nose blinks). **Moving:** a tap moves one pixel, a '

542        'hold makes the ship glide after 0.17 s. The newest key wins: a move key stops the fire, and '

543        'while R is held the ship does not glide.')

544

545      doc.h('3.4 The four buzzers', 2)

546      R('Each buzzer is connected to the output pin of its own 16-bit hardware timer. In CTC mode with '

547        '"toggle on compare match" the timer flips the pin by itself, so a note costs no processor time '

548        'and **four notes can sound at once** (Arduino\'s `tone()` can play only one). The timers run '

549        'on a slow clock (÷256), which keeps notes up to about C7 within 1 % of their pitch.')

550      doc.picture(pics['cu_buzzers'], 8, 'The four buzzers in Wokwi.')

551      doc.table(['Buzzer', 'Pin / timer', 'Job'], [

552          ['1', '5 / Timer3', 'lead guitar (melody)'],

553          ['2', '6 / Timer4', 'bass'],

554          ['3', '11 / Timer1', 'every sound effect and menu sound (music only in short jingles)'],

555          ['4', '46 / Timer5', 'rhythm guitar: a fifth above the bass (bass + rhythm = a power chord)'],

556      ], [2, 3.5, 11.5], 'The voices.')

557

558      doc.h('3.5 Start-up logo and welcome music', 2)

559      R('When the device starts, a 5.6-second logo plays in sync with the welcome melody: the picture '

560        'runs on the song\'s own clock, so every movement lands on a note. "VOID" is a vertical sign '

561        'of bold 10 × 5 letters, and the status bars fill up like a start-up progress bar. Any key '

562        'skips the logo.')

563      doc.gallery([(pics['logo_warp'], '0.6 s warp'), (pics['logo_ship'], '1.6 s the ship'),

564                   (pics['logo_letters'], '3.1 s letters'), (pics['logo_runner'], '4.2 s RUNNER'),

565                   (pics['logo_flash'], '4.9 s flash'), (pics['logo_glow'], '5.4 s the sign')],

566                  6, 2.6, 'The start-up logo.')

567      doc.table(['Time', 'Picture', 'Sound'], [

568          ['0.0 – 1.2 s', 'warp: star streaks speed up', 'rising whoosh + low A2 drone'],

569          ['1.2 – 2.0 s', 'the ship rises and brakes', 'braking notes E4 C4 A3'],

570          ['2.0 – 2.4 s', 'the ship shoots up and out', 'quick rising glide'],

571          ['2.4 – 3.6 s', 'V, O, I, D light up one by one, flickering on; drones fly past',

572           'A4 C5 E5 A5, a crackle on each flicker'],

573          ['3.6 – 4.8 s', '"RUNNER" crosses on a dark ribbon, towed by the ship',

574           'melody + harmony + bass line'],

575          ['4.8 – 5.6 s', 'the whole screen flashes, then the sign glows', 'A-minor chord'],

576      ], [2.5, 8, 6.5], 'Timeline of the logo.')

577

578      doc.h('3.6 Title screen, demo and menu', 2)

579      R('The title screen keeps the VOID sign lit while ships fly past it, and every 7 seconds a '

580        'banner shows the pages "PUSH", a big R, "TO", "PLAY". If nobody touches a key for 20 seconds '

581        'a **demo** starts: the autopilot plays the game by itself until any key is pressed. R opens '

582        'the menu: PLAY, SHIP, OPTS and BEST (W/S choose, R open, Q back); the chosen item is '

583        'underlined.')

584      doc.gallery([(pics['title'], 'title'), (pics['title_push'], 'banner'), (pics['menu'], 'menu')],

585                  3, 3.4, 'Title screen and main menu.')

586

587      doc.h('3.7 Hangar and options', 2)

588      R('In the **hangar** the player chooses one of three ships with A/D. The ship is shown three '

589        'times bigger, and the three status bars show its speed, ammo and fire power. The **options** '

590        'are pages: LITE (brightness), SND (sound), DIFF (EASY / NORM / HARD) and TEST (the LED test). '

591        'W/S choose a page, A/D or R change it, page dots at the bottom show where you are. Every '

592        'setting is saved in the EEPROM.')

593      doc.gallery([(pics['hangar_bolt'], 'BOLT'), (pics['hangar_dart'], 'DART'),

594                   (pics['hangar_twin'], 'TWIN')], 3, 3.4, 'The hangar.')

595      doc.table(['Ship', 'Speed', 'Fire', 'Ammo', 'Note'], [

596          ['BOLT', '70 ms / pixel', 'every 0.20 s', '60', 'balanced'],

597          ['DART', '45 ms / pixel', 'every 0.22 s', '40', 'fastest, less ammo'],

598          ['TWIN', '95 ms / pixel', 'every 0.26 s', '80', 'two bullets per shot (2 ammo), slowest'],

599      ], [2.5, 3, 3, 2, 6.5], 'The three ships.')

600      doc.gallery([(pics['opt_lite'], 'LITE'), (pics['opt_snd'], 'SND'), (pics['opt_diff'], 'DIFF'),

601                   (pics['opt_test'], 'TEST')], 4, 3, 'The option pages.')

602

603      doc.h('3.8 Playing the game', 2)

604      R('A game has three phases. Each phase starts with the pages "ZONE" and a big phase number, the '

605        'phase name and "GO!" with a fanfare. Then the enemies and rocks of the phase arrive from its '

606        'spawn list. When the list is finished and the screen is empty, "WARN" and a blinking warning '

607        'sign appear with a siren while the red bar charges up, and the boss comes with its own music. '

608        'When the boss explodes, the pages "ZONE DONE", the bonus points and "LIFE UP" follow, and the '

609        'next phase begins. After the last boss the victory screen comes straight away.')

610      doc.gallery([(pics['zone1'], 'ZONE 1'), (pics['rockbelt'], 'phase name'), (pics['go'], 'GO!'),

611                   (pics['phase1_0'], 'phase 1'), (pics['phase1_1'], 'phase 1'),

612                   (pics['phase1_3'], 'phase 1')], 6, 2.6, 'Start of phase 1, ROCK BELT, and play.')

613      doc.table(['Topic', 'Rule'], [

614          ['Lives', 'EASY 4, NORMAL 3, HARD 3 (at most 4). A hit: explosion, screen shake and 2 s of '

615           'blinking shield. +1 life after each phase; extra-life pickups.'],

616          ['Ammo', 'Per ship (60 / 40 / 80), full at the start of every phase. Ammo crates give '

617           '+20 / 15 / 12 by difficulty. At 5 or less a warning beeps, the yellow bar blinks and a '

618           'crate is always sent. Empty: a dry click.'],

619          ['Enemies', 'All slower than the player; at most 4 / 5 / 6 on the screen at once (phase '

620           '1 / 2 / 3). They shoot only from the upper rows and never right next to the ship, and '

621           'they never move onto each other, so the small screen stays readable.'],

622          ['Space rocks', 'Hollow, lumpy shapes (small 3 × 3, big 4 × 3) that fall slowly and drift; '

623           'a big rock takes 2 hits and breaks into two small ones. They hurt on contact.'],

624          ['Explosions', 'Only the bosses and the player\'s ship explode into flying pixels; enemies '

625           'just vanish with a boom (flying debris looked like bullets).'],

626          ['Collisions', 'Pixel-exact: a hit counts only when lit pixels touch.'],

627          ['Score', 'Drone 10, weaver 15, miner 20, cruiser 40, rocks 5 / 10, mine 2, pickup 5, '

628           'bosses 200 / 300 / 500, phase bonus 100 × phase; at most 9999.'],

629      ], [3, 14], 'The rules of the game.')

630      items = [(pics['sprite_enemy_drone'], 'drone'), (pics['sprite_enemy_weaver'], 'weaver'),

631               (pics['sprite_enemy_miner'], 'mine-layer'), (pics['sprite_enemy_cruiser'], 'cruiser'),

632               (pics['sprite_rock_small'], 'small rock'), (pics['sprite_rock_big'], 'big rock'),

633               (pics['sprite_pickup_ammo'], 'ammo crate'), (pics['sprite_pickup_life'], 'extra life')]

634      doc.gallery(items, 8, 1.6, 'Enemies, rocks and pickups, pixel by pixel.')

635      doc.table(['Enemy', 'Behaviour'], [

636          ['Drone', 'sinks slowly, shoots straight down'],

637          ['Weaver', 'sinks while zig-zagging, shoots now and then'],

638          ['Mine-layer', 'crosses the top of the screen and drops mines (mines can be shot)'],

639          ['Cruiser', '5 hits; stops near the top, slides sideways, fires shots aimed at the player, '

640           'drops an extra life'],

641          ['Rocks', 'fall and drift sideways; a big rock breaks into two'],

642      ], [3, 14], 'Enemy types.')

643

644      doc.h('3.9 The status bars', 2)

645      R('The three bars above the game screen show the state of the game at a glance. Red: how far '

646        'the phase has come; during the warning it charges up, and in the boss fight it shows the '

647        'boss\'s health, dropping with every hit. Yellow: the ammo left (it blinks when 5 or fewer are '

648        'left). Green: the lives, one block of three LEDs per life.')

649

650      doc.h('3.10 The three phases and their bosses', 2)

651      doc.table(['Phase', 'Enemies', 'Boss'], [

652          ['1 – ROCK BELT', 'drones and many rocks', '**WARDEN**: wide (10 × 4), slides along the top, '

653           '3-way spread, drops rocks; below half health faster with a 5-way spread'],

654          ['2 – DEEP MIST', 'weavers, mine-layers, drones', '**HUNTER**: small and fast; dashes to a '

655           'new spot, fires an aimed pair, and every 7 s blinks and dives at the player'],

656          ['3 – THE VOID', 'cruisers, everything else, rock showers', '**VOID KING**: huge (12 × 5), '

657           '5-way spread, calls in drones; at half health its eye opens (roar, shake), then aimed '

658           'triple volleys and rock rain'],

659      ], [3, 4.5, 9.5], 'Phases and bosses. Boss health: 24 / 22 / 40, scaled by the difficulty.')

660      doc.gallery([(pics['warn1'], 'WARN'), (pics['warn2'], 'warning sign'),

661                   (pics['warden_0'], 'WARDEN'), (pics['warden_1'], 'WARDEN'),

662                   (pics['bossdown'], 'boss down'), (pics['zonedone'], 'ZONE DONE')], 6, 2.6,

663                  'Phase 1: the warning, the WARDEN fight and the end of the phase.')

664      doc.gallery([(pics['pts'], 'bonus'), (pics['lifeup'], 'LIFE UP'), (pics['phase2a'], 'phase 2'),

665                   (pics['phase2b'], 'phase 2'), (pics['hunter1'], 'HUNTER'),

666                   (pics['hunter2'], 'HUNTER')], 6, 2.6, 'Phase 2, DEEP MIST, and the HUNTER.')

667      doc.gallery([(pics['phase3a'], 'phase 3'), (pics['phase3b'], 'phase 3'),

668                   (pics['king_0'], 'VOID KING'), (pics['king'], 'VOID KING'),

669                   (pics['king_2'], 'eye open'), (pics['king2'], 'form 2')], 6, 2.6,

670                  'Phase 3, THE VOID, and the VOID KING in both forms.')

671      doc.gallery([(pics['sprite_boss_warden'], 'WARDEN'), (pics['sprite_boss_hunter'], 'HUNTER'),

672                   (pics['sprite_boss_king'], 'VOID KING'),

673                   (pics['sprite_boss_king_angry'], 'VOID KING, form 2')], 4, 3.5,

674                  'The bosses, pixel by pixel.')

675      doc.table(['Difficulty', 'Lives', 'Enemy fire', 'Boss health', 'Ammo crate'], [

676          ['EASY', '4', '1.5 × slower', '80 %', '+20'],

677          ['NORMAL', '3', 'normal', '100 %', '+15'],

678          ['HARD', '3', '1.33 × faster', '130 %', '+12'],

679      ], [3, 2, 4, 3.5, 3], 'Difficulty levels (OPTS → DIFF).')

680

681      doc.h('3.11 Pause, game over, victory and best score', 2)

682      R('**P** (or Q) pauses the game: the game clock stops, the score and a big pause sign are shown, '

683        'and the pages "P:GO" / "Q:EX" explain the keys. When the last life is lost, **GAME OVER** '

684        'shows the score (blinking for a new record) and the best score. After the VOID KING the '

685        '**victory** screen shows fireworks and the pages "YOU WIN!", "VOID SAFE" and the score. '

686        'The **BEST** page shows the high score, how far that game got (ZONE 1–3, or WIN) and its '

687        'difficulty; it is kept in the EEPROM, so it survives a power cut on a real Arduino.')

688      doc.gallery([(pics['pause_score'], 'pause'), (pics['pause_sign'], 'pause'),

689                   (pics['over1'], 'game over'), (pics['over2'], 'game over'),

690                   (pics['best_empty'], 'no record yet')], 5, 2.9, 'Pause and game over.')

691      doc.gallery([(pics['vic_d'], 'YOU WIN!'), (pics['vic_a'], 'VOID SAFE'), (pics['vic_c'], 'score'),

692                   (pics['best1'], 'BEST'), (pics['best2'], 'how far'), (pics['best3'], 'difficulty')],

693                  6, 2.6, 'Victory and the best score.')

694

695      doc.h('3.12 Sound and music', 2)

696      R('All music is written as note lists, one per buzzer, stored in flash. The in-game songs are '

697        'original metal tracks written for this project in a short notation '

698        '(`tools/compose_music.py`): buzzer 1 plays the lead guitar, buzzer 2 the bass and buzzer 4 a '

699        'rhythm guitar a fifth above the bass, which together sound like a power chord. Buzzer 3 is '

700        'kept free for sound effects, so an effect never cuts the music.')

701      doc.table(['Song', 'Style', 'Character'], [

702          ['ROCK BELT (phase 1)', 'fast thrash, E minor, about 190 BPM, loop 10.2 s',

703           'the gallop rhythm on low E; a question-and-answer melody'],

704          ['DEEP MIST (phase 2)', 'slow and heavy, drop-D, half-time, loop 12.8 s',

705           'short muted chugs with gaps; sparse, spooky lead'],

706          ['THE VOID (phase 3)', 'syncopated groove, E minor, loop 9.6 s',

707           'off-beat stabs; a hook you can hum'],

708          ['Boss (all three)', 'tremolo picking, chromatic, about 210 BPM, loop 4.5 s',

709           'nonstop 16th notes; a wailing lead figure'],

710      ], [4, 6, 7], 'The in-game songs.')

711      R('Other music: the start-up melody, the phase fanfare, phase clear, victory and game over. '

712        'Sound effects use priorities, so an important sound is never cut by a small one: shot, '

713        'auto-fire chirp, empty click, hit, explosion, big explosion, player hit, pickup, extra life, '

714        'low ammo, warning siren, boss roar and the LED-test sweep. Explosions are noise (a random '

715        'frequency every 6 ms); the other effects slide between two frequencies.')

716

717      doc.h('3.13 LED test', 2)

718      R('OPTS → TEST checks the hardware: every LED on, then a row sweep (status bars first), a column '

719        'sweep and a checkerboard, with rising beeps. A broken LED or wire is easy to spot.')

720      doc.gallery([(pics['led1'], 'all on'), (pics['led2'], 'rows'), (pics['led3'], 'columns'),

721                   (pics['led4'], 'checkerboard')], 4, 3, 'The LED test.')

722

723      doc.h('3.14 Reliability and measurements', 2)

724      doc.bullets([

725          '**Nothing blocks:** there is no `delay()` in the game. A new picture is made every 20 ms, '

726          'and each part of the program does one small step per picture. The game has its own clock '

727          '(`gameMs`) that stops while paused.',

728          '**Screen:** the Timer2 interrupt shows the picture whatever the game code is doing; '

729          '3 % of the processor.',

730          '**Simulation speed:** the timers run on slow clocks and unused timers are stopped, because '

731          'Wokwi spends time on every timer count. Measured: about 93 % of real time during the '

732          'logo and 91 % with every LED lit.',

733          '**Memory:** only fixed-size arrays (no `malloc`), so nothing can run out during a game. '

734          '33 KB of 248 KB flash and 2.0 KB of 8 KB RAM are used; fonts, pixel art, music and spawn '

735          'lists are kept in flash.',

736          '**Fairness:** at most 4–6 enemies at once, no shots from close range, a crate always '

737          'comes when the ammo runs low, and every phase was checked to be winnable.',

738      ])

739

740      doc.h('3.15 Testing', 2)

741      R('Besides playing it in Wokwi, the real game code is compiled on the PC together with a small '

742        'stand-in for the Arduino (registers as variables, a simulated clock, a simulated keypad). '

743        'Test scripts press keys and check the Serial output and the screen, so a whole game runs in '

744        'under a second. An **autopilot** that plays like a careful player was used to check that '

745        'every phase can be won, and to tune the difficulty.')

746      doc.table(['Test', 'What it checks', 'Result'], [

747          ['controls', 'tap, glide, row limit, single shot, auto-fire rate, move cancels fire, '

748           'no glide while firing', 'pass'],

749          ['screens', 'logo skip, title, menu, hangar, options, LED test, best, pause, game over and '

750           'new record, global keys, power cycle (EEPROM), demo', 'pass'],

751          ['playthrough', 'all three phases and bosses, VOID KING form 2, victory', 'pass'],

752          ['balance', 'autopilot without shield, reacting only every 250 ms (roughly a human), '

753           '48 games per difficulty', 'EASY 47/48, NORMAL 38/48, HARD 27/48 won'],

754          ['QA run', 'four test agents played and stress-tested every part (1000+ games and '

755           'random key runs)', 'no crash or hang; findings fixed or listed'],

756      ], [2.5, 10, 4.5], 'Automatic tests.')

757      R('Serial test commands (typed in the Wokwi terminal, starting with `!`) help with testing: '

758        '`!d` shows the screen as text, `!s` the game state, `!1`–`!3` start at a phase, `!b` skips '

759        'to the boss, `!i` gives a shield and `!x` turns on the autopilot.')

760

761

762  CODE_FILES = [

763      ('void_runner/void_runner.ino', 'The main file',

764       'The sketch that Wokwi compiles; every other file is a tab of it (one translation unit). '

765       '`setup()` stops the unused timers, starts the screen, the keypad and the sound, and loads '

766       'the saved settings. `loop()` makes one picture every 20 ms: it reads the keys, handles the '

767       'global keys M and B, runs the current screen and plays the sound. The screens form a state '

768       'machine (logo → title → menu → game → pause / game over / victory …): each screen function '

769       'returns the screen to show next. The Serial test commands (`!d`, `!s`, …) are handled here too.',

770       ['menu']),

771      ('void_runner/config.h', 'Settings',

772       'All settings in one place: the frame time, the names of the keys, the list of screens, the '

773       'difficulty tables (lives, enemy fire speed, boss health, ammo per crate) and game constants '

774       'such as the number of lives and the phase bonus. Changing the balance of the game only needs '

775       'this file.', []),

776      ('void_runner/pins.h', 'Pin numbers (generated)',

777       'Generated together with the circuit by `tools/gen_circuit.py`, so the pins in the code always '

778       'match the wires of the circuit: the 27 LED row pins, the 16 LED column pins (the bits of '

779       'PORTF and PORTK), the keypad row and column pins and the four buzzer pins.', ['wiring']),

780      ('void_runner/display.h', 'The LED screen',

781       'The Timer2 interrupt that lights one LED row at a time (71 full pictures per second), the '

782       'picture in memory (`frame`) and every drawing function: pixels, rectangles, bitmaps, letters '

783       'of the two fonts, numbers, message pages and the status bars. Brightness (100 / 50 / 20 %) is '

784       'made by switching the row off early in its time slot.', ['cu_leds']),

785      ('void_runner/input.h', 'The keypad',

786       'Scans the 3 × 3 push-button matrix row by row, as taught in the course, and debounces every '

787       'key. It remembers which keys are held and which were just pressed or released, so the other '

788       'parts can ask "was R just pressed?" or "is D held?".', ['cu_keypad']),

789      ('void_runner/sound.h', 'The sound engine',

790       'Drives the four buzzers with four 16-bit timers in CTC toggle mode, so each buzzer plays its '

791       'note without using processor time. A small music engine plays songs written as note lists '

792       '(one per voice, looping or once), and an effect engine on voice 3 plays the sound effects with '

793       'priorities. Mute (M) silences everything.', ['cu_buzzers']),

794      ('void_runner/tunes.h', 'Songs and sound effects',

795       'The note lists of every song and the parts of every sound effect, all stored in flash. The '

796       'four in-game songs are generated by `tools/compose_music.py` from a short note notation; the '

797       'jingles (fanfare, clear, victory, game over) and the effects are written here directly.', []),

798      ('void_runner/storage.h', 'EEPROM: settings and best score',

799       'Saves the chosen ship, the difficulty, the brightness, the sound setting and the best score '

800       '(with the zone it reached and its difficulty) in the Mega\'s EEPROM, using the EEPROM '

801       'registers directly. A check byte tells whether the EEPROM holds valid data.',

802       ['best1', 'best3']),

803      ('void_runner/font.h', 'Fonts (generated)',

804       'Two pixel fonts: small 3 × 5 letters (four fit on a line) and big 5 × 7 letters and digits. '

805       'They are drawn as ASCII art in `tools/gen_font.py`, which writes this file.',

806       ['zonedone', 'vic_d']),

807      ('void_runner/art.h', 'Pixel art',

808       'Every picture of the game as binary numbers: one 16-bit word per row, the 1s are the lit '

809       'pixels. The three ships, the enemies, the rocks, the pickups, the bosses, the warning sign and '

810       'the letters of the logo.',

811       ['sprite_ship_bolt', 'sprite_ship_dart', 'sprite_ship_twin', 'sprite_enemy_cruiser',

812        'sprite_boss_warden']),

813      ('void_runner/logo.h', 'Start-up logo and welcome melody',

814       'The 5.6-second logo animation and its melody. The picture is driven by the song\'s own clock, '

815       'so every movement lands on a note: warp, the ship, the VOID letters, "RUNNER" on a ribbon, '

816       'the flash and the glowing sign.', ['logo_ship', 'logo_letters', 'logo_runner']),

817      ('void_runner/title.h', 'Title screen',

818       'The VOID sign stays lit, ships keep flying past it, and every 7 seconds a banner shows the '

819       'pages "PUSH", a big R, "TO", "PLAY". After 20 s without a key the demo starts.',

820       ['title', 'title_push']),

821      ('void_runner/menu.h', 'Main menu',

822       'The menu PLAY, SHIP, OPTS, BEST: W/S move the underline, R opens the item, Q goes back to '

823       'the title.', ['menu']),

824      ('void_runner/world.h', 'Everything on the game screen',

825       'The data of the game world in fixed-size pools: the player, bullets and mines, enemies and '

826       'rocks, pickups, explosion particles and the boss, with the tables of the three ships and the '

827       'enemy types and small helpers to create them. There is no `malloc`, so nothing can run out '

828       'during a game.', ['phase1_1']),

829      ('void_runner/player.h', 'The player\'s ship',

830       'The key rules for moving and firing (tap, hold, auto-fire, newest key wins), ammo and the '

831       'low-ammo warning, getting hit (a life, explosion, shake, shield) and drawing the ship.',

832       ['hangar_bolt', 'hangar_dart', 'hangar_twin']),

833      ('void_runner/enemies.h', 'Enemies and rocks',

834       'How each enemy type moves and shoots, and the rule that keeps them apart: a move may not bring '

835       'an enemy closer than one empty pixel to another one, so it steps aside, turns or waits. New '

836       'enemies take the nearest free column.',

837       ['sprite_enemy_drone', 'sprite_enemy_weaver', 'sprite_enemy_miner', 'sprite_rock_small',

838        'sprite_rock_big']),

839      ('void_runner/levels.h', 'The three phases',

840       'The spawn list of each phase (wait, what, where), its enemy limit, its name, its song and its '

841       'boss. The lists are stored in flash.', ['zone1', 'rockbelt', 'go']),

842      ('void_runner/boss.h', 'The bosses',

843       'The behaviour of the WARDEN, the HUNTER (a small state machine: dash, aim, charge) and the '

844       'VOID KING (two forms), their health and their drawing.',

845       ['warden_1', 'hunter1', 'king2']),

846      ('void_runner/game.h', 'One game',

847       'Runs one game: the stages of a phase (intro, waves, warning, boss, boss down, clear), the '

848       'spawner, all collisions (bullets, enemies, rocks, mines, pickups, boss), the score, the '

849       'messages on top of the game and the three status bars.', ['warn2', 'bossdown', 'pts']),

850      ('void_runner/autopilot.h', 'Autopilot',

851       'A bot that plays like a careful player. It is used by the title screen\'s demo and by the '

852       'automatic tests: for every column it works out how soon something would hit the ship there, '

853       'flies to the safest column near its target and fires when lined up.', ['phase1_3']),

854      ('void_runner/screens.h', 'The other screens',

855       'The hangar, the options (with the LED test), the best score, pause, game over and victory. '

856       'Each screen function handles its keys, draws one picture and returns the next screen.',

857       ['opt_diff', 'pause_sign', 'over1', 'led4']),

858  ]

859

860  TOOL_FILES = [

861      ('tools/gen_circuit.py', 'Circuit generator',

862       'Generates `diagram.json` (432 LEDs, 9 buttons, 4 buzzers and every wire) and `pins.h`, '

863       'routes the wires so they never cross, and checks every connection.'),

864      ('tools/gen_font.py', 'Font generator', 'Turns the ASCII-art letters into `font.h`.'),

865      ('tools/compose_music.py', 'Music composer',

866       'The in-game songs in a short note notation; writes their note lists into `tunes.h` and '

867       'can render them as WAV files to listen to.'),

868      ('tools/render_dumps.py', 'Screen renderer',

869       'Draws screen dumps (the `!d` command) as PNG pictures or GIF animations in the real LED colours.'),

870      ('tools/draw_wiring.py', 'Wiring diagram', 'Draws the simplified wiring diagram.'),

871      ('tools/render_circuit.py', 'Circuit photo',

872       'Draws the whole circuit with Wokwi\'s own part pictures, lit with a game picture.'),

873      ('tools/make_doc.py', 'This document', 'Builds this document from the code and the game itself.'),

874      ('tests/host/Arduino.h', 'PC stand-in for Arduino',

875       'Registers as variables, a simulated clock and keypad, so the real game code runs on a PC.'),

876      ('tests/host/sim.cpp', 'PC test runner', 'Runs the game code from test scripts (keys, Serial, dumps).'),

877      ('tests/host/run_tests.py', 'Test runner', 'Builds the PC version and runs all tests and the balance check.'),

878      ('tests/host/controls.txt', 'Test: controls', 'The key rules.'),

879      ('tests/host/screens.txt', 'Test: screens', 'Every screen and the EEPROM.'),

880      ('tests/host/playthrough.txt', 'Test: whole game', 'All phases to the victory.'),

881  ]

882

883

884  def code_part(doc, pics):

885      doc.page_break()

886      doc.h('4. Project code', 1)

887      R = doc.rich

888      R('The program is written in C++ for the Arduino Mega (Arduino core functions plus direct '

889        'register access where speed or timing needs it; no libraries). It is one sketch, '

890        '`void_runner.ino`, with one header file per module; in Wokwi every file is a tab. All files '

891        'together are compiled as one unit. The game logic advances in steps of 20 ms; the screen and '

892        'the sound run on hardware timers.')

893      rows = []

894      for path, title, _, _ in CODE_FILES:

895          n = len((ROOT / path).read_text(encoding='utf-8').splitlines())

896          rows.append([Path(path).name, title, str(n)])

897      total = sum(int(r[2]) for r in rows)

898      rows.append(['**total**', '', '**%d**' % total])

899      doc.table(['File', 'Contents', 'Lines'], rows, [4.5, 10.5, 2], 'The files of the program.')

900      R('Each file below starts with a short explanation and the list of its functions (taken from the '

901        'description written above every function), followed by the full source code.')

902      for i, (path, title, text, pic_names) in enumerate(CODE_FILES, 1):

903          src = (ROOT / path).read_text(encoding='utf-8')

904          doc.page_break()

905          doc.h('4.%d %s – %s' % (i, Path(path).name, title), 2)

906          R(text)

907          if pic_names:

908              per = len(pic_names)

909              width = {1: 3.6, 2: 3.4, 3: 3.2, 4: 3, 5: 2.8}[per]

910              if pic_names[0] in ('wiring', 'cu_leds', 'cu_keypad', 'cu_buzzers'):

911                  doc.picture(pics[pic_names[0]], {'wiring': 15, 'cu_leds': 5}.get(pic_names[0], 7),

912                              'Belongs to %s.' % Path(path).name)

913              else:

914                  doc.gallery([(pics[n], '') for n in pic_names], per, width,

915                              'What %s draws or uses.' % Path(path).name)

916          funcs = functions_of(src)

917          if funcs:

918              doc.table(['Function', 'What it does'], [[f, d or '–'] for f, d in funcs], [6, 11],

919                        size=8.5)

920          doc.h('Source code', 3)

921          doc.code(src, 'c')

922

923      doc.page_break()

924      doc.h('4.%d Development tools and tests' % (len(CODE_FILES) + 1), 2)

925      R('These programs are not part of the Arduino sketch. They generate parts of it (the circuit, '

926        'the pin list, the font, the music), make the pictures, and test the game on the PC.')

927      doc.table(['File', 'Job'], [[p, t + ': ' + e] for p, t, e in TOOL_FILES], [5, 12], size=9)

928      for path, title, text in TOOL_FILES:

929          src = (ROOT / path).read_text(encoding='utf-8')

930          doc.h('%s – %s' % (path, title), 3)

931          R(text)

932          doc.code(src, 'c' if path.endswith(('.h', '.cpp')) else 'py')

933

934

935  def main():

936      pics = make_media()

937      doc = Doc()

938      cover(doc)

939      hardware_page(doc, pics)

940      performance(doc, pics)

941      code_part(doc, pics)

942      OUT.parent.mkdir(exist_ok=True)

943      doc.d.save(OUT)

944      print('wrote', OUT)

945

946

947  if __name__ == '__main__':

948      main()

### tests/host/Arduino.h – PC stand-in for Arduino

Registers as variables, a simulated clock and keypad, so the real game code runs on a PC.

1  /*

2   * Arduino.h (PC version) -- lets the real VOID RUNNER code compile and run on a PC for

3   * automated tests (tests/host/sim.cpp). It is NOT used for the Arduino build.

4   *

5   * Everything the sketch takes from Arduino is imitated just well enough:

6   *   - flash access (PROGMEM, pgm_read_..., PSTR, F) reads normal memory

7   *   - timer and port registers are plain variables; the harness reads the buzzer

8   *     timers back to log the notes

9   *   - the EEPROM registers drive a 4 KB array

10   *   - millis()/micros() come from the harness's simulated clock

11   *   - digitalRead() of a keypad column answers from the keys the test script holds

12   *   - random() is avr-libc's generator, so runs are repeatable

13   */

14  #pragma once

15  #include <cctype>

16  #include <cstdint>

17  #include <cstdio>

18  #include <cstring>

19  #include <cstdlib>

20  #include <string>

21

22  // ---- flash memory = normal memory on a PC

23  #define PROGMEM

24  #define PSTR(s) (s)

25  class __FlashStringHelper;

26  #define F(s) (reinterpret_cast<const __FlashStringHelper*>(s))

27  #define pgm_read_byte(p) (*(const uint8_t*)(p))

28  #define pgm_read_word(p) (*(const uint16_t*)(p))

29  #define pgm_read_ptr(p) (*(const void* const*)(p))

30  #define memcpy_P memcpy

31  #define strchr_P strchr

32  #define strlen_P strlen

33

34  // ---- pins and basic helpers

35  const uint8_t LOW = 0, HIGH = 1;

36  const uint8_t INPUT = 0, OUTPUT = 1, INPUT_PULLUP = 2;

37  const uint8_t A0 = 54, A1 = 55, A2 = 56, A3 = 57, A4 = 58, A5 = 59, A6 = 60, A7 = 61,

38                A8 = 62, A9 = 63, A10 = 64, A11 = 65, A12 = 66, A13 = 67, A14 = 68, A15 = 69;

39  #define _BV(bit) (1 << (bit))

40  #define constrain(amt, low, high) ((amt) < (low) ? (low) : ((amt) > (high) ? (high) : (amt)))

41  #define lowByte(w) ((uint8_t)((w) & 0xff))

42  #define highByte(w) ((uint8_t)((w) >> 8))

43  inline bool isAlpha(int c) { return isalpha(c) != 0; }

44  inline void noInterrupts() {}

45  inline void interrupts() {}

46  inline void delayMicroseconds(unsigned) {}

47  #define ISR(vector) void vector()

48  const unsigned long F_CPU = 16000000UL;

49

50  extern uint8_t pinModes[70], pinLevels[70];

51  inline void pinMode(uint8_t pin, uint8_t mode) { pinModes[pin] = mode; }

52  inline void digitalWrite(uint8_t pin, uint8_t level) { pinLevels[pin] = level; }

53  int digitalRead(uint8_t pin);            // in sim.cpp (keypad)

54  inline int analogRead(uint8_t) { return 0; }

55

56  // ---- simulated clock

57  extern uint64_t simMicros;

58  inline unsigned long millis() { return (unsigned long)(simMicros / 1000); }

59  inline unsigned long micros() { return (unsigned long)simMicros; }

60

61  // ---- random(): the same generator as avr-libc

62  extern long randomState;

63  inline long avrRandom() {

64    long x = randomState ? randomState : 123459876L;

65    long hi = x / 127773L, lo = x % 127773L;

66    x = 16807L * lo - 2836L * hi;

67    if (x < 0) x += 0x7fffffffL;

68    randomState = x;

69    return x;

70  }

71  inline long random(long howBig) { return howBig ? avrRandom() % howBig : 0; }

72  inline long random(long howSmall, long howBig) {

73    return howSmall >= howBig ? howSmall : howSmall + random(howBig - howSmall);

74  }

75  inline void randomSeed(unsigned long seed) { if (seed) randomState = (long)seed; }

76

77  // ---- registers (timers 1-5, ports, EEPROM)

78  #define REG8(name) inline volatile uint8_t name = 0

79  #define REG16(name) inline volatile uint16_t name = 0

80  REG8(TCCR1A); REG8(TCCR1B); REG16(OCR1A); REG16(TCNT1);

81  REG8(TCCR2A); REG8(TCCR2B); REG8(OCR2A); REG8(OCR2B); REG8(TIMSK2);

82  REG8(TCCR3A); REG8(TCCR3B); REG16(OCR3A); REG16(TCNT3);

83  REG8(TCCR4A); REG8(TCCR4B); REG16(OCR4A); REG16(TCNT4);

84  REG8(TCCR5A); REG8(TCCR5B); REG16(OCR5A); REG16(TCNT5);

85  REG8(PORTF); REG8(PORTK);

86  const uint8_t WGM12 = 3, CS12 = 2, COM1A0 = 6;

87  const uint8_t WGM21 = 1, CS22 = 2, CS21 = 1, OCIE2A = 1, OCIE2B = 2;

88

89  inline uint8_t eepromData[4096];

90  inline volatile uint16_t EEAR = 0;

91  inline volatile uint8_t EEDR = 0;

92  const uint8_t EERE = 0, EEPE = 1, EEMPE = 2;

93  struct EecrRegister {                    // setting EERE reads, setting EEPE writes

94    operator uint8_t() const { return 0; } // never busy

95    EecrRegister& operator|=(int bits) {

96      if (bits & _BV(EERE)) EEDR = eepromData[EEAR % 4096];

97      if (bits & _BV(EEPE)) eepromData[EEAR % 4096] = EEDR;

98      return *this;

99    }

100  };

101  inline EecrRegister EECR;

102

103  // ---- Serial: prints to the harness, reads the commands of the test script

104  void serialWrite(const std::string& text);   // in sim.cpp

105  extern std::string serialInput;

106  struct SerialPort {

107    void begin(unsigned long) {}

108    int available() { return (int)serialInput.size(); }

109    int read() {

110      if (serialInput.empty()) return -1;

111      char c = serialInput[0];

112      serialInput.erase(0, 1);

113      return (unsigned char)c;

114    }

115    void print(const __FlashStringHelper* s) { serialWrite(reinterpret_cast<const char*>(s)); }

116    void print(const char* s) { serialWrite(s); }

117    void print(char c) { serialWrite(std::string(1, c)); }

118    void print(int n) { serialWrite(std::to_string(n)); }

119    void print(unsigned n) { serialWrite(std::to_string(n)); }

120    void print(long n) { serialWrite(std::to_string(n)); }

121    void print(unsigned long n) { serialWrite(std::to_string(n)); }

122    void print(unsigned char n) { serialWrite(std::to_string(n)); }

123    template <typename T> void println(T value) { print(value); serialWrite("\n"); }

124    void println() { serialWrite("\n"); }

125  };

126  inline SerialPort Serial;

### tests/host/sim.cpp – PC test runner

Runs the game code from test scripts (keys, Serial, dumps).

1  /*

2   * sim.cpp -- runs the real VOID RUNNER code on a PC, driven by a test script.

3   * No Wokwi minutes needed: a whole game takes a few seconds.

4   *

5   * Build (from the project folder, MSYS2 g++):

6   *   g++ -std=c++17 -O2 -Itests/host -o build/sim.exe tests/host/sim.cpp

7   * Run:

8   *   build/sim.exe tests/host/<script>.txt > log.txt

9   *

10   * Script commands (one per line, # = comment):

11   *   run <ms>                 let the simulated time run

12   *   down <key> / up <key>    hold / release a keypad key (W, A, R, ...)

13   *   tap <key> [ms]           press a key for ms (default 60), then wait 60 ms

14   *   send <text>              type text into Serial (test commands like !d, !s)

15   *   until <text> [ms]        run until the Serial output shows text (default limit 60 s)

16   *   dump                     print the screen (same as send !d)

17   *   record <file> <ms>       from now on, write the screen to <file> every <ms>

18   *   stop                     stop recording

19   *   sound <file>             log every buzzer frequency change to <file>

20   *   reset                    power off and on again (the EEPROM keeps its data)

21   *   seed <n>                 start the random generator at n (another game)

22   *   times                    show the time (ms) in front of every Serial line

23   * A failed "until" prints [SIM] FAIL and ends with exit code 1.

24   */

25  #include "Arduino.h"

26  #include "../../void_runner/void_runner.ino"

27  #include <fstream>

28  #include <iostream>

29  #include <sstream>

30

31  uint8_t pinModes[70], pinLevels[70];

32  uint64_t simMicros = 0;

33  long randomState = 1;

34  std::string serialInput;

35

36  // ---- Serial output: printed, and kept so "until" can search it

37  std::string serialOutput;

38  size_t searchFrom = 0;                   // "until" only looks at output after this point

39

40  bool showTimes = false;                  // "times": put the time in front of every line

41  bool lineStart = true;

42

43  void serialWrite(const std::string& text) {

44    for (char c : text) {

45      if (showTimes && lineStart) std::cout << '[' << millis() << "] ";

46      std::cout << c;

47      lineStart = c == '\n';

48    }

49    serialOutput += text;

50  }

51

52  // ---- keypad: a column reads LOW when its key is held and its row is driven LOW

53  bool keyDown[KEY_ROWS][KEY_COLS];

54

55  int digitalRead(uint8_t pin) {

56    for (uint8_t c = 0; c < KEY_COLS; c++) {

57      if (KEY_COL_PINS[c] != pin) continue;

58      for (uint8_t r = 0; r < KEY_ROWS; r++) {

59        uint8_t rowPin = KEY_ROW_PINS[r];

60        if (keyDown[r][c] && pinModes[rowPin] == OUTPUT && pinLevels[rowPin] == LOW) return LOW;

61      }

62      return HIGH;

63    }

64    return HIGH;

65  }

66

67  bool setKey(char key, bool down) {

68    for (uint8_t r = 0; r < KEY_ROWS; r++)

69      for (uint8_t c = 0; c < KEY_COLS; c++)

70        if (KEY_NAMES[r][c] == key) {

71          keyDown[r][c] = down;

72          return true;

73        }

74    return false;

75  }

76

77  // ---- screen recording, in the same text format as the !d dump

78  std::ofstream recordFile;

79  uint32_t recordEveryMs = 0, recordNextMs = 0;

80

81  void recordScreen() {

82    recordFile << "[DUMP] begin t=" << millis() << "\n";

83    for (uint8_t i = 0; i < TOTAL_ROWS; i++) {

84      uint8_t r = (i < STATUS_ROWS) ? SCREEN_H + i : i - STATUS_ROWS;

85      if (i == STATUS_ROWS) recordFile << "----------------\n";

86      for (uint8_t x = 0; x < SCREEN_W; x++) recordFile << ((shown[r] >> x) & 1 ? '#' : '.');

87      recordFile << "\n";

88    }

89    recordFile << "[DUMP] end\n";

90  }

91

92  // ---- sound log: the frequency each buzzer timer plays (0 = silent)

93  std::ofstream soundFile;

94  uint16_t lastHz[BUZZER_COUNT];

95

96  void logSound() {

97    for (uint8_t v = 0; v < BUZZER_COUNT; v++) {

98      const TimerRegs& t = VOICE_TIMER[v];

99      bool on = (*t.tccrB & 7) && (*t.tccrA & _BV(COM1A0));

100      uint16_t hz = on ? F_CPU / (2UL * 256 * (*t.ocrA + 1)) : 0;

101      if (hz != lastHz[v]) {

102        lastHz[v] = hz;

103        soundFile << millis() << " " << (int)v << " " << hz << "\n";

104      }

105    }

106  }

107

108  // ---- one millisecond of the Arduino's life

109  void tick() {

110    simMicros += 1000;

111    loop();

112    if (recordFile.is_open() && millis() >= recordNextMs) {

113      recordScreen();

114      recordNextMs = millis() + recordEveryMs;

115    }

116    if (soundFile.is_open()) logSound();

117  }

118

119  void runMs(uint32_t ms) {

120    for (uint32_t i = 0; i < ms; i++) tick();

121  }

122

123  bool runUntil(const std::string& text, uint32_t limitMs) {

124    for (uint32_t i = 0; i <= limitMs; i++) {

125      size_t at = serialOutput.find(text, searchFrom);

126      if (at != std::string::npos) {

127        searchFrom = at + text.size();

128        return true;

129      }

130      tick();

131    }

132    return false;

133  }

134

135  int main(int argc, char** argv) {

136    if (argc < 2) {

137      std::cerr << "usage: sim <script>\n";

138      return 2;

139    }

140    std::ifstream script(argv[1]);

141    if (!script) {

142      std::cerr << "cannot open " << argv[1] << "\n";

143      return 2;

144    }

145    memset(eepromData, 0xFF, sizeof(eepromData));   // a new chip: EEPROM erased

146    setup();

147    std::string line;

148    int lineNo = 0;

149    while (std::getline(script, line)) {

150      lineNo++;

151      if (!line.empty() && line.back() == '\r') line.pop_back();

152      std::istringstream in(line);

153      std::string cmd;

154      if (!(in >> cmd) || cmd[0] == '#') continue;

155      std::string arg;

156      std::getline(in >> std::ws, arg);

157      if (cmd == "run") {

158        runMs(std::stoul(arg));

159      } else if (cmd == "down" || cmd == "up") {

160        if (!setKey(arg[0], cmd == "down")) std::cout << "[SIM] unknown key " << arg << "\n";

161      } else if (cmd == "tap") {

162        std::istringstream a(arg);

163        char key;

164        uint32_t ms = 60;

165        a >> key >> ms;

166        setKey(key, true);

167        runMs(ms);

168        setKey(key, false);

169        runMs(60);

170      } else if (cmd == "send") {

171        serialInput += arg;

172        runMs(5);

173      } else if (cmd == "dump") {

174        serialInput += "!d";

175        runMs(5);

176      } else if (cmd == "until") {

177        uint32_t limit = 60000;

178        size_t q1 = arg.find('"'), q2 = arg.rfind('"');

179        std::string text = arg;

180        if (q1 != std::string::npos && q2 > q1) {

181          text = arg.substr(q1 + 1, q2 - q1 - 1);

182          std::istringstream rest(arg.substr(q2 + 1));

183          rest >> limit;

184        }

185        if (!runUntil(text, limit)) {

186          std::cout << "\n[SIM] FAIL line " << lineNo << ": no \"" << text << "\" within "

187                    << limit << " ms (t=" << millis() << ")\n";

188          return 1;

189        }

190      } else if (cmd == "record") {

191        size_t lastSpace = arg.rfind(' ');           // the file name may contain spaces

192        recordEveryMs = std::stoul(arg.substr(lastSpace + 1));

193        recordFile.open(arg.substr(0, lastSpace));

194        recordNextMs = millis();

195      } else if (cmd == "stop") {

196        recordFile.close();

197      } else if (cmd == "sound") {

198        soundFile.open(arg);

199      } else if (cmd == "times") {

200        showTimes = true;

201      } else if (cmd == "seed") {

202        randomState = std::stol(arg);

203      } else if (cmd == "reset") {

204        memset((void*)keyDown, 0, sizeof(keyDown));

205        setup();

206      } else {

207        std::cout << "[SIM] unknown command on line " << lineNo << ": " << line << "\n";

208        return 2;

209      }

210    }

211    std::cout << "\n[SIM] PASS t=" << millis() << "\n";

212    return 0;

213  }

### tests/host/run_tests.py – Test runner

Builds the PC version and runs all tests and the balance check.

1  #!/usr/bin/env python3

2  """

3  VOID RUNNER - automated tests on the PC (no Wokwi minutes needed).

4

5  Builds tests/host/sim.cpp (the real game code + a PC stand-in for Arduino) with g++ and

6  runs every test script in tests/host/*.txt. Each script presses keys, sends Serial test

7  commands and waits for the game's Serial messages; see sim.cpp for the commands.

8

9      python tests/host/run_tests.py              build + run all test scripts

10      python tests/host/run_tests.py --balance    also: autopilot games without the shield

11                                                  on EASY / NORMAL / HARD (wins per level)

12

13  Needs g++ (MSYS2 UCRT64 on this PC: C:\\msys64\\ucrt64\\bin). Logs go to build/.

14  """

15  import os

16  import shutil

17  import subprocess

18  import sys

19  from pathlib import Path

20

21  ROOT = Path(__file__).resolve().parents[2]

22  HOST = ROOT / 'tests' / 'host'

23  BUILD = ROOT / 'build'

24  EXE = '.exe' if os.name == 'nt' else ''

25

26

27  def find_gpp():

28      gpp = shutil.which('g++')

29      if not gpp and Path(r'C:\msys64\ucrt64\bin\g++.exe').exists():

30          gpp = r'C:\msys64\ucrt64\bin\g++.exe'

31      if not gpp:

32          sys.exit('g++ not found (install MSYS2 UCRT64 or add it to PATH)')

33      return gpp

34

35

36  def build(name, defines=()):

37      exe = BUILD / (name + EXE)

38      cmd = [find_gpp(), '-std=c++17', '-O2', '-static', '-I', str(HOST),

39             *('-D' + d for d in defines), '-o', str(exe), str(HOST / 'sim.cpp')]

40      subprocess.run(cmd, check=True, cwd=ROOT)

41      return exe

42

43

44  def run(exe, script_text, log):

45      script = BUILD / (log.stem + '.script')

46      script.write_text(script_text, encoding='utf-8')

47      result = subprocess.run([str(exe), str(script)], capture_output=True, text=True, cwd=ROOT)

48      log.write_text(result.stdout, encoding='utf-8')

49      return result.returncode == 0, result.stdout

50

51

52  def run_tests():

53      exe = build('sim')

54      failed = 0

55      for script in sorted(HOST.glob('*.txt')):

56          ok, out = run(exe, script.read_text(encoding='utf-8'), BUILD / ('test_' + script.stem + '.log'))

57          last = [l for l in out.splitlines() if l.startswith('[SIM]')][-1:]

58          print('%-6s %-12s %s' % ('PASS' if ok else 'FAIL', script.stem, last[0] if last else ''))

59          failed += not ok

60      return failed

61

62

63  # Autopilot games without the shield. The autopilot here "thinks" only every 250 ms,

64  # a rough stand-in for a human's reaction time, so the result says how hard each level is.

65  def balance(seeds=48, react_ms=250):

66      exe = build('sim_balance', ['AUTOPILOT_REACT_MS=%d' % react_ms])

67      for level, key in (('EASY', 'tap A'), ('NORMAL', ''), ('HARD', 'tap D')):

68          wins, bosses = 0, []

69          for seed in range(1, seeds + 1):

70              script = '\n'.join([

71                  'seed %d' % seed, 'until "[VR] screen logo"', 'tap R', 'until "[VR] screen title"',

72                  'tap R', 'until "[VR] screen menu"', 'tap S', 'tap S', 'tap R',

73                  'until "[VR] screen options"', 'tap S', 'tap S', key, 'tap Q',

74                  'until "[VR] screen menu"', 'send !x', 'send !1', 'until "[VR] phase 1"',

75                  'until "[VR] screen victory" 900000', ''])

76              ok, out = run(exe, script, BUILD / 'balance.log')

77              wins += ok

78              bosses.append(out.count('[VR] boss down'))

79          print('%-6s wins %2d/%d   bosses beaten per game: %s' % (level, wins, seeds, ' '.join(map(str, bosses))))

80

81

82  if __name__ == '__main__':

83      BUILD.mkdir(exist_ok=True)

84      failed = run_tests()

85      if '--balance' in sys.argv:

86          balance()

87      sys.exit(1 if failed else 0)

### tests/host/controls.txt – Test: controls

The key rules.

1  # The agreed key rules, pressed on the real (simulated) keypad during phase 1's intro,

2  # before any enemy arrives. The ship starts at x=6 y=20 with 60 ammo (BOLT).

3  until "[VR] screen logo"

4  send !i

5  send !1

6  until "[VR] phase 1"

7  send !s

8  until "ammo=60 x=6 y=20"

9

10  # tap = one pixel

11  tap D

12  send !s

13  until "ammo=60 x=7 y=20"

14

15  # hold = glide (first pixel at once, then after 170 ms one pixel every 70 ms)

16  down A

17  run 500

18  up A

19  run 50

20  send !s

21  until "ammo=60 x=2 y=20"

22

23  # up is limited to row 9, so the ship stays in the lower part of the screen

24  down W

25  run 1500

26  up W

27  run 50

28  send !s

29  until "x=2 y=9"

30  down S

31  run 1500

32  up S

33  run 50

34

35  # tap R = one bullet

36  tap R

37  send !s

38  until "ammo=59 x=2 y=21"

39

40  # hold R = auto-fire, one bullet every 200 ms (0, 200, 400, 600, 800 ms)

41  down R

42  run 900

43  up R

44  run 50

45  send !s

46  until "ammo=54 "

47

48  # a move key stops the fire, even with R still held; the ship moves instead

49  down R

50  run 300

51  down D

52  run 400

53  up D

54  run 400

55  up R

56  run 50

57  send !s

58  until "ammo=52 x=6 y=21"

59

60  # while R is held, a held move key does not glide; letting go of R lets it glide again

61  down D

62  run 300

63  down R

64  run 400

65  up R

66  run 200

67  up D

68  run 50

69  send !s

70  until "ammo=50 x=12 y=21"

### tests/host/screens.txt – Test: screens

Every screen and the EEPROM.

1  # Every screen with the real keys: logo, title, menu, hangar, options, LED test, best,

2  # a game with pause, game over, the global keys, and a power cycle (EEPROM).

3  until "[VR] saved: best=0 ship=0 difficulty=1"

4  until "[VR] screen logo"

5  run 2000

6  tap R

7  until "[VR] screen title"

8  run 500

9  dump

10  tap R

11  until "[VR] screen menu"

12  dump

13

14  # SHIP: the hangar, choose DART

15  tap S

16  tap R

17  until "[VR] screen hangar"

18  run 400

19  dump

20  tap D

21  run 400

22  dump

23  tap R

24  until "[VR] ship DART"

25  until "[VR] screen menu"

26

27  # OPTS: difficulty HARD, then the LED test

28  tap S

29  tap R

30  until "[VR] screen options"

31  dump

32  tap S

33  tap S

34  tap D

35  until "[VR] option DIFF 2"

36  dump

37  tap S

38  tap R

39  until "[VR] screen ledtest"

40  run 800

41  dump

42  run 1500

43  dump

44  run 2500

45  dump

46  run 1500

47  dump

48  until "[VR] screen options" 10000

49  tap Q

50  until "[VR] screen menu"

51

52  # BEST before any game

53  tap S

54  tap R

55  until "[VR] screen best"

56  dump

57  tap Q

58  until "[VR] screen menu"

59

60  # PLAY with DART on HARD: 3 lives, 40 ammo; pause and continue; lose

61  tap W

62  tap W

63  tap W

64  tap R

65  until "[VR] phase 1"

66  send !s

67  until "lives=3 ammo=40"

68  send !i

69  send !x

70  run 15000

71  tap P

72  until "[VR] screen pause"

73  run 1000

74  dump

75  tap P

76  until "[VR] screen game"

77  run 1000

78  send !k

79  until "[VR] game over score="

80  until "[VR] screen gameover" 5000

81  until "[VR] new best" 100

82  run 1500

83  dump

84  run 1500

85  dump

86  tap R

87  until "[VR] screen menu"

88

89  # global keys

90  tap B

91  until "[VR] brightness 50"

92  tap M

93  until "[VR] sound off"

94  tap M

95  until "[VR] sound on"

96

97  # BEST after the game

98  tap S

99  tap S

100  tap S

101  tap R

102  until "[VR] screen best"

103  dump

104

105  # power off and on: settings and best score come back from the EEPROM

106  reset

107  until "[VR] saved: best="

108  until " ship=1 difficulty=2"

109

110  # demo: the title screen untouched for 20 s starts a demo game; any key ends it

111  until "[VR] screen title" 10000

112  until "[VR] demo" 25000

113  run 4000

114  dump

115  tap S

116  until "[VR] screen title"

### tests/host/playthrough.txt – Test: whole game

All phases to the victory.

1  # Whole game played by the autopilot (shield on, normal speed): all three phases,

2  # warnings, bosses (with the Void King's second form) and the victory screen.

3  until "[VR] screen logo"

4  send !i

5  send !x

6  send !1

7  until "[VR] phase 1"

8  run 3000

9  dump

10  until "[VR] warning" 200000

11  send !s

12  until "[VR] boss" 10000

13  run 3000

14  dump

15  send !s

16  until "[VR] boss down" 120000

17  until "[VR] phase clear" 10000

18  until "[VR] phase 2" 10000

19  until "[VR] warning" 200000

20  send !s

21  until "[VR] boss" 10000

22  run 3000

23  dump

24  until "[VR] boss down" 120000

25  until "[VR] phase 3" 10000

26  until "[VR] warning" 200000

27  send !s

28  until "[VR] boss" 10000

29  run 3000

30  dump

31  until "[VR] boss form 2" 120000

32  run 500

33  dump

34  until "[VR] boss down" 120000

35  until "[VR] screen victory" 10000

36  until "[VR] victory score=" 100

37  run 2000

38  dump